MISE Instrument Project

MISE Nominal Day-in-the-Life (DITL) Test Procedure

THIS PROCEDURE DOES NOT CONTAIN HAZARDOUS OPERATIONS

Prepared by:  Rick Redick

Paper copies of this document may not be current and should not be relied on for official purposes.

The current version is in the MISE PBAT Repository at the following URL: https://pbat.jpl.nasa.gov

Date Started:	_____________

Date Closed:	_____________

Instance #:	_____________

National Aeronautics and

Space Administration

Jet Propulsion Laboratory

California Institute of Technology

Pasadena, California

| Instance No. |  |
| --- | --- |
| Temperature |  |
| Sections Run. |  |
| IBAT No. |  |
| Operator/ Test Buddy |  |

| Procedure Instance Open |  |
| --- | --- |
| Sign |  |
| Date |  |

| Procedure Instance Closed |  |
| --- | --- |
| Sign |  |
| Date |  |
| Notes (i.e. complete, new rev, etc.) |  |

Signature Page

Prepared By:

Zachary Bailey		Date

MISE Deputy ISE

Approved by:

____________________________		__________________________

Rick Redick						Date

MISE Instrument System Engineer

____________________________		__________________________

Sarah Lundeen					Date

MISE Ops Lead

____________________________		__________________________

Danielle Nuding          				Date

MISE I&T Lead

Electronic Signatures in PBAT

“Page intentionally left blank for pasting in the electronic approval/signature screen from PBAT”

CHANGE LOG

| DATE | SECTIONS CHANGED | REASON FOR CHANGE | REVISION |
| --- | --- | --- | --- |
| 2022-01-07 | All |  | INITIAL RELEASE Only valid for dry-runs on MISE Test Model (TM) |
| 2022-05-18 |  | Incorporate redlines, references to SOP procedure. Incorporated changes to macros and observation timelines (based on TM dry run 1), including removing some python scripts. Removed all except one cube processing for this dry run. | Rev. A |
| 2022-07-31 | Section 4 Section 5 Appendix A | Updated “Dry Run” references to “Ambient Functional.” Updated startup macro. Updated macros for CEU parameters. Corrected cooler goal temps to match FM sensors. Added Appendix A for cooler temp goals (K to DN). Added notes to clarify configurations and safety for ambient functional. Incorporated redlines and lessons from TM Dry Run 2. Added number of commands in each script | Rev. B |

| Personnel Usage Log |  |  |  |
| --- | --- | --- | --- |
| Initials | Full Name | Badge No. | Role |

CONTENTS

1	INTRODUCTION	9

1.1	Objective	9

1.2	Hazardous Operations	9

2	REQUIREMENTS	9

2.1	Personnel	9

2.2	Required Equipment	9

2.2.1	Electrical Ground Support Equipment:	10

2.2.2	GSE Software:	10

2.3	Applicable Documents	10

2.4	Reference Documents	10

2.5	Safety	11

2.6	General	12

2.7	Hardware Safety	13

2.7.1	CEU Launch Lock Potential Damage	13

3	Preparation	14

3.1	Setup	14

4	Configure for DITL Test	16

4.1	Power ON DPU to Boot Mode	16

4.2	Transition to Application Mode	17

4.3	Define Startup and Test-Specific Macros	18

4.4	Boot FPMC	21

4.5	Dump Analogs	22

4.6	Dump Monitor Limits & Parameters	22

4.7	Dump Macros	23

4.8	Power Off DPU	23

5	Day-in-the-Life (DITL) Test	26

5.1	Apoapsis Activities (Memory Management)	27

5.1.2	Power On the DPU	27

5.1.3	Apoapsis Commanding	29

5.1.4	Power Off the DPU	33

5.2	Flyby Activities	35

5.2.2	Power On the DPU	36

5.2.3	Cooldown, Flyby Data Acquisition, and Processing Part 1	37

5.2.4	Power Off the DPU	43

6	Cleanup	46

6.1	Power-Off SCE	46

A	Cryocooler Setpoint Conversion	47

FIGURES

Figure 3-1: MISE Ambient Functional (Full Instrument) Test Configuration	14

Figure 4-1: MISE DPU Window	16

Figure 4-2: MISE DPU Screen – Boot Button	18

TABLES

Table 1-1: Verification Items	9

# INTRODUCTION

This activity corresponds to the following V&V Activity in DOORS Next Generation (DNG):

978576 “Day in the Life” Validation.

## Objective

The objective of this procedure is to simulate a single Jovian orbit of MISE operations.  This includes apoapsis memory management, flyby prep, science data acquisition, onboard processing, and transfer.  Note that this a nominal test procedure; it does not cover fault responses and recovery.

The following VIs are under test:

Table 1-1: Verification Items

| DNG ID | Name | Test Section |
| --- | --- | --- |
| 641779 | For a reference Europa observation set, the total volume of MISE data shall be less than 7.2 Gbits. | Multiple |

## Hazardous Operations

No hazardous operations are contained in the procedure.

# REQUIREMENTS

## Personnel

The following personnel are required for the operations described in this procedure.

- Spacecraft Emulator Operator (SCE, Station Identifier)

## Required Equipment

The following equipment is required for the operations described in this procedure.

### Electrical Ground Support Equipment:

- Spacecraft emulator (SCE) GSE - GSE provides data and power interfaces to DPU & CEU.

- Power Supply for DPU and CEU

- GSEOS Computer

### GSE Software:

The following GSE software will be used to support MISE calibration testing. Additional description of GSE software and links for s/w repositories available here: https://wiki.jpl.nasa.gov/pages/viewpage.action?pageId=487901605

- GSEOS - Software running on SCE PC controls power interface to MISE DPU & CEU, provides command interface to DPU, and decodes DPU telemetry.

Telemetry Parser - Scripts to decode recorded DPU engineering & science packets.

## Applicable Documents

- D-1348; JPL Standard for Electrostatic Discharge Control.

D-34880; JPL Standard for Systems Safety

D-8091; JPL Anomaly Resolution

## Reference Documents

Requirements:

- Europa Clipper MISE FDD JPL D-56160, Revision B

MISE Instrument EICD, D-100471 Revision A

MCDE Electrical Interface Control Document ICD50025 Revision B

MCDE Communication Protocol ICD50026 Revision D

Specs:

- MISE Flight Software Specification, 7489-9100

- MISE User Manual, D-100505 Initial Release

Test Procedures

- MISE DPU Aliveness Test Procedure, 7489-9411 Rev. D

MISE DPU Functional Test Procedure, 7489-9412 Rev. A

MISE System Functional and Performance Test Procedure, 7489-9430 Rev. -

MISE TM Full Functional (DPU & FPIE) Test Procedure, P15924 Init. Rel.

MISE TM CEU Functional Test Procedure, P15971 Init. Rel.

MISE Standard Operating Procedure (SOP), P18247 Rev. A

## Safety

The safety requirements contained in the documents listed above shall be observed during all operations controlled by this procedure.

Warnings precede steps that could place humans in danger.

Cautions indicate steps that have the potential for damaging equipment.

## General

- ESD sensitive EM and FM Electronics, Wrist strap must be worn and cautions must be taken at all times during the test.

All BOB and BOB cables shall be discharged prior to mating with EM and FM modules, EGSE or the Instrument connectors.

Wrist straps must be tested each time the test conductor wears a new one or removes and reuse the same one.

Test area shall be designated as ESD approved. Follow JPL Rules Doc ID 34906 and ESD training at all times when handling flight hardware.

Humidity in the test area shall be between 30% and 70% at ambient pressure. An Air Ionizer must be used if Humidity is between 15% and 29%. Testing must stop if humidity gets to below 15%.

All required lab equipment shall be calibrated and calibration stickers shall be up to date and expiration date specified in the test procedure.

Personnel involved with the test shall be certified in Clean room, Flight Hardware Handling (JCI), Flight Mate and De-mate and ESD control.

Two people (Buddy system) are required to perform test on flight boards at all times.

All problems shall be documented via an IR (Inspection Report) form prior to first power-on of the flight module and via a PFR (Problem Failure Report) form after powering up the module.

All rework If any, shall be performed to a dispositioned and QA-concurred-with IR and on a QA-approved IBAT and in an approved clean and ESD certified room and by a certified flight technician.

Test results shall be documented in test data sheets of this test procedure. A printout copy shall be kept in each book for the specific serial number and an electronic copy shall be archived in PBAT.

## Hardware Safety

Hardware safety requirements in the JPL D-34880 will be followed for all steps of this procedure.  Additionally, the following safety precautions must be taken.

# CAUTION

The interface tests in this procedure could result in damage to equipment if the steps are not correctly followed.

### CEU Launch Lock Potential Damage

NOTE: Refer to the MISE User Manual D-100505 for additional details on CEU launch lock constraints

DPU Commanding of Launch Lock

The DPU autonomous management of the launch lock differs between the modes of cryocooler operations.  For the temperature control mode, the DPU FSW always sends a “launch unlock” command to the CEU prior to sending the “H-bridge on” command.  For the fixed voltage mode, the DPU FSW does NOT send a “launch unlock” command.  When commanding the fixed voltage mode, the MISE operator must send a “launch unlock” command, MISE_COOL_OP(launch unlock), prior to powering on the cryocooler.

Note: This procedure does not use Cryocooler fixed voltage mode commanding MISE_COOL_OP(voltage_test).  Any deviations from approved Cryocooler commands in this procedure require approval of the MISE I&T Lead.

# Preparation

The full functional test is conducted using the MISE FM hardware configuration shown in Figure 3-1.  This procedure is written to accommodate numerous hardware configurations.  Configuration-specific steps are described as such.

Figure 3-1: MISE Ambient Functional (Full Instrument) Test Configuration

## Setup

The following steps will be repeated every time the procedure is run.

SCE	Verify work area is clean and free from visible contaminants.

SCE	Start a test log on MISE I&T Wiki

Test Log #:

SCE	Setup GSEOS and spacecraft emulator per MISE Standard Operating Procedure P18247, Section 3:

P18247 Instance ID:

SCE	Record the instrument configuration for this test

FM or TM:

Is the scanner connected to the DPU?

Is the FPIE connected to the DPU?

Is the FPA connected to the FPIE?

Is the RCM attached to the base frame? _____________

Is the Cryocooler connected to the CEU?

- Is the TGSE heater attached to the cryocooler cold tip?  _______

Is the CEU connected to the DPU?

Describe instrument configuration:

# Configure for DITL Test

## Power ON DPU to Boot Mode

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    	(0.020)

GSEOS CEU current [A]:    	(0.000)

Power supply current [A]: 	(0.008)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

SCE	Select the Summary tab located at the bottom of the main GSEOS screen, in the MISE DPU screen. Click the DPU right gray button (see Figure 4-1) to set the DPU power current limit to 2.5A.

Verify SCE_CTRL_SET_CURRENT_LIMIT(CHANNEL_1, 5A, 2.5) command is dispatched in Cmd.log screen

Figure 4-1: MISE DPU Window

SCE	From the GSEOS SCE-> tab, in the SCE Status screen, in the box for SPACEWIRE Interface, confirm Active Side is set to A.

PEER	[PEER WITNESS] Witness DPU power on and monitor current during power on in the following step.

SCE	From GSEOS Summary tab, in MISE DPU screen, enable Power to DPU (see Figure 4-1)

Verify SCE_CTRL_SET_POWER_ENABLE_5A_CH_1() command is dispatched in Cmd.log screen

SCE	Record Current and Voltage as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS MISE Volts [V]:     	(30.0)

GSEOS DPU current [A]:    	(0.15)

Power supply current [A]: 	(0.15)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

PEER	[PEER WITNESS] Verify the DPU is powered on.

SCE	In Boot Status screen, observe time tag is incrementing once per second to confirm program booted correctly.

SCE	In Health & Status screen, observe time tag is incrementing and verify Mode shows Boot.

Mode:	(Boot)

## Transition to Application Mode

SCE	In MISE DPU screen, click Boot button (see Figure 4-2).

Verify MISE_ROM_BOOT() command is dispatched in Cmd.log screen

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    	(0.28)

Power supply current [A]: 	(0.28)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

Figure 4-2: MISE DPU Screen – Boot Button

SCE	In Boot Status screen confirm time tags stop incrementing.

SCE	In Health & Status screen, confirm Mode changes from Boot to App and confirm time tag continues incrementing. Rejected counter is expected to be 1.

Mode:	(App)

SCE	In Health & Status screen, confirm Accepted and Executed counters are incrementing.

SCE	Under GSEOS menus, select Core -> ROM -> GO. Enter address 0x410000, click OK.

Verify MISE_ROM_GO(0x410000) command is dispatched in Cmd.log screen

Verify MISE_ROM_GO command fails in Cmd_echo.log screen

Verify commands rejected counter incremented by 1

SCE	In Health & Status screen, confirm, Commands Log is updating.

SCE	In Status screen, record the DPU ProcCard FSW version number

ProcCard FSW Version:		(3-22081106)

## Define Startup and Test-Specific Macros

- NOTE:  DO NOT SEND MISE_MAC_RESTORE AT THIS TIME.

SCE	In the GSEOS Core tab, from the Status window, record the number of uplinked commands.

- Uplink Executed:  ____________

- Uplink Rejected:  ____________

SCE	Define setup macros.  This setup file is appropriate to run for all hardware configurations.  Under GSEOS menus, select File -> Open. Select file type “Python Modules (*.py, *.pyd)”, navigate to GitHub folder “C:\Users\mise\GitHub\Procedures\P17868_DITL\Rev B\”, choose the file “P17868_revB_setup.py” and click Open. The details of this file are included in Appendix A.

Selected macro file:

Verify commands are dispatched in Cmd.log screen

Verify all commands succeed. The Uplink Executed value should have incremented by 229 commands.  The Uplink Rejected should not have incremented.

SCE	Select appropriate file for MISE configuration and hardware present:

| Configuration | Macro File | # Cmds | Selected |
| --- | --- | --- | --- |
| DPU Standalone | P17868_revB_setup_DPUonly.py | 11 |  |
| DPU Plus Scanner | P17868_revB_setup_DPU+scanner.py | 11 |  |
| DPU Plus Scanner Plus OBA Plus FPIE (no RCM/cooler, no FPA) | P17868_revB_setup_DPU+scanner+OBA+FPIE.py Note: This script assumes all OBA heaters and temp sensors are connected to the DPU. If they are not connected when this script is run, there may be temperature sensor errors on other connected sensors. | 11 |  |
| DPU Plus Scanner Plus OBA/FPIE/FPA (no RCM/cooler) | P17868_revB_setup_DPU+scanner+OBA+FPIE+FPA.py Note: This script assumes all OBA heaters and temp sensors are connected to the DPU. If they are not connected when this script is run, there may be temperature sensor errors on other connected sensors. | 9 |  |
| Full Instrument / Flight | P17868_revB_setup_full_instrument.py Note: This script assumes all DPU heaters and temp sensors are connected to the DPU. If they are not connected when this script is run, there may be temperature sensor errors on other connected sensors. | 21 |  |

SCE	In the GSEOS Core tab, from the Status window, record the number of uplinked commands.

- Uplink Executed:  ____________

- Uplink Rejected:  ____________

SCE	Define startup and test-specific macros.  Under GSEOS menus, select File -> Open. Select file type “Python Modules (*.py, *.pyd)”, navigate to GitHub folder “C:\Users\mise\GitHub\Procedures\P17868_DITL\Rev B\”, choose the file selected in the previous step and click Open. The details of this file are included in Appendix A.

Selected macro file:

Verify commands are dispatched in Cmd.log screen

Verify all commands succeed. The Uplink Executed value should have incremented by the number of commands in the table above for the selected file.  The Uplink Rejected should not have incremented.

# CAUTION

Complete the next step ONLY IF this test will be run on the FM full instrument during TVAC, with the FM hardware under vacuum and at operating temperatures.  Otherwise, skip the next step.  If this next step is performed for any other configuration or environment, then it will cool the detector and spectrometer below the dew point and gather water on the detector and spectrometer.

SCE	In the GSEOS Core tab, from the Status window, record the number of uplinked commands.

- Uplink Executed:  ____________

- Uplink Rejected:  ____________

SCE	If and only if this setup for the full FM instrument with the RCM (and FM cryocooler) to be operated in TVAC (at vacuum and operating temperatures), under GSEOS menus, select File -> Open. Select file type “Python Modules (*.py, *.pyd)”, navigate to GitHub folder “C:\Users\mise\GitHub\Procedures\P17868_DITL\Rev B\”, choose the file “P17868_setup_CAUTION_FM_RCM_TVAC_ONLY.py” and click Open. The details of this file are included in Appendix A.

Verify commands are dispatched in Cmd.log screen

Verify all commands succeed. The Uplink Executed value should have incremented by 17 commands.  The Uplink Rejected should not have incremented.

SCE	Save defined macros to non-volatile memory. Under GSEOS menus, select Core -> Macro -> Save.

Verify MISE_MAC_SAVE() command is dispatched in Cmd.log screen

Verify MISE_MAC_SAVE command success in Cmd_echo.log screen

## Boot FPMC

SCE	From FPIE-FPMC Tab, in the Detector screen, click Yellow power button.

Verify MISE_DET_POWER(FPMCOnly) command is dispatched in Cmd.log screen

Verify MISE_DET_POWER command success in Cmd_echo.log screen

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.30)

Power supply current [A]: 		(FM: 0.30)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

SCE	From FPIE-FPMC Tab, in the FPMC screen, click the Boot button.

Verify MISE_FPMEM_GO(0x607e0000) command is dispatched in Cmd.log screen

Verify MISE_FPMEM_GO command success in Cmd_echo.log screen

SCE	Confirm the “Valid” light on FPMC screen turns from grey to green.

SCE	In the FPMC screen, verify FPMC is running the application program by checking FPMC FSW version number.

FPMC FSW Version:	 	(2-22031013)

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.34)

Power supply current [A]: 		(FM: 0.31)

- UTC Date & Time: ___ /___ /______T____:____ [UTC]

## Dump Analogs

SCE	From GSEOS Summary tab, review Analog screen and confirm that values are updating.  Attach a screenshot of analog screen to the activity report.

## Dump Monitor Limits & Parameters

SCE 	From GSEOS MISE tab, in Monitor Limits screen, click Read button.  Take a screenshot of this screen, and attach to the activity report.

Verify MISE_MEM_STR_READ(0) command is dispatched in Cmd.log screen

Verify MISE_MEM_STR_READ command success in Cmd_echo.log screen

SCE	From GSEOS MISE tab, in Parameters screen, click Read button.  Take a screenshot of this screen, and attach to the activity report.

Verify MISE_MEM_STR_READ(1) command is dispatched in Cmd.log screen

Verify MISE_MEM_STR_READ command success in Cmd_echo.log screen

SCE	From GSEOS MISE tab, in FPIE Register Settings screen, click Read button.  Take a screenshot of this screen, and attach to the activity report.

Verify MISE_MEM_STR_READ(2) command is dispatched in Cmd.log screen

Verify MISE_MEM_STR_READ command success in Cmd_echo.log screen

SCE	From GSEOS FPIE-FPMC tab, in FPMC Parameters screen, click Read button.  Take a screenshot of this screen, and attach to the activity report.

Verify MISE_FPMEM_STR_READ(0) command is dispatched in Cmd.log screen

Verify MISE_FPMEM_STR_READ command success in Cmd_echo.log screen

## Dump Macros

SCE	Under GSEOS menus, select Core -> Macro -> Read with arguments: First Macro = 0, Last Macro = 255 to dump macro contents.

Verify MISE_MAC_READ(0,255) command is dispatched in Cmd.log screen

Verify MISE_MAC_READ command success in Cmd_echo.log screen

# Day-in-the-Life (DITL) Test

This section of the procedure will direct the test conductor to simulate MISE operations over a Jovian orbit.  A reference diagram is shown below.  This is notionally the order of activities for a nominal flyby.  Note that some orbits may be longer than 14 days.

This test will include jumping the SLCK between the Apoapsis Activities and Flyby Activities. There may be other SCLK jumps throughout the procedure, depending on the test configuration and scope.

Since we do not have spacecraft avionics or spacecraft FSW simulated, many activities performed by the test conductor will be analogous to activities that will be performed by the spacecraft in flight.  Generally, manual commands entered through GSEOS in this test will be issued in flight to MISE by the spacecraft via spacecraft sequencing.  Generally, python script loads entered through GSEOS in this test will be issued in flight to MISE as command file loads from the spacecraft via spacecraft sequencing.

# FM AMBIENT FUNCTIONAL SHIFT DESCRIPTON

For the FM Ambient Functional, this procedure consists solely of apoapsis, cooldown, flyby, and processing of only one cube.  There are 10,000 seconds separating that first processing from subsequent processing sequences.  This procedure will instruct the tester to disable the observation timeline and stop the test after that first processing.

## Apoapsis Activities (Memory Management)

SCE	Time jump to start of apoapsis activities (First Cube - 7days).  Under GSEOS menus, select SCE Cmds -> MET -> Set MET and enter a value of “1074277669.000” to update the mission elapsed time counter.  Click OK.

- Verify GSEOS_CMD_SET_MET(1074277669.000) command is dispatched in Cmd.log screen

-

-

-

### Apoapsis Commanding

SCE	Under GSEOS menus, select Core -> Macro -> Restore to restore macros from non-volatile memory.

Verify MISE_MAC_RESTORE() command is dispatched in Cmd.log screen

Verify MISE_MAC_RESTORE command success in Cmd_echo.log screen

SCE	Run startup macro.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 3.

Verify MISE_MAC_RUN(3) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 5 seconds for macro 3 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	Run the macro to power on the FPMC-only.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 66.

Verify MISE_MAC_RUN(66) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

- Note that two FPMEM_GO command failures are expected.

SCE	Wait approximately 8 seconds for macro 66 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	Run the macro to dump the bad block map.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 76.

Verify MISE_MAC_RUN(76) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 64 seconds for macro 76 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	In the GSEOS Core tab, from the Status window, record the number of uplinked commands.

- Uplink Executed:  ____________

- Uplink Rejected:  ____________

SCE	Load macro to set new bad blocks.  (Note that this macro does not include bad block setting commands; it is a placeholder for this activity.)  Under GSEOS menus, select File -> Open. Select file type “Python Modules (*.py, *.pyd)”, navigate to GitHub folder “C:\Users\mise\GitHub\Procedures\P17868_DITL\Rev B\”, choose the file “P17868_revB_new_bad_block.py” and click Open. The details of this file are included in Appendix A.

Verify commands are dispatched in Cmd.log screen

Verify all commands succeed. The Uplink Executed value should have incremented by 3 commands.  The Uplink Rejected should not have incremented.

SCE	Run the macro to modify the bad block map.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 77.

Verify MISE_MAC_RUN(77) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 2 seconds for macro 77 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	Run the macro to dump the bad block map.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 76.

Verify MISE_MAC_RUN(76) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 64 seconds for macro 76 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	Run the macro to erase the Flash and reset the cube counter.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 78.

Verify MISE_MAC_RUN(78) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 90 seconds for macro 78 to complete.

Verify MISE_NVSTORE_DELETE deferred command success in Cmd_echo.log screen

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	Select appropriate file for frame acquisition type to be used during this test:

| Configuration | Macro File | # Cmds | Selected |
| --- | --- | --- | --- |
| FPA Test Pattern | P17868_revB_MISEMacroDefinitions_FPA_Test_Pattern.py | 6 |  |
| FPIE Test Pattern | P17868_revB_MISEMacroDefinitions_FPIE_Test_Pattern.py | 5 |  |
| Science Frames | P17868_revB_MISEMacroDefinitions_Science_Data.py | 12 |  |
| OGSE Calibration | P17868_revB_MISEMacroDefinitions_OGSE_Cal.py | 12 |  |

SCE	In the GSEOS Core tab, from the Status window, record the number of uplinked commands.

- Uplink Executed:  ____________

- Uplink Rejected:  ____________

SCE	Define startup and test-specific macros.  Under GSEOS menus, select File -> Open. Select file type “Python Modules (*.py, *.pyd)”, navigate to GitHub folder “C:\Users\mise\GitHub\Procedures\P17868_DITL\Rev B\”, choose the file selected in the previous step and click Open. The details of this file are included in Appendix A.

Selected macro file:

Verify commands are dispatched in Cmd.log screen

Verify all commands succeed. The Uplink Executed value should have incremented by the number of commands in the table above for the file sent.  The Uplink Rejected should not have incremented.

SCE	In the GSEOS Core tab, from the Status window, record the number of uplinked commands.

- Uplink Executed:  ____________

- Uplink Rejected:  ____________

SCE	Load processing macros for all configurations.  Under GSEOS menus, select File -> Open. Select file type “Python Modules (*.py, *.pyd)”, navigate to GitHub folder “C:\Users\mise\GitHub\Procedures\P17868_DITL\Rev B\”, choose the file “P17868_revB_66E42_processingMacros.py” and click Open. The details of this file are included in Appendix A.

Verify commands are dispatched in Cmd.log screen

Verify all commands succeed. The Uplink Executed value should have incremented by 309 commands.  The Uplink Rejected should not have incremented.

SCE	In the GSEOS Core tab, from the Status window, record the number of uplinked commands.

- Uplink Executed:  ____________

- Uplink Rejected:  ____________

SCE	Load flyby observation timeline and sequences for all configurations.  Under GSEOS menus, select File -> Open. Select file type “Python Modules (*.py, *.pyd)”, navigate to GitHub folder “C:\Users\mise\GitHub\Procedures\P17868_DITL\Rev B\”, choose the file “P17868_revB_66E42_acquisitionCommands.py” and click Open. The details of this file are included in Appendix A.

Verify commands are dispatched in Cmd.log screen

Verify all commands succeed. The Uplink Executed value should have incremented by 86 commands.  The Uplink Rejected should not have incremented.

SCE	Select appropriate file for MISE configuration and scope:

| Configuration | Macro File | # Cmds | Selected |
| --- | --- | --- | --- |
| Ambient Functional | P17868_revB_66E42_processingCommands_Ambient_Functional.py | 137 |  |
| All Others | P17868_revB_66E42_processingCommands.py | 137 |  |

SCE	In the GSEOS Core tab, from the Status window, record the number of uplinked commands.

- Uplink Executed:  ____________

- Uplink Rejected:  ____________

SCE	Define the observation timeline for processing.  Under GSEOS menus, select File -> Open. Select file type “Python Modules (*.py, *.pyd)”, navigate to GitHub folder “C:\Users\mise\GitHub\Procedures\P17868_DITL\Rev B\”, choose the file selected in the previous step and click Open. The details of this file are included in Appendix A.

Selected macro file:

Verify commands are dispatched in Cmd.log screen

Verify all commands succeed. The Uplink Executed value should have incremented by the number of commands in the table above for the file sent.  The Uplink Rejected should not have incremented.

SCE	Run the macro to save macros, observation sequences, observation timeline, and scan profiles to MRAM.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 79.

Verify MISE_MAC_RUN(79) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 2 seconds for macro 79 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

### Power Off the DPU (for electrical reconfiguration)

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.28)

GSEOS CEU current [A]:    		(FM: 0.00)

Power supply current [A]: 		(FM: 0.28)

- UTC Date & Time: ___ /___ /______T____:____ [UTC]

SCE	Under GSEOS menus, select Core -> Macro -> Run, with argument: Macro = 1 to run the nominal shutdown macro.  Note that this will take approximately one minute to complete.

Verify MISE_MAC_RUN(1) command is dispatched in Cmd.log screen

Verify the following commands are executed successfully in the Cmd_echo.log screen. (NOTE: the # of macros executed should increase by 25; the arguments displayed in the commands below are for reference only and do not appear in the Cmd_echo.log screen):

MISE_MAC_NEST(2)

MISE_OBS_ENB(Disable)

MISE_DET_POWER(Off)

MISE_COOL_POWER(Off)

MISE_COOL_HEATER(Off)

MISE_SCAN_ANGLE(-2318518)

MISE_SCAN_MODE(Slew)

MISE_MAC_DELAY(55)

MISE_SCAN_MODE(Off)

MISE_SCAN_LOOP(CoarseOpen)

Verify MISE_MAC_RUN command success in Cmd_echo.log screen.

Verify two entries for “MISE_MAC_END” at end of Cmd_echo.log screen

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.28)

GSEOS CEU current [A]:    		(FM: 0.00)

Power supply current [A]: 		(FM: 0.28)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

PEER	[PEER WITNESS] Witness DPU power off in the following step.

SCE	From GSEOS Summary tab, in MISE DPU screen, click the DPU left grey button to disable power to DPU.

Verify SCE_CTRL_SET_POWER_DISABLE_5A_CH_1() command is dispatched in Cmd.log screen

SCE	Confirm power to DPU is off and current on power supply is <10mA

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    	(0.00)

GSEOS CEU current [A]:    	(0.00)

Power supply current [A]: 	(0.00)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

PEER	[PEER WITNESS] Verify the DPU is powered OFF.

## Flyby Activities

These activities start roughly at Closest Approach (CA) minus 37 hours.

TBD	This is a placeholder to power on the cryocooler warm-up heaters.  This activity is not performed until TVAC with the RCM is integrated and the heaters connected to a power supply.  Until then, skip this step, including the time jump to CA-37hr.

- Time jump to start of cryocooler warmup heating (First Cube -33 hr, approximately CA-37hr).  Under GSEOS menus, select SCE Cmds -> MET -> Set MET and enter a value of “1074761862.000” to update the mission elapsed time counter.  Click OK.

Verify GSEOS_CMD_SET_MET(1074763669.000) command is dispatched in Cmd.log screen

Verify Time field in Health & Status screen resets to 1074763669

- Wait approximately 5 hours for warm-up heating to complete.  Do not proceed to the next step until the Thermal engineer confirms warm-up is complete.  If the Thermal engineer confirms warm-up is complete prior to 5 hours, the Test Conductor can move forward with the next steps.

SCE	Time jump to start of detector and spectrometer cooldown (First Cube -28 hr, approximately CA-32hr).  Under GSEOS menus, select SCE Cmds -> MET -> Set MET and enter a value of “1074781669.000” to update the mission elapsed time counter.  Click OK.

Verify GSEOS_CMD_SET_MET(1074781669.000) command is dispatched in Cmd.log screen

### Power On the DPU

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.020)

GSEOS CEU current [A]:    		(FM: 0.000)

Power supply current [A]: 		(FM: 0.008)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

-

PEER	[PEER WITNESS] Witness DPU power on and monitor current during power on in the following step.

SCE	From GSEOS Summary tab, in MISE DPU screen, enable Power to DPU (see Figure 4-1).

Verify SCE_CTRL_SET_POWER_ENABLE_5A_CH_1() command is dispatched in Cmd.log screen

SCE	Record Current and Voltage as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS MISE Volts [V]:     		(30.0)

GSEOS DPU current [A]:    		(FM: 0.15)

Power supply current [A]: 		(FM: 0.15)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

PEER	[PEER WITNESS] Verify the DPU is powered on.

SCE	In Boot Status screen, observe time tag is incrementing once per second to confirm program booted correctly.

SCE	In Health & Status screen, observe time tag is incrementing and verify Mode shows Boot.

Mode:	(Boot)

SCE	In MISE DPU screen, click Boot button (see Figure 5-2).

Verify MISE_ROM_BOOT() command is dispatched in Cmd.log screen

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.28)

Power supply current [A]: 		(FM: 0.28)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

SCE	In Boot Status screen confirm time tags stop incrementing.

SCE	In Health & Status screen, confirm Mode changes from Boot to App and confirm time tag continues incrementing. Rejected counter is expected to be 1.

Mode:	(App)

SCE	In Health & Status screen, confirm Accepted and Executed counters are incrementing.

SCE	Under GSEOS menus, select Core -> ROM -> GO. Enter address 0x410000, click OK.

Verify MISE_ROM_GO(0x410000) command is dispatched in Cmd.log screen

Verify MISE_ROM_GO command fails in Cmd_echo.log screen

Verify commands rejected counter incremented by 1

SCE	Under GSEOS menus, select Core -> ROM -> GO. Enter address 0x420000, click OK.

Verify MISE_ROM_GO(0x420000) command is dispatched in Cmd.log screen

Verify MISE_ROM_GO command fails in Cmd_echo.log screen

Verify commands rejected counter incremented by 1

SCE	In Health & Status screen, confirm, Commands Log is updating.

SCE	In Status screen, record the DPU ProcCard FSW version number

ProcCard FSW Version:		(2-22031007)

### Cooldown, Flyby Data Acquisition, and Processing Part 1

SCE	Under GSEOS menus, select Core -> Macro -> Restore to restore macros from non-volatile memory.

Verify MISE_MAC_RESTORE() command is dispatched in Cmd.log screen

Verify MISE_MAC_RESTORE command success in Cmd_echo.log screen

SCE	Run startup macro.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 3.

Verify MISE_MAC_RUN(3) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

Wait approximately 5 seconds for macro 3 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	Run the macro to power on the DPU-controlled cryocooler operational heater.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 71.

Verify MISE_MAC_RUN(71) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 2 seconds for macro 71 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	In the MISE tab and the Parameters window, verify the DPU parameters listed below.

- Cryocooler Temp Thresh: ____________ (expect -78 degC, DN 1553)

- Cryocooler Temp Hysteresis: __________ (expect 5 degC, DN 46)

- Note:  With these settings, the heater will not power on in the Ambient Functional test.

SCE 	Power on the CEU.  From the GSEOS Summary tab, in DPU screen click the green button to enable CEU power.  This will have the EGSE apply power to the CEU.  (Note that power on the CEU before commanding the DPU to start the CEU is opposite from the spacecraft-commanded behavior in flight.)

Verify SCE_CTRL_SET_POWER_ENABLE_5A_CH_2() command is dispatched in Cmd.log screen

- NOTE: THIS STEP IS NOT FLIGHT-LIKE.  WHEN IN THE STB OR INTEGRATED TO THE S/C, THE S/C WILL POWER ON THE CEU IN RESPONSE TO THE MACRO BELOW.

PEER	[PEER WITNESS] Witness DPU power off in the following step.

SCE	Run the macro to power on the cryocooler into temperature-control mode.  (NOTE: The command to apply power to the cryocooler is only in the script P17868_setup_CAUTION_FM_RCM_TVAC_ONLY.py.  If this script was not run earlier, then power will not be applied to the cryocooler.)  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 67.

Verify MISE_MAC_RUN(67) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 10 seconds for macro 67 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	In the Health and Status tab, verify the telemetry listed below.

- Autonomy Bit 2: ____________ (expected 1, shown as a green light under bit 2)

- Cryocooler Temp Setpoint (“Temp Goal” in the Cooler window of the Cooler tab): __________ (expected -188 degC / 929 DN in TVAC, 30 degC / 142 DN all other tests)

- Record Cryocooler Control Temp is decreasing (TVAC-only)

SCE	[This step is n/a for the Ambient Functional.] Wait approximately 20 hours for the cryocooler to cool the detector and spectrometer to operating temperatures.  Do not proceed to the next step until the Thermal engineer confirms cooldown is complete.  If the Thermal engineer confirms cooldown is complete prior to 20 hours, the Test Conductor can move forward with the next steps.

SCE	Time jump to start of FPA power on for cooldown with the additional heat load (First Cube -8 hr, approximately CA-12hr).  Under GSEOS menus, select SCE Cmds -> MET -> Set MET and enter a value of “1074853669.000” to update the mission elapsed time counter.  Click OK.

Verify GSEOS_CMD_SET_MET(1074853669.000) command is dispatched in Cmd.log screen

Verify Time field in Health & Status screen resets to 1074853669

SCE	Run the macro to power on the FPMC and FPIE.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 62.

Verify MISE_MAC_RUN(62) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 8 seconds for macro 62 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	Run the macro to power on the FPA.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 63.

Verify MISE_MAC_RUN(63) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 3 seconds for macro 63 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

- NOTE:  THIS REVISION OF THIS PROCEDURE DOES NOT INCLUDE FILE UPLOADS TO SIMULATE AN EPHEMERIS UPDATE.  THE FILE UPLOADS MUST COMPLETE BEFORE THE NEXT STEP.  AFTER UPLOADS ARE COMPLETE, MACRO 70 MUST BE RUN.

SCE	[This step is n/a for the Ambient Functional.]  Wait for approximately 3 hours.

SCE	[This step is n/a for the Ambient Functional.]  Placeholder for simulating the ephemeris update.

SCE	[This step is n/a for the Ambient Functional.]  Wait for approximately 4 hours.

SCE	[This step is n/a for the Ambient Functional.]  Do not proceed to the next step until the Thermal engineer confirms cooldown is complete.  If the Thermal engineer confirms cooldown is complete prior to 20 hours, the Test Conductor can move forward with the next steps.

SCE	[This step applies ONLY to the Ambient Functional.]  Time jump to what would be the end of the ephemeris update (First Cube -1 hr, approximately CA-5hr).  Under GSEOS menus, select SCE Cmds -> MET -> Set MET and enter a value of “1074878869.000” to update the mission elapsed time counter.  Click OK.

Verify GSEOS_CMD_SET_MET(1074878869.000) command is dispatched in Cmd.log screen

Verify Time field in Health & Status screen resets to 1074878869

SCE	Under GSEOS menus, select Core -> Macro -> Restore to restore macros from non-volatile memory

Verify MISE_MAC_RESTORE() command is dispatched in Cmd.log screen

Verify MISE_MAC_RESTORE command success in Cmd_echo.log screen

SCE	Run the macro to restore scan profiles, observation sequences, and the observation timeline.  Under GSEOS menus, select Core -> Macro -> Run… with argument “macro” = 70.

Verify MISE_MAC_RUN(70) command is dispatched in Cmd.log screen

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Wait approximately 2 seconds for macro 70 to complete.

Verify MISE_MAC_END command success in Cmd_echo.log screen

SCE	Enable observation timeline.  Under GSEOS menus, select MISE -> Observation -> Autonomous -> Enable.

Verify MISE_OBS_ENB(Enable) command is dispatched in Cmd.log screen

Verify MISE_OBS_ENB command success in Cmd_echo.log screen

SCE	[This step applies ONLY to the Ambient Functional.]  Time jump to 2 minutes before the first cube (approximately CA-4hr).  Under GSEOS menus, select SCE Cmds -> MET -> Set MET and enter a value of “1074882349.000” to update the mission elapsed time counter.  Click OK.

Verify GSEOS_CMD_SET_MET(1074882349.000) command is dispatched in Cmd.log screen

Verify Time field in Health & Status screen resets to 1074882349

- Two minutes after this time jump, the observation timeline will begin to acquire science data.  This includes moving the scan mirror.  There are 10 cubes and 10 scan motions in this timeline.  It will take approximately 5 hours to complete this observation.  After these observations complete, the timeline will power off the cryocooler and cryocooler operational heater.  It will also power off the FPIE and FPA.

- [This step still applies ONLY to the Ambient Functional.]  At TC direction, the time jumps below are also acceptable.  These jumps skip the idle time between MISE observations.  For completed time jumps, initial or N/A the row.

| - Around the MET below, … | - …jump to the MET below. | - Initial or N/A |
| --- | --- | --- |
| - 1074883075 | - 1074884825 |  |
| - 1074885510 | - 1074890481 |  |
| - 1074891470 | - 1074893483 |  |
| - 1074894125 | - 1074896156 |  |
| - 1074897940 | - 1074899315 |  |

- NOTE: Revision B of this procedure processes only one cube.  The observation timeline has 10,000 seconds between that cube and the following.  During those 10,000 seconds, the observation timeline will be disabled (by the shutdown macro) and the test will be concluded.

SCE	At MET 1074899974, look for the H&S CEU Power bit to change to 1 (power off CEU).  Once that happens, open the CEU power switch.  From the GSEOS Summary tab, in DPU screen, click the gray button next to “CEU” to remove power from the CEU switch.

Verify SCE_CTRL_SET_POWER_DISABLE_5A_CH_2() command is dispatched in Cmd.log screen

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.34)

GSEOS CEU current [A]:    		(FM: 0.00)

Power supply current [A]: 		(FM: 0.31)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

SCE	At MET 1074899984 (10 seconds after the MET above), in the Detector screen, verify that the FPIE and FPA are off, and the FPMC is on.

TBD	This is a placeholder to power off the cryocooler warm-up heaters.  This activity is not performed until the RCM is integrated and the heaters are connected to a power supply.  Until then, skip this step, including the time jump to CA-37hr.

SCE	At MET 1074900493, the first cube processing activity should be complete.  Verify that macro 100 was the most recently completed macro.  Verify in the Cmd_echo.log that the commands listed below were the most recently completed command.

- 'MISE_ACCOUNT_START', 320266, 1

- 'MISE_NVSTORE_READ',   9,  'Frame', 'Disable',   0,   485

- 'MISE_ACCOUNT_END'

- 'MISE_MAC_END'

- NOTE:  MISE_NVSTORE_READ AND MISE_ACCOUNT_END ARE A DEFERRED COMMANDS.  LOOK FOR THE COMPLETION OF THE DEFERRED COMMAND ECHOS AFTER THE MISE_MAC_END COMMAND ECHO.

- NOTE: AFTER THIS THIS STEP, THE TEST CONDUCTOR HAS ABOUT 10,000 SECONDS (UNTIL MET 1074899803) TO POWER OFF THE DPU.

### Power Off the DPU

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.28)

GSEOS CEU current [A]:    		(FM: 0.00)

Power supply current [A]: 		(FM: 0.28)

- UTC Date & Time: ___ /___ /______T____:____ [UTC]

SCE	Under GSEOS menus, select Core -> Macro -> Run, with argument: Macro = 1 to run the nominal shutdown macro.  Note that this will take approximately one minute to complete.

Verify MISE_MAC_RUN(1) command is dispatched in Cmd.log screen

Verify the following commands are executed successfully in the Cmd echo.log screen:

MISE_MAC_NEST(2)

MISE_OBS_ENB(Disable)

MISE_DET_POWER(Off)

MISE_COOL_POWER(Off)

MISE_COOL_HEATER(OfF)

MISE_SCAN_ANGLE(-2318518)

MISE_SCAN_MODE(Slew)

MISE_MAC_DELAY(55)

MISE_SCAN_MODE(Off)

MISE_SCAN_LOOP(CoarseOpen)

MISE_MAC_END

MISE_MAC_END (note, this command will complete twice, once for macro 1 and again for macro 2)

Verify MISE_MAC_RUN command success in Cmd_echo.log screen

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.28)

GSEOS CEU current [A]:    		(FM: 0.00)

Power supply current [A]: 		(FM: 0.28)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

SCE	From GSEOS Summary tab, in MISE DPU screen, click the DPU left grey button to disable power to DPU.

Verify SCE_CTRL_SET_POWER_DISABLE_5A_CH_1() command is dispatched in Cmd.log screen

SCE	Confirm power to DPU is off and current on power supply is <10mA.

SCE	Record Current as displayed on power supply and in GSEOS Summary tab, DPU screen.

GSEOS DPU current [A]:    		(FM: 0.28)

GSEOS CEU current [A]:    		(FM: 0.00)

Power supply current [A]: 		(FM: 0.28)

UTC Date & Time: ___ /___ /______T____:____ [UTC]

PEER	[PEER WITNESS] Verify the DPU is powered OFF.

# Cleanup

Requirements Verified:

- N/A

Objectives:

- Shutdown SCE and archive data

## Power-Off SCE

SCE	Shutdown GSEOS and spacecraft emulator per MISE Standard Operating Procedure P18247, Section 5:

P18247 Instance ID:

SCE 	Complete Test Log

Cryocooler Setpoint Conversion

| Temp (K) | T1 DN stpt | T2 DN stpt |
| --- | --- | --- |
| 20 | 8746 | 8806 |
| 21 | 8086 | 8139 |
| 22 | 7506 | 7553 |
| 23 | 6992 | 7034 |
| 24 | 6535 | 6573 |
| 25 | 6126 | 6160 |
| 26 | 5758 | 5789 |
| 27 | 5425 | 5453 |
| 28 | 5124 | 5149 |
| 29 | 4849 | 4873 |
| 30 | 4598 | 4620 |
| 31 | 4369 | 4389 |
| 32 | 4157 | 4176 |
| 33 | 3962 | 3980 |
| 34 | 3782 | 3799 |
| 35 | 3616 | 3631 |
| 36 | 3461 | 3475 |
| 37 | 3317 | 3330 |
| 38 | 3182 | 3195 |
| 39 | 3057 | 3069 |
| 40 | 2939 | 2951 |
| 41 | 2829 | 2840 |
| 42 | 2726 | 2736 |
| 43 | 2629 | 2638 |
| 44 | 2537 | 2546 |
| 45 | 2451 | 2460 |
| 46 | 2369 | 2378 |
| 47 | 2292 | 2300 |
| 48 | 2219 | 2227 |
| 49 | 2150 | 2157 |
| 50 | 2084 | 2091 |
| 51 | 2022 | 2029 |
| 52 | 1963 | 1969 |
| 53 | 1906 | 1913 |
| 54 | 1852 | 1859 |
| 55 | 1801 | 1807 |
| 56 | 1752 | 1758 |
| 57 | 1705 | 1711 |
| 58 | 1661 | 1666 |
| 59 | 1618 | 1623 |
| 60 | 1577 | 1582 |
| 61 | 1538 | 1543 |
| 62 | 1500 | 1505 |
| 63 | 1464 | 1469 |
| 64 | 1429 | 1434 |
| 65 | 1396 | 1400 |
| 66 | 1364 | 1368 |
| 67 | 1333 | 1337 |
| 68 | 1303 | 1308 |
| 69 | 1275 | 1279 |
| 70 | 1247 | 1251 |
| 71 | 1221 | 1225 |
| 72 | 1195 | 1199 |
| 73 | 1170 | 1174 |
| 74 | 1146 | 1150 |
| 75 | 1123 | 1127 |
| 76 | 1101 | 1105 |
| 77 | 1079 | 1083 |
| 77.35 | 1072 | 1075 |
| 78 | 1059 | 1062 |
| 79 | 1038 | 1042 |
| 80 | 1019 | 1022 |
| 81 | 1000 | 1003 |
| 82 | 981 | 985 |
| 83 | 964 | 967 |
| 84 | 946 | 949 |
| 85 | 929 | 933 |
| 86 | 913 | 916 |
| 87 | 897 | 900 |
| 88 | 882 | 885 |
| 89 | 867 | 870 |
| 90 | 853 | 855 |
| 91 | 839 | 841 |
| 92 | 825 | 828 |
| 93 | 811 | 814 |
| 94 | 798 | 801 |
| 95 | 786 | 788 |
| 96 | 774 | 776 |
| 97 | 762 | 764 |
| 98 | 750 | 752 |
| 99 | 738 | 741 |
| 100 | 727 | 730 |
| 101 | 717 | 719 |
| 102 | 706 | 708 |
| 103 | 696 | 698 |
| 104 | 686 | 688 |
| 105 | 676 | 678 |
| 106 | 666 | 669 |
| 107 | 657 | 659 |
| 108 | 648 | 650 |
| 109 | 639 | 641 |
| 110 | 630 | 632 |
| 111 | 622 | 624 |
| 112 | 613 | 616 |
| 113 | 605 | 607 |
| 114 | 597 | 599 |
| 115 | 589 | 592 |
| 116 | 582 | 584 |
| 117 | 574 | 576 |
| 118 | 567 | 569 |
| 119 | 560 | 562 |
| 120 | 553 | 555 |
| 121 | 546 | 548 |
| 122 | 539 | 541 |
| 123 | 533 | 535 |
| 124 | 526 | 528 |
| 125 | 520 | 522 |
| 126 | 514 | 516 |
| 127 | 508 | 510 |
| 128 | 502 | 504 |
| 129 | 496 | 498 |
| 130 | 490 | 492 |
| 131 | 485 | 487 |
| 132 | 479 | 481 |
| 133 | 474 | 476 |
| 134 | 468 | 470 |
| 135 | 463 | 465 |
| 136 | 458 | 460 |
| 137 | 453 | 455 |
| 138 | 448 | 450 |
| 139 | 443 | 445 |
| 140 | 439 | 440 |
| 141 | 434 | 436 |
| 142 | 429 | 431 |
| 143 | 425 | 427 |
| 144 | 421 | 422 |
| 145 | 416 | 418 |
| 146 | 412 | 414 |
| 147 | 408 | 409 |
| 148 | 404 | 405 |
| 149 | 400 | 401 |
| 150 | 396 | 397 |
| 151 | 392 | 393 |
| 152 | 388 | 389 |
| 153 | 384 | 386 |
| 154 | 380 | 382 |
| 155 | 377 | 378 |
| 156 | 373 | 375 |
| 157 | 370 | 371 |
| 158 | 366 | 368 |
| 159 | 363 | 364 |
| 160 | 359 | 361 |
| 161 | 356 | 357 |
| 162 | 353 | 354 |
| 163 | 349 | 351 |
| 164 | 346 | 348 |
| 165 | 343 | 345 |
| 166 | 340 | 342 |
| 167 | 337 | 339 |
| 168 | 334 | 336 |
| 169 | 331 | 333 |
| 170 | 328 | 330 |
| 171 | 325 | 327 |
| 172 | 323 | 324 |
| 173 | 320 | 321 |
| 174 | 317 | 318 |
| 175 | 314 | 316 |
| 176 | 312 | 313 |
| 177 | 309 | 311 |
| 178 | 307 | 308 |
| 179 | 304 | 305 |
| 180 | 301 | 303 |
| 181 | 299 | 300 |
| 182 | 297 | 298 |
| 183 | 294 | 296 |
| 184 | 292 | 293 |
| 185 | 289 | 291 |
| 186 | 287 | 289 |
| 187 | 285 | 286 |
| 188 | 283 | 284 |
| 189 | 280 | 282 |
| 190 | 278 | 280 |
| 191 | 276 | 277 |
| 192 | 274 | 275 |
| 193 | 272 | 273 |
| 194 | 270 | 271 |
| 195 | 268 | 269 |
| 196 | 266 | 267 |
| 197 | 264 | 265 |
| 198 | 262 | 263 |
| 199 | 260 | 261 |
| 200 | 258 | 259 |
| 201 | 256 | 257 |
| 202 | 254 | 256 |
| 203 | 252 | 254 |
| 204 | 251 | 252 |
| 205 | 249 | 250 |
| 206 | 247 | 248 |
| 207 | 245 | 247 |
| 208 | 244 | 245 |
| 209 | 242 | 243 |
| 210 | 240 | 241 |
| 211 | 239 | 240 |
| 212 | 237 | 238 |
| 213 | 235 | 236 |
| 214 | 234 | 235 |
| 215 | 232 | 233 |
| 216 | 230 | 232 |
| 217 | 229 | 230 |
| 218 | 227 | 229 |
| 219 | 226 | 227 |
| 220 | 224 | 226 |
| 221 | 223 | 224 |
| 222 | 221 | 223 |
| 223 | 220 | 221 |
| 224 | 219 | 220 |
| 225 | 217 | 218 |
| 226 | 216 | 217 |
| 227 | 214 | 216 |
| 228 | 213 | 214 |
| 229 | 212 | 213 |
| 230 | 210 | 211 |
| 231 | 209 | 210 |
| 232 | 208 | 209 |
| 233 | 206 | 208 |
| 234 | 205 | 206 |
| 235 | 204 | 205 |
| 236 | 203 | 204 |
| 237 | 201 | 203 |
| 238 | 200 | 201 |
| 239 | 199 | 200 |
| 240 | 198 | 199 |
| 241 | 197 | 198 |
| 242 | 195 | 197 |
| 243 | 194 | 195 |
| 244 | 193 | 194 |
| 245 | 192 | 193 |
| 246 | 191 | 192 |
| 247 | 190 | 191 |
| 248 | 189 | 190 |
| 249 | 188 | 189 |
| 250 | 187 | 188 |
| 251 | 186 | 187 |
| 252 | 184 | 185 |
| 253 | 183 | 184 |
| 254 | 182 | 183 |
| 255 | 181 | 182 |
| 256 | 180 | 181 |
| 257 | 179 | 180 |
| 258 | 178 | 179 |
| 259 | 177 | 178 |
| 260 | 176 | 177 |
| 261 | 175 | 176 |
| 262 | 175 | 176 |
| 263 | 174 | 175 |
| 264 | 173 | 174 |
| 265 | 172 | 173 |
| 266 | 171 | 172 |
| 267 | 170 | 171 |
| 268 | 169 | 170 |
| 269 | 168 | 169 |
| 270 | 167 | 168 |
| 271 | 166 | 167 |
| 272 | 166 | 166 |
| 273 | 165 | 166 |
| 273.15 | 165 | 166 |
| 274 | 164 | 165 |
| 275 | 163 | 164 |
| 276 | 162 | 163 |
| 277 | 161 | 162 |
| 278 | 161 | 161 |
| 279 | 160 | 161 |
| 280 | 159 | 160 |
| 281 | 158 | 159 |
| 282 | 157 | 158 |
| 283 | 157 | 158 |
| 284 | 156 | 157 |
| 285 | 155 | 156 |
| 286 | 154 | 155 |
| 287 | 154 | 154 |
| 288 | 153 | 154 |
| 289 | 152 | 153 |
| 290 | 151 | 152 |
| 291 | 151 | 152 |
| 292 | 150 | 151 |
| 293 | 149 | 150 |
| 294 | 149 | 149 |
| 295 | 148 | 149 |
| 296 | 147 | 148 |
| 297 | 146 | 147 |
| 298 | 146 | 147 |
| 299 | 145 | 146 |
| 300 | 144 | 145 |
| 301 | 144 | 145 |
| 302 | 143 | 144 |
| 303 | 142 | 143 |
| 304 | 142 | 143 |
| 305 | 141 | 142 |
| 306 | 141 | 141 |
| 307 | 140 | 141 |
| 308 | 139 | 140 |
| 309 | 139 | 140 |
| 310 | 138 | 139 |
| 311 | 137 | 138 |
| 312 | 137 | 138 |
| 313 | 136 | 137 |
| 314 | 136 | 137 |
| 315 | 135 | 136 |
| 316 | 135 | 135 |
| 317 | 134 | 135 |
| 318 | 133 | 134 |
| 319 | 133 | 134 |
| 320 | 132 | 133 |
| 321 | 132 | 132 |
| 322 | 131 | 132 |
| 323 | 131 | 131 |
| 324 | 130 | 131 |
| 325 | 129 | 130 |

End of Procedure