| Event ID |  | 020A8B01 |  |
| --- | --- | --- | --- |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Media scan started |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE: CHL:%.1d ID:%.1d Starting Media Scan |  |
| Cause |  | Media scan on a disk drive started. |  |
| Action |  |  |  |
| Event ID |  | 020A8B02 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Media scan started |  |
| 1st Line Message |  | Scan mode: # |  |
| Old Message |  | ID:%.1x NOTICE: Starting Media Scan(%d) |  |
| Cause |  | Media scan on a disk drive started. |  |
| Action |  |  |  |
| Event ID |  | 020A8B03 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Media scan started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x NOTICE: CHL:%.1d ID:%.1d Starting Media Scan |  |
| Cause |  | Media scan on a disk drive started. |  |
| Action |  |  |  |
| Event ID |  | 02088C01 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Media scan completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE: Media Scan of CHL:%.1d ID:%.1d Completed |  |
| Cause |  | Media scan on a disk drive completed. |  |
| Action |  |  |  |
| Event ID |  | 020A8C02 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Media scan completed. Media error: #, recovered error: # |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%lx Logical Drive NOTICE: Media Scan Completed (%d,%d) |  |
| Cause |  | Media scan on a disk drive completed. |  |
| Action |  |  |  |
| Event ID |  | 020A8C03 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Media scan completed. Media error: #, recovered error: # |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x NOTICE: Media Scan of CHL:%.1d ID:%.1d Completed (%d,%d) |  |
| Cause |  | Media scan on a disk drive completed. |  |
| Action |  |  |  |
| Event ID |  | 02081302 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Drive media error - # (#) |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Media Error Encountered - 0x%s (%02x/%02x/%02x) |  |
| Cause |  | Media errors found in disk drives. |  |
| Action |  | If this message occurs frequently, replace the disk drive. |  |
| Event ID |  | 02081301 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Drive media error - # |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Media Error Encountered - 0x%s |  |
| Cause |  | Media errors found in disk drives. |  |
| Action |  | If this message occurs frequently, replace the disk drive. |  |
| Event ID |  | 01081902 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Drive error recovered - # (#) |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive NOTICE: Drive Recovered Error - 0x%s (%02x/%02x/%02x) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 01081901 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Drive error recovered - # |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive NOTICE: Drive Recovered Error - 0x%s |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020B4501 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume status back to normal |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Back to Normal |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A4507 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive status back to normal |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Back to Normal |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02081701 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive error block reassigned - # |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive NOTICE: Block Successfully Reassigned - 0x%s |  |
| Cause |  | Writing to drives reassigned due to bad blocks. Media errors found in disk drives. |  |
| Action |  | If this message occurs frequently, replace the disk drive |  |
| Event ID |  | 0105A204 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure fan back to normal |  |
| 1st Line Message |  | I2C device back to normal (#) |  |
| Old Message |  | Peripheral Device Notification (F%02d) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0105A211 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # fan # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status |  |
| Old Message |  | SES(C%d I%d)Cooling Fan %d NOTICE: Fan Back On-Line |  |
| Cause |  | Missing or failed fan back online. |  |
| Action |  |  |  |
| Event ID |  | 0107A301 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | CPU temperature back to normal |  |
| 1st Line Message |  | Low temperature raised to #(C) |  |
| Old Message |  | NOTICE:CPU Cold Temperature Back To Normal(%d.0(C)) |  |
| Cause |  | CPU low temperature back within normal levels. |  |
| Action |  |  |  |
| Event ID |  | 0107A302 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | CPU temperature back to normal |  |
| 1st Line Message |  | High temperature down to #(C) |  |
| Old Message |  | NOTICE:CPU Hot Temperature Back To Normal(%d.0(C)) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0107A304 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller temperature back to normal |  |
| 1st Line Message |  | ASIC high temperature down to #(C) |  |
| Old Message |  | NOTICE:Controller Hot Temperature Back To Normal(1)(%d.0(C)) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0107A306 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller temperature back to normal |  |
| 1st Line Message |  | Drive channel I/O chip high temperature down to #(C) |  |
| Old Message |  | NOTICE:Controller Hot Temperature Back To Normal(2)(%d.0(C)) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0107A308 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller temperature back to normal |  |
| 1st Line Message |  | Host board I/O chip high temperature down to #(C) |  |
| Old Message |  | NOTICE:Controller Hot Temperature Back To Normal(3)(%d.0(C)) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0107A303 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure backplane temperature sensor detected |  |
| 1st Line Message |  | I2C device back to normal (#) |  |
| Old Message |  | Peripheral Device Notification (T%02d) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0107A310 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # backplane temperature back to normal |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Temp Sensor %d NOTICE: Temperature Back To Normal |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0107A311 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure backplane temperature back to normal |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Device NOTICE: Temperature Back To Normal |  |
| Cause |  | Backplane temperature within normal levels. |  |
| Action |  |  |  |
| Event ID |  | 0103A108 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure power supply sensor back to normal |  |
| 1st Line Message |  | I2C device back to normal (#) |  |
| Old Message |  | Peripheral Device Notification (P%02d) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0103A111 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # power supply # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status |  |
| Old Message |  | SES(C%d I%d)Power Supply %d NOTICE: Power Supply Back On-Line |  |
| Cause |  | Power supply successfully replaced. |  |
| Action |  |  |  |
| Event ID |  | 02011104 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Redundant controller firmware updated |  |
| 1st Line Message |  |  |  |
| Old Message |  | Redundant Controller Firmware Updated |  |
| Cause |  | Firmware for the redundant controller updated |  |
| Action |  | Reset the controller. |  |
| Event ID |  | 020B8301 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume rebuild resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Continue Rebuild Operation |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8302 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive rebuild resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Continue Rebuild Operation |  |
| Cause |  | Rebuilding a logical drive resumed 1. Rebuilding was interrupted due to system reset 2. Media errors corrected while logical drive is rebuilding. |  |
| Action |  |  |  |
| Event ID |  | 020B8303 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume rebuild started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Starting Rebuild |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8304 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive rebuild started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Starting Rebuild |  |
| Cause |  | Rebuilding a logical drive started 1. A member of a logical drive failed 2. Faulty drive replaced. |  |
| Action |  |  |  |
| Event ID |  | 020B8901 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume migration resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Continue Migrate Operation |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8902 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive RAID migration resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Continue Migrate Operation |  |
| Cause |  | Logical drive RAID level migration ongoing. |  |
| Action |  | No action required. The logical drive has been restored to its previous status due to a power-out or media errors, allowing the operation to continue. |  |
| Event ID |  | 020B8903 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume add drive action resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Continue Add Drive Operation |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8904 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive add drive action resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Continue Add Drive Operation |  |
| Cause |  | Adding a disk drive to a logical drive ongoing. |  |
| Action |  | No action required. The logical drive has been restored to its previous status due to a power-out or media errors, allowing the operation to continue. |  |
| Event ID |  | 020B8905 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume migration started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Starting Migrate Operation |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8906 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive RAID migration started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Starting Migrate Operation |  |
| Cause |  | Logical drive RAID level migration started. |  |
| Action |  |  |  |
| Event ID |  | 020B8907 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume add drive action started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Starting Add Drive Operation |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8908 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive add drive action started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Starting Add Drive Operation |  |
| Cause |  | Adding a disk to a logical drive started. |  |
| Action |  |  |  |
| Event ID |  | 020B8A05 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume migration completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Migrate to Logical Volume Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8A06 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive RAID migration completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Migrate to Logical Drive Completed |  |
| Cause |  | Logical drive RAID level migration completed. |  |
| Action |  |  |  |
| Event ID |  | 020B8A07 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume add drive action completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Add Drive to Logical Volume Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8A08 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive add drive completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Add Drive to Logical Drive Completed |  |
| Cause |  | Adding a disk drive to a logical drive completed. |  |
| Action |  |  |  |
| Event ID |  | 020B8701 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume online expansion started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Starting On-Line Expansion |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8702 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive online expansion started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Starting On-Line Expansion |  |
| Cause |  | Expanding a logical drive started. |  |
| Action |  | No action required. Online expansion allows additional capacity of a logical drive to appear immediately as a usable partition, ready for host I/O. This partition may need to be associated with a separate ID/LUN on the host bus. |  |
| Event ID |  | 020B8703 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume offline expansion started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Starting Off-Line Expansion |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8704 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive offline expansion started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Starting Off-Line Expansion |  |
| Cause |  | Offline expansion of a logical drive started. |  |
| Action |  | No action required. Offline expansion is faster than online expansion but halts host I/O to expanded partitions until the process is complete. Access to existing data will not be affected. |  |
| Event ID |  | 020B8801 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume online expansion completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: On-Line Expansion of Logical Volume Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8802 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive online expansion completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: On-Line Expansion of Logical Drive Completed |  |
| Cause |  | Online expansion of a logical drive completed. |  |
| Action |  | No action required. Additional capacity appears as a new partition, which has to be mapped as a separate ID/LUN on the host bus. Adjustments should also be made in the host OS volume management utility. |  |
| Event ID |  | 020A0501 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive expansion aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Expansion Aborted |  |
| Cause |  | Logical drive expansion operation aborted because user cancels off-line initialization. |  |
| Action |  | Users can restart the Logical drive expansion manually. |  |
| Event ID |  | 020B8803 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume offline expansion completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Off-Line Expansion of Logical Volume Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8804 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive offline expansion completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Off-Line Expansion of Logical Drive Completed |  |
| Cause |  | Offline expansion of a logical drive completed. |  |
| Action |  | No action required. Offline expansion is faster than online expansion but halts host I/O to expanded partitions until the process is complete. Access to existing data will not be affected. |  |
| Event ID |  | 020B8401 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume rebuild completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Rebuild of Logical Volume Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8402 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive rebuild completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Rebuild of Logical Drive Completed |  |
| Cause |  | Rebuilding a logical drive completed. |  |
| Action |  |  |  |
| Event ID |  | 020A0301 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive rebuild aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Rebuild Aborted |  |
| Cause |  | User cancelled the operation. |  |
| Action |  | Restart rebuild operation manually. |  |
| Event ID |  | 020B8501 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume parity regeneration resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Continue Parity Regeneration Operation |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8502 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive parity regeneration resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Continue Parity Regeneration Operation |  |
| Cause |  | Parity regeneration ongoing. |  |
| Action |  |  |  |
| Event ID |  | 020B8503 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume parity regeneration started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Starting Parity Regeneration |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8504 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive parity regeneration started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Starting Parity Regeneration |  |
| Cause |  | Parity regeneration started. |  |
| Action |  |  |  |
| Event ID |  | 020B8601 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume parity regeneration completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Parity Regeneration of Logical Volume Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8602 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive parity regeneration completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Parity Regeneration of Logical Drive Completed |  |
| Cause |  | Parity regeneration completed. |  |
| Action |  |  |  |
| Event ID |  | 020B8101 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume online initialization started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Starting On-Line Initialization |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8102 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive online initialization started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Starting On-Line Initialization |  |
| Cause |  | Online initialization for a logical drive started. |  |
| Action |  | No action required. Online initialization takes time to complete and allows the logical drive to run concurrent host I/O. |  |
| Event ID |  | 020B8103 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume offline initialization started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Starting Off-Line Initialization |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8104 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive offline initialization started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Starting Off-Line Initialization |  |
| Cause |  | Offline initialization for a logical drive started. |  |
| Action |  | No action required. Offline initialization completes nearly instantly but does not allow the logical drive to run concurrent host I/O. |  |
| Event ID |  | 020B8201 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume online initialization completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: On-Line Initialization of Logical Volume Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8202 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive online initialization completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: On-Line Initialization of Logical Drive Completed |  |
| Cause |  | Online initialization of a logical drive has completed. |  |
| Action |  |  |  |
| Event ID |  | 020B8203 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume offline initialization completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Off-Line Initialization of Logical Volume Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8204 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive offline initialization completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Off-Line Initialization of Logical Drive Completed |  |
| Cause |  | Offline initialization of a logical drive has completed. |  |
| Action |  | No action required. In offline initialization, host I/O can start after initialization has completed. |  |
| Event ID |  | 020B8105 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume creation started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Starting Creation |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8106 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive creation started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Starting Creation |  |
| Cause |  | Creating a logical drive started. |  |
| Action |  | No action required. Creating a logical drive and initializing it are separate processes. The former finishes in seconds but the latter might take hours depending on storage capacity. |  |
| Event ID |  | 020B8205 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume created |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Creation of Logical Volume Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A8206 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive created |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Creation of Logical Drive Completed |  |
| Cause |  | Creating a logical drive completed. |  |
| Action |  |  |  |
| Event ID |  | 02084301 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Drive media error recovered - # |  |
| Old Message |  | CHL:%.1d ID:%.1d NOTICE:Media Error Recovered-0x%s |  |
| Cause |  | Data on bad blocks restored. |  |
| Action |  |  |  |
| Event ID |  | 0208C101 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive scanned |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive NOTICE: Scan Drive Successful |  |
| Cause |  | Scanning disk drive completed without errors. |  |
| Action |  | No action required. |  |
| Event ID |  | 02018901 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller NVRAM factory default settings restored |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE:NVRAM Factory Defaults Restored |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02018902 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller NVRAM restore from file completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE:NVRAM Restore From File is Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02018903 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller NVRAM restore from drive completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE:NVRAM Restore From Disk is Completed |  |
| Cause |  | Factory default settings restored. |  |
| Action |  |  |  |
| Event ID |  | 020AA101 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Drive clone resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d Continue Clone Operation |  |
| Cause |  | Cloning a drive in a logical drive ongoing. |  |
| Action |  |  |  |
| Event ID |  | 020AA102 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Drive clone started |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d Starting Clone |  |
| Cause |  | Cloning a drive in a logical drive started. |  |
| Action |  |  |  |
| Event ID |  | 020AA201 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Drive copy and replace completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d Copy and Replace Completed |  |
| Cause |  | Copying and replacing a drive in a logical drive completed. |  |
| Action |  |  |  |
| Event ID |  | 020AA202 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Drive clone completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d Clone Completed |  |
| Cause |  | Cloning a drive in a logical drive completed. |  |
| Action |  |  |  |
| Event ID |  | 02018101 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller initialization completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | Controller Initialization Completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02018102 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller slot B booted as primary controller |  |
| 1st Line Message |  |  |  |
| Old Message |  | Controller boots up from slot B as primary controller |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02093F0A |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Drive channel error recovered |  |
| 1st Line Message |  | Fibre channel loop error recovered |  |
| Old Message |  | CHL:%.1d NOTICE:Fibre Channel Loop Connection Restored |  |
| Cause |  | Fibre channel loop connection restored. |  |
| Action |  | Check host and drive side activity. |  |
| Event ID |  | 02093F0B |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Drive channel error recovered |  |
| 1st Line Message |  | Redundant path error recovered: # |  |
| Old Message |  | CHL:%.1d NOTICE:Redundant Path for Chl:%.1d Restored |  |
| Cause |  | Redundant paths restored. |  |
| Action |  | Check host and drive side activity. |  |
| Event ID |  | 02093F0C |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Drive channel error recovered |  |
| 1st Line Message |  | Redundant path error recovered: # # |  |
| Old Message |  | CHL:%.1d NOTICE:Redundant Path for Chl:%.1d ID:%.1d Restored |  |
| Cause |  | Redundant paths restored. |  |
| Action |  | Check host and drive side activity. |  |
| Event ID |  | 02093F0D |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive channel error recovered |  |
| 1st Line Message |  | Redundant path error recovered: # # |  |
| Old Message |  | CHL:%.1d ID:%.1d NOTICE:Redundant Path for Chl:%.1d ID:%.1d Restored |  |
| Cause |  | Redundant paths restored. |  |
| Action |  | Check host and drive side activity. |  |
| Event ID |  | 020A4402 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive inconsistent parity block recovered |  |
| 1st Line Message |  | Inconsistent parity block regenerated - # |  |
| Old Message |  | LG:%.1d ALERT:Inconsistent Parity Block %bx%08lx Regenerated |  |
| Cause |  | RAID calculates data to generate parity and compare with parity on hard drive. When it finds miscompare parity, firmware will overwrite incorrect parity and issue the event to inform user. |  |
| Action |  | No action required. It just informs users inconsistent parities were found and recovered. |  |
| Event ID |  | 0104A501 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller battery backup unit (BBU) back to normal |  |
| 1st Line Message |  | Absent status recovered to normal status |  |
| Old Message |  | NOTICE: Controller BBU Present ! |  |
| Cause |  | Battery backup module restored. |  |
| Action |  |  |  |
| Event ID |  | 0104A502 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller battery backup unit (BBU) back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status |  |
| Old Message |  | NOTICE: Controller BBU Back On-Line ! |  |
| Cause |  | Battery backup module restored. |  |
| Action |  |  |  |
| Event ID |  | 0104A503 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller battery backup unit (BBU) fully charged |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE: Controller BBU Fully Charged ! |  |
| Cause |  | Battery backup module fully charged. |  |
| Action |  |  |  |
| Event ID |  | 01042506 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller battery backup unit (BBU) charging |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT:Controller BBU is Charging ! |  |
| Cause |  | Battery backup module being charged |  |
| Action |  |  |  |
| Event ID |  | 02011201 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller write policy forced to write-through mode by trigger cause |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE:Force Controller Write-Through on Trigger Cause |  |
| Cause |  | Write-back caching disabled. System errors detected. |  |
| Action |  | Check other system notifications and address errors. Write-through is more conservative than write-back. When the risk of losing data rises due to system errors such as component failures and abnormal temperatures, write-through is automatically enabled. |  |
| Event ID |  | 02019201 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller write policy default settings restored |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE:Controller Default Write Policy Restore |  |
| Cause |  | Write-back caching disabled 1. System errors detected 2. Firmware versions on the two controllers are different. |  |
| Action |  | 1. Check other system notifications and address errors 2. Make sure firmware synchronization has completed Write-through is more conservative than write-back. When the risk of losing data rises due to system errors such as component failures and abnormal |  |
| Event ID |  | 02019202 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller shutdown started |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE:Controller Shutdown Start |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02019203 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller shutdown completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE:Controller Shutdown Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02018A01 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  |  |  |
| 1st Line Message |  | Cache data present during system power on |  |
| Old Message |  | NOTICE:Data exists in cache at power on |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020B4602 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume status changed from offline to online |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: State change from off-line to on-line |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020B4603 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | All member drives of a logical volume removed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Remove all member drives of LD completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020B4604 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | All member drives of a logical volume restored |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Restore all member drives of LD completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020B4605 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume undeleted |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Undeleted successfully |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A4607 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive status changed from offline to online |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: State change from off-line to on-line |  |
| Cause |  | Logical drive now online. |  |
| Action |  |  |  |
| Event ID |  | 020A4608 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | All member drives of logical drive removed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Remove all member drives of LD completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A4609 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | All member drives of logical drive restored |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Restore all member drives of LD completed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020A460A |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive undeleted |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Undeleted successfully |  |
| Cause |  | Deleted logical drive restored |  |
| Action |  |  |  |
| Event ID |  | 02018201 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Firmware synchronization started |  |
| 1st Line Message |  |  |  |
| Old Message |  | WARNING: Firmware synchronization started |  |
| Cause |  | Firmware being updated. This process may take a while, please be patient. |  |
| Action |  | Reset the controller. |  |
| Event ID |  | 02018202 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Firmware synchronization completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | WARNING: Firmware synchronization completed |  |
| Cause |  | Automatic firmware synchronization completed. |  |
| Action |  |  |  |
| Event ID |  | 01098102 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel connected |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%d Host Channel link is up |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 01028101 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer # closed |  |
| 1st Line Message |  |  |  |
| Old Message |  | INFORM: Enclosure drawer %d status is back to close |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 01028102 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # drawer # closed |  |
| 1st Line Message |  |  |  |
| Old Message |  | INFORM: SES(C%d I%d)Enclosure drawer %d status is back to close |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0103A102 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage back to normal |  |
| 1st Line Message |  | +3.3V high voltage nominal |  |
| Old Message |  | NOTICE:+3.3V Upper Voltage Back To Normal(%s) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0103A103 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage back to normal |  |
| 1st Line Message |  | +5V high voltage nominal |  |
| Old Message |  | NOTICE:+5V Upper Voltage Back To Normal(%s) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0103A104 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage back to normal |  |
| 1st Line Message |  | +12V high voltage nominal |  |
| Old Message |  | NOTICE:+12V Upper Voltage Back To Normal(%s) |  |
| Cause |  | 12V line within safe operating range and below upper threshold. |  |
| Action |  |  |  |
| Event ID |  | 0103A105 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage back to normal |  |
| 1st Line Message |  | +3.3V low voltage nominal |  |
| Old Message |  | NOTICE:+3.3V Lower Voltage Back To Normal(%s) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0103A106 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage back to normal |  |
| 1st Line Message |  | +5V low voltage nominal |  |
| Old Message |  | NOTICE:+5V Lower Voltage Back To Normal(%s) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0103A107 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage back to normal |  |
| 1st Line Message |  | +12V low voltage nominal |  |
| Old Message |  | NOTICE:+12V Lower Voltage Back To Normal(%s) |  |
| Cause |  | 12V within safe operating range and above lower threshold. |  |
| Action |  |  |  |
| Event ID |  | 01028103 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer # back to normal |  |
| 1st Line Message |  | Drawer ready |  |
| Old Message |  | INFORM: Enclosure drawer %d status back to normal |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 01028104 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # drawer # back to normal |  |
| 1st Line Message |  | Drawer ready |  |
| Old Message |  | INFORM: SES(C%d I%d)Enclosure drawer %d status back to normal |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0103A10D |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # power supply # failed status has been recovered to normal. |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: PSU0 Back On-Line |  |
| Cause |  | Power supply successfully replaced. |  |
| Action |  |  |  |
| Event ID |  | 0103A10E |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure power supply # failed status has been recovered to normal. |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d Device NOTICE: PSU0 Back On-Line |  |
| Cause |  | Power supply successfully replaced. |  |
| Action |  |  |  |
| Event ID |  | 0103A10F |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # power supply # absent status has recovered to normal. |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: PSU0 is Present |  |
| Cause |  | Missing power supply successfully detected. |  |
| Action |  |  |  |
| Event ID |  | 0103A110 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure power supply # absent status has been recovered to normal. |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d Device NOTICE: PSU0 is Present |  |
| Cause |  | Missing power supply successfully detected. |  |
| Action |  |  |  |
| Event ID |  | 0105A20B |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # fan # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: Cooling Fan0 Back On-Line |  |
| Cause |  | Missing or failed fan back online. |  |
| Action |  |  |  |
| Event ID |  | 0105A20C |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure fan # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status |  |
| Old Message |  | Peripheral Set %d Device NOTICE: Cooling Fan0 Back On-Line |  |
| Cause |  | Missing or failed fan back online. |  |
| Action |  |  |  |
| Event ID |  | 0105A20D |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # fan # back to normal |  |
| 1st Line Message |  | Absent status recovered to normal status |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: Cooling Fan0 is Present |  |
| Cause |  | Missing or failed fan back online. |  |
| Action |  |  |  |
| Event ID |  | 0105A20E |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure fan # back to normal |  |
| 1st Line Message |  | Absent status recovered to normal status |  |
| Old Message |  | Peripheral Set %d Device NOTICE: Cooling Fan0 is Present |  |
| Cause |  | Missing or failed fan back online. |  |
| Action |  |  |  |
| Event ID |  | 0105A20F |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # fan # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status, RPM at # |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: Cooling Fan0 Back On-Line(%.1d RPM) |  |
| Cause |  | Missing or failed fan back online. |  |
| Action |  |  |  |
| Event ID |  | 0105A210 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure fan # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status, RPM at # |  |
| Old Message |  | Peripheral Set %d Device NOTICE: Cooling Fan0 Back On-Line(%.1d RPM) |  |
| Cause |  | Missing or failed fan back online. |  |
| Action |  |  |  |
| Event ID |  | 0105A24B |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # drawer fan # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: Drawer fan0 Back On-Line |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0105A24C |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer fan # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status |  |
| Old Message |  | Peripheral Set %d Device NOTICE: Drawer fan0 Back On-Line |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0105A24D |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # drawer fan # back to normal |  |
| 1st Line Message |  | Absent status recovered to normal status |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: Drawer fan0 is Present |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0105A24E |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer fan # back to normal |  |
| 1st Line Message |  | Absent status recovered to normal status |  |
| Old Message |  | Peripheral Set %d Device NOTICE: Drawer fan0 is Present |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0105A24F |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # drawer fan # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status, RPM at # |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: Drawer fan0 Back On-Line(%.1d RPM) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0105A250 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer fan # back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status, RPM at # |  |
| Old Message |  | Peripheral Set %d Device NOTICE: Drawer fan0 Back On-Line(%.1d RPM) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0107A30A |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # backplane temperature back to normal |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: Backplane Temperature Back To Normal |  |
| Cause |  | Backplane temperature within normal levels. |  |
| Action |  |  |  |
| Event ID |  | 0107A30B |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure backplane temperature back to normal |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d Device NOTICE: Backplane Temperature Back To Normal |  |
| Cause |  | Backplane temperature within normal levels. |  |
| Action |  |  |  |
| Event ID |  | 0107A30E |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # backplane temperature back to normal |  |
| 1st Line Message |  | Temperature: #(C) |  |
| Old Message |  | Peripheral Set %d(%d) Device NOTICE: Backplane Temperature Back To Normal(%.1d(C)) |  |
| Cause |  | Backplane temperature within normal levels. |  |
| Action |  |  |  |
| Event ID |  | 0107A30F |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure backplane temperature back to normal |  |
| 1st Line Message |  | Temperature: #(C) |  |
| Old Message |  | Peripheral Set %d Device NOTICE: Backplane Temperature Back To Normal(%.1d(C)) |  |
| Cause |  | Backplane temperature within normal levels. |  |
| Action |  |  |  |
| Event ID |  | 0104A504 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller Super-Capacitor back to normal |  |
| 1st Line Message |  | Absent status recovered to normal status |  |
| Old Message |  | NOTICE: Controller Super-Capacitor Present ! |  |
| Cause |  | Super-Capacitor module restored. |  |
| Action |  |  |  |
| Event ID |  | 0104A505 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller Super-Capacitor back to normal |  |
| 1st Line Message |  | Failed status recovered to normal status |  |
| Old Message |  | NOTICE: ControllerSuper-Capacitor Back On-Line ! |  |
| Cause |  | Super-Capacitor module restored. |  |
| Action |  |  |  |
| Event ID |  | 0104A506 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller Super-Capacitor fully charged |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE: ControllerSuper-Capacitor Fully Charged ! |  |
| Cause |  | Super-Capacitor module fully charged. |  |
| Action |  |  |  |
| Event ID |  | 0104250A |  |
| Severity |  | Information |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller Super-Capacitor charging |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT:Controller Super-Capacitor is Charging ! |  |
| Cause |  | Super-Capacitor module being charged |  |
| Action |  |  |  |
| Event ID |  | 01098103 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel speed changed to #Gb |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%d Host Channel speed is changed back to %dG |  |
| Cause |  | Host channel speed changed from its previous status. |  |
| Action |  | 1. If speed is not at full spec, check switch settings 2. Check cable and connection status. |  |
| Event ID |  | 01098104 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel speed changed to #Mb |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%d Host Channel speed is changed back to %d00Mb |  |
| Cause |  | Host channel speed changed from its previous status. |  |
| Action |  | 1. If speed is not at full spec, check switch settings 2. Check cable and connection status. |  |
| Event ID |  | 0106A40C |  |
| Severity |  | Information |  |
| Category-Module |  | HW - UPS |  |
| Prefix Parameter |  |  |  |
| Message |  | UPS Connection Detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | UPS Connection Detected |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0106A40D |  |
| Severity |  | Information |  |
| Category-Module |  | HW - UPS |  |
| Prefix Parameter |  |  |  |
| Message |  | UPS AC Power Restored |  |
| 1st Line Message |  |  |  |
| Old Message |  | UPS AC Power Restored |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0106A40E |  |
| Severity |  | Information |  |
| Category-Module |  | HW - UPS |  |
| Prefix Parameter |  |  |  |
| Message |  | UPS Battery Level Restored to Safety - #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | UPS Battery Restored To Safe Level %d% |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0107A313 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller temperature back to normal |  |
| 1st Line Message |  | Host board I/O chip high temperature down to #(C) |  |
| Old Message |  | NOTICE:Controller Hot Temperature Back To Normal(4)(%d.0(C)) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0105A212 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | CPU FAN # Back On-Line |  |
| 1st Line Message |  |  |  |
| Old Message |  | CPU FAN %d Back On-Line |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 01010407 |  |
| Severity |  | Information |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  |  |  |
| 1st Line Message |  | Memory ECC single bit error corrected in DIMM # |  |
| Old Message |  | Controller WARNING: Memory ECC Single-bit Error Detected in DIMM %d |  |
| Cause |  | Data bus contains signal errors. |  |
| Action |  | Contact technical support for issue escalation. |  |
| Event ID |  | 020E0001 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pool created |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Creation Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020E0003 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pool expansion completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Expand Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020E0005 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pool deleted |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Deleted |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02130008 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Status changed to online |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) online |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020E000A |  |
| Severity |  | Information |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pool migration started |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Migration Start |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020E000B |  |
| Severity |  | Information |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pool migration completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Migration Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020F000D |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Volume created |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Creation Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020F000F |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Volume deleted |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Deleted Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02150011 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/SI |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Volume/snapshot image activated |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Activated |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020F0012 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Volume expansion completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Expand Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120015 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair created |  |
| 1st Line Message |  | Volume set:# |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Creation Complete in %s( VS SrcID:%08x%08x ) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120017 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair deleted |  |
| 1st Line Message |  | Volume set:# |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Terminated in ( VS ID:%08lx%08x ) |  |
| Cause |  | The pair was deleted due to user preference. |  |
| Action |  |  |  |
| Event ID |  | 02120018 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Synchronous replication started |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Sync Start |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120019 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Synchronous replication completed, data transfered: # # |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Sync Complete (%d %s Transfered) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0212001A |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Synchronous replication paused |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Sync Pause |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0212001B |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Synchronous replication resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Sync Resume |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0212001C |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Synchronous replication pair split |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Sync Split |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0212001E |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Asynchronous replication started |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Async Start |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0212001F |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Asynchronous replication paused |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Async Pause |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120020 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Asynchronous replication resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Async Resume |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120021 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Asynchronous replication completed, data transfered: # # |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Async Complete (%d %s Transfered) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120022 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Asynchronous replication pair split |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Async Split |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120023 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Replication pair role switched |  |
| 1st Line Message |  | Volume set: # |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Role Switch to SrcVSID:%08lx%08lx |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02110024 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - RD |  |
| Prefix Parameter |  | ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Remote disk connected |  |
| Old Message |  | REMOTE DRIVE ID:%08lx%08lx Connected |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020B0028 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Logical volume created |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( LV ID:%08lx%08lx ) Creation Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020B002A |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Logical volume expansion completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( LV ID:%08lx%08lx ) Expand Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020B002C |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Logical volume deleted |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( LV ID:%08lx%08lx ) Deleted |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020C002D |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Partition created |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( PT ID:%08lx%08lx ) Creation Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020C002F |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Partition deleted |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( PT ID:%08lx%08lx ) Deleted Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020C0031 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Partition activated |  |
| Old Message |  | %s( PT ID:%08lx%08lx ) Activated |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02140035 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Free space recovered for future volume usage |  |
| 1st Line Message |  | Free space recovered >= 35% of current volume used |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) VV has enough free space |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02100036 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - SI |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Free space recovered for future snapshot usage |  |
| 1st Line Message |  | Free space recovered >= 35% of current snapshot used |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) SI has enough free space |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120037 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Free space recovered for future replication pair usage |  |
| 1st Line Message |  | Free space recovered >= 35% of current asychronous pair used |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Async has enough free space |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02160039 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/DATASERVICE |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Free space recovered for data allocation |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) recovered free space |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 03100001 |  |
| Severity |  | Information |  |
| Category-Module |  | SW - SI |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Snapshot image created |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 03100003 |  |
| Severity |  | Information |  |
| Category-Module |  | SW - SI |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Snapshot image deleted |  |
| 1st Line Message |  | ReasonCode: # |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02130006 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space utilization exceeded threshold: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Utilization Exceeded - (%lu%%) |  |
| Cause |  | Capacity utilization exceeded a default or customized threshold percentage and posted an event notice with severity information. |  |
| Action |  | Check capacity usage for insufficient space. Fix the issue by expansion or remove unused volumes or snapshot images to free more capacity for data allocation |  |
| Event ID |  | 02130007 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space utilization dropped below threshold: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Utilization Below - (%lu%%) |  |
| Cause |  | Capacity utilization is freed and it is below the threshold setting. |  |
| Action |  | No required action if user engages in expansion or deletion operations to lower space utilization manually |  |
| Event ID |  | 02130026 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space allocation exceeded #%, used: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Allocation Exceeded - (%lu %%)(Used=%lu %%) |  |
| Cause |  | Used data exceeded specific percentage and posted an event notice with severity information. |  |
| Action |  | Check capacity usage for insufficient space. Fix the issue by expansion or remove unused volumes or snapshot images to free more capacity for data allocation or to shrink the reservation size. |  |
| Event ID |  | 02130027 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space allocation below #%, used: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Allocation Below - (%lu %%)(Used=%lu %%) |  |
| Cause |  | Used data is freed and it is below specified percentage. |  |
| Action |  | No required action if user initiated expansion or deletion to lower space utilization manually. |  |
| Event ID |  | 0212003D |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy started |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) initial copy has started success |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0212003F |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy completed, transfer size: #, rate: # |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%09lx ) initial copy complete size %d%s transfer rate %d%s |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120040 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy continued |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%10lx ) initial copy has continued |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120042 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) initial copy has resumed success |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120044 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair recovered |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%13lx ) has recovered |  |
| Cause |  | 1. Pair recovered because the network connection was recovered. 2. Reboot the controller of the source volume and the pair will be recovered automatically. |  |
| Action |  |  |  |
| Event ID |  | 02120046 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization started |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) pair synchronies has started success |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120048 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization completed, transfer size: #, rate: # |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%09lx ) pair synchronies complete size %d%s transfer rate %d%s |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120049 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization continued |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%10lx ) pair synchronies has continued |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0212004B |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization resumed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) pair synchronies has resumed success |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0212004E |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  |  |  |
| Message |  | #-# Hour 1 transfer/latency/loss: #/#/# Hour 2: #/#/# |  |
| 1st Line Message |  |  |  |
| Old Message |  | (%d.%d%s, %dms, %d.%d%%)(%d.%d%s, %dms, %d.%d%%)(%d.%d%s, %dms, %d.%d%%)(%d.%d%s, %dms, %d.%d%%) |  |
| Cause |  | Event used to describe data transfer rate, latency, and packet loss information for data replication during pair synchronization of specific period. |  |
| Action |  |  |  |
| Event ID |  | 0214004F |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Tier migration started |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Tier Migration Start |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02140050 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Tier migration completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Tier Migration Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02140051 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Tier migration aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Tier Migration Abort |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02140052 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Volume expansion completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Expand Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02100080 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - SI |  |
| Prefix Parameter |  |  |  |
| Message |  | The snapshot images (ID: #) of the volume (ID: #) has been backed up to cloud completely. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 020E0088 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool (ID: #) has been deleted, but the related cloud storage (Name: #) can not be deleted. It may cause by the device cannot connect to the cloud provider. Please delete the cloud storage via the user interface provided by the cloud provider. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Can not connect to provider |  |
| Action |  | User can delete the bucket from provider later |  |
| Event ID |  | 120A8C04 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Media scan stopped. Media error: #, recovered error: # |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%lx Logical Drive NOTICE: Media Scan Stopped (%d,%d) |  |
| Cause |  | Media scan operation stopped due to logical drive status changed to failed or other status. |  |
| Action |  | Recover the LD issue by rebuilding or restarting the LD first. Then restart the media scan again. |  |
| Event ID |  | 120A8C05 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Unable to start media scan |  |
| 1st Line Message |  | Previous media scan still ongoing |  |
| Old Message |  | ID:%lx Logical Drive WARN: Media Scan is Postponed Because Previous Task is still In-progress |  |
| Cause |  | Media scan operation postponed because a previous similar task is still in progress. |  |
| Action |  | 1. Wait for previous task completion, then start new media scan. 2. Stop ongoing task and restart media scan. |  |
| Event ID |  | 120B0801 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume cache data purged |  |
| 1st Line Message |  |  |  |
| Old Message |  | LV:%d Logical Volume WARNING: Cache Data Purged |  |
| Cause |  | Cache data missing and detected when RAID boots up. This causes from user improperly power off the RAID, and RAID does not have BBU installed or BBU malfunction, so the data from memory was lost during power outage. |  |
| Action |  | Cache data already lost. Restore data from a backup source if necessary and remind user should always shut down controller first before power off RAID. |  |
| Event ID |  | 120A0802 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive cache data purged |  |
| 1st Line Message |  |  |  |
| Old Message |  | LG:%d Logical Drive WARNING: Cache Data Purged |  |
| Cause |  | Cache data missing and detected when RAID boots up. This causes from user improperly power off the RAID, and RAID does not have BBU installed or BBU malfunction, so the data from memory was lost during power outage. |  |
| Action |  | Cache data already lost. Restore data from a backup source if necessary and remind user should always shut down controller first before power off RAID. |  |
| Event ID |  | 120B8A01 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume migration paused |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Migrate Operation Paused |  |
| Cause |  | Data migration operation in logical volume pasued. 1. User cancelled the operation manually. 2. Member Logical Drive in Logical Volume failed. |  |
| Action |  | 1. If the operation was paused by users, simply restart migration. 2. Check previous events if any Logical Drive issue caused migration paused. Follow the service guide to fix the Logical Drive issue then restart migration. |  |
| Event ID |  | 120A8A02 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive RAID migration paused |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Migrate Operation Paused |  |
| Cause |  | Data migration operation in logical drive pasued due to: 1. User cancelled the operation manually. 2. Drive failure occurred. 3. There is not enough space for migration when migrating RAID5 to RAID6. |  |
| Action |  | 1. If the operation was paused by users, they can simply restart migration. 2. If the operation was interrupted by drive failure, check previous event logs for cause of interruption, and replace the failed drive. 3. Add a drive and expand the logical driv |  |
| Event ID |  | 120B8A03 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume add drive action paused |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: Add Drive Operation Paused |  |
| Cause |  | Add drive operation in a logical volume paused due to member logical drive issues. |  |
| Action |  | 1. Check previous event logs of drives or logical drives for cause of interruption. 2. Replace affected drives or address logical drive errors with corresponding event actions. |  |
| Event ID |  | 120A8A04 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive add drive action paused |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: Add Drive Operation Paused |  |
| Cause |  | Add drive operation for logical drive was paused. 1. User cancelled the operation manually. 2. The new added drive contains errors. 3. The size of the new drive is smaller than other member drives. 4. The logical drive contains a failed member drive. |  |
| Action |  | 1. If the operation was paused by users, they can simply restart it 2. If the operation was interrupted by drive failure, check previous event logs for cause of interruption, and use another drive. 3. Make sure the size of new added drive is equal to or l |  |
| Event ID |  | 120A4203 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive bad data block recovered |  |
| 1st Line Message |  | Bad data block recovered - # |  |
| Old Message |  | LG:%.1d Logical Drive NOTICE: Bad Data Block Recovered %bx%08lx |  |
| Cause |  | Data blocks marked as bad recovered. |  |
| Action |  | No action required. This notice simply lets you know bad blocks were found and recovered. If the event occurrs often, users should replace the member drives in the logical drive. |  |
| Event ID |  | 1208C102 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Exiled drive detected |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive NOTICE: Exiled Drive Detected |  |
| Cause |  | Unstable LD member drive used to be failed once would become Exiled drive after rebooting RAID subsystem or re-inserting the failed drive. |  |
| Action |  | Replace the Exiled drive with a new drive for LD rebuilding. |  |
| Event ID |  | 12084203 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Drive SMART error state detected |  |
| Old Message |  | WARNING:SMART-CH %.1d ID:%.1d Predictable Failure Detected |  |
| Cause |  | Drive SMART error threshold exceeded. |  |
| Action |  | 1. Clone and replace problematic drive. 2. Replace the original cloned drive with new one. |  |
| Event ID |  | 12084204 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error recovery procedure started |  |
| 1st Line Message |  | Drive contains SMART errors |  |
| Old Message |  | WARNING:SMART-CH %.1d ID:%.1d Predictable Failure Detected-Starting Clone |  |
| Cause |  | Drive SMART errors was found and trigger the clone opearion for replacement. |  |
| Action |  | No action required except for replacing the cloned disk that has errors when recovery procedure is completed. |  |
| Event ID |  | 120A4404 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive media error recovered |  |
| 1st Line Message |  | Media error recovered during parity check - # |  |
| Old Message |  | LG:%.1d Logical Drive ALERT:Media Error During Check Parity Recovered %bx%08lx |  |
| Cause |  | Data affected by media errors during parity check recovered. |  |
| Action |  | Replace the disk drive. If the error/recovery message appears frequently, the drive is unreliable. If the event occurrs often, users should also check drive and logical drive related events for escalation. |  |
| Event ID |  | 11098101 |  |
| Severity |  | Warning |  |
| Category-Module |  | HW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel disconnected |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%d Host Channel link is down |  |
| Cause |  | Connection to specific host channel was broken due to: 1. No cable is connected to the host channel after controller initialization complete. 2. Cable connected to the host channel is removed by user. 3. Cable connected from the host channel to switch or |  |
| Action |  | 1. It's normal and no action is required. 2. It's uer's operation and no action is required. 3. Replace the cable connected to the host channel. 4. Switch the cable to other ports on the controller, switch or host. 5. If issue was not fixed, select Syste |  |
| Event ID |  | 11098103 |  |
| Severity |  | Warning |  |
| Category-Module |  | HW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel speed changed to #Gb |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%d Host Channel speed is changed back to %dG |  |
| Cause |  | 1. User removes the cable, and inserts back. 2. Port on the contorller, cable, switch, port on the host connection may be unstable or faulty but recovered. |  |
| Action |  | 1. It is normal if user reconnects cable. 2. If this message repeats, replace the cable connected to the controller 3. Check if othe devices connect to the same switch also have problem to isolate the general issue. 4. Check if switch setting is not full |  |
| Event ID |  | 11098104 |  |
| Severity |  | Warning |  |
| Category-Module |  | HW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel speed changed to #Mb |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%d Host Channel speed is changed back to %d00Mb |  |
| Cause |  | 1. User removes the cable, and inserts back. 2. Port on the contorller, cable, switch, port on the host connection may be unstable or faulty but recovered. |  |
| Action |  | 1. User removes the cable, and inserts back. 2. If this message repeats, replace the cable connected to the controller 3. Check if othe devices connect to the same switch also have problem to isolate the general issue 4. Check if switch setting is not ful |  |
| Event ID |  | 11098105 |  |
| Severity |  | Warning |  |
| Category-Module |  | HW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel speed changed to #Gb |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%d Host Channel speed is changed to %dG |  |
| Cause |  | Port on the contorller, cable, switch, port on the host connection may be unstable or faulty, causing the speed change. |  |
| Action |  | 1. It's normal and no action is required. 2. If it's uer's operation, no action is required. 3. Replace the cable connected to the host channel. 4. Switch the cable to other ports on the controller, switch or host. 5. If issue was not fixed, select Syste |  |
| Event ID |  | 11098106 |  |
| Severity |  | Warning |  |
| Category-Module |  | HW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel speed changed to #Mb |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%d Host Channel speed is changed to %d00Mb |  |
| Cause |  | Host channel speed changed from its previous status. |  |
| Action |  | 1. If speed is not at full spec, check switch settings 2. Check cable and connection status. |  |
| Event ID |  | 11020101 |  |
| Severity |  | Warning |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer # opened |  |
| 1st Line Message |  |  |  |
| Old Message |  | WARN: Enclosure drawer %d open detected |  |
| Cause |  | Enclosure drawer not fully closed or is open. |  |
| Action |  | Check drawer and make sure it is closed. |  |
| Event ID |  | 11020102 |  |
| Severity |  | Warning |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # drawer # opened |  |
| 1st Line Message |  |  |  |
| Old Message |  | WARN: SES(C%d I%d)Enclosure drawer %d open detected |  |
| Cause |  | Expansion enclosure drawer not fully closed or is open. |  |
| Action |  | Check drawer and make sure it is closed. |  |
| Event ID |  | 12091601 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | IP address conflict detected, conflicted IP(v4): # |  |
| 1st Line Message |  | Conflicted MAC address: # |  |
| Old Message |  | CHL %d : IP conflict is detected: (IP: %s, Mac address: %s) |  |
| Cause |  | IP address conflict occurs, and the channel had been assigned the same address with another network equipment. |  |
| Action |  | Check communication related setings of the channel, and change its IP address if possible. |  |
| Event ID |  | 12091602 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | IP address conflict detected, conflicted IP(v6): # |  |
| 1st Line Message |  | Conflicted MAC address: # |  |
| Old Message |  | CHL %d : IP conflict is detected: (IP: %s, Mac address: %s) |  |
| Cause |  | IP address conflict occurs, and the channel had been assigned the same address with another network equipment. |  |
| Action |  | Check communication related setings of the channel, and change its IP address if possible. |  |
| Event ID |  | 11084311 |  |
| Severity |  | Warning |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive lifetime estimate warning threshold exceeded |  |
| 1st Line Message |  | Drive lifetime estimate warning threshold exceeded (life remaining: #) |  |
| Old Message |  | CHL:%d ID:%d Drive Lifetime Estimate Warning Threshold Exceeded (Life Remaining: %d%% ) |  |
| Cause |  |  |  |
| Action |  | Replace the drive with a new one. |  |
| Event ID |  | 11091E01 |  |
| Severity |  | Warning |  |
| Category-Module |  | HW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Invalid SFP installation detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | CH:%d Invalid %dG SFP installation detected while host board mode is %dG |  |
| Cause |  |  |  |
| Action |  | Change the SFP with compatible speed. |  |
| Event ID |  | 12130009 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Status changed to offline |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) offline |  |
| Cause |  | LV/Pool status changed from good to offline due to: 1. The RAID system is shutdown. 2. Member LD/Pool Element in a LV/Pool is shutdown or fatal fail. 3. If it's scale-out Pool, the member Pool Element is missing due to network connection issue. |  |
| Action |  | 1. No action is required if it's triggered by user's operation. LV/Pool would become online after restarting RAID system. 2. Restart RAID subsystem or check previous events if any drive issue caused LD/Pool Element fatal fail. Follow the service guide to fix the LD/Pool element issue. 3. Check and diagnose network connection between master and slave units. Pool would become online after fixing the network issue. 4. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 12170013 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - SETTING |  |
| Prefix Parameter |  |  |  |
| Message |  | Non-optimal configuration may impact system performance |  |
| 1st Line Message |  |  |  |
| Old Message |  | Non-optimal configuration may impact performance |  |
| Cause |  | Non-optimal configuration means controller battery backup unit (BBU) issues result from: 1. Faulty BBU. 2. Event triggers for the BBU not enabled. |  |
| Action |  | 1. Replace faulty BBU. 2. Enable BBU event triggers in system settings. |  |
| Event ID |  | 12100014 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - SI |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Snapshot image purge triggered - (#%) |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Purge Triggered - -(%lu%%) |  |
| Cause |  | Snapshot image purge operation is triggered by threshold policy settings. When the LV/Pool available size reached the threshold setting, user can set the event notification and snapshot images purge operation. Purge operation will delete one or several sn |  |
| Action |  | No action is required. You can check capacity usage after purge, or check if purge policy is adequate. |  |
| Event ID |  | 12110025 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - RD |  |
| Prefix Parameter |  | ID:# |  |
| Message |  |  |  |
| 1st Line Message |  | Remote disk disconnected |  |
| Old Message |  | REMOTE DRIVE ID:%08lx%08lx Disconnected |  |
| Cause |  | Remote disk is the Partition/VV assigned from a remote device to a local source device. It can be a remote Partition/VV connected to the source unit when creating a replication pair. Remote disk disconnected may be caused by: 1. User unassigned the target |  |
| Action |  | 1. Assignment can be recovered automatically by resyncing the replication pair. 2. Check and diagnose network status and bandwidth utilization for remote replication. 3. Check if the target Partition/VV is mapped. |  |
| Event ID |  | 12140032 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Free space might be insufficient for future volume usage |  |
| 1st Line Message |  | Free space < 35% of current volume used |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) VV no enough free space |  |
| Cause |  | Free space in logical volume or pool may not be enough for future volume allocation. |  |
| Action |  | Expand storage or remove unused volumes or snapshot images to free capacity for future use. |  |
| Event ID |  | 12100033 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - SI |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Free space might be insufficient for future snapshot usage |  |
| 1st Line Message |  | Free space < 35% of current snapshot usage |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) SI no enough free space |  |
| Cause |  | Free space in logical volume or pool may not be enough for future snapshot image creation. |  |
| Action |  | Expand storage or remove unused volumes or snapshot images to free capacity for future use. |  |
| Event ID |  | 12120034 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Free space might be insufficient for future replication pair usage |  |
| 1st Line Message |  | Free space < 35% of current asychronous pair usage. |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Async no enough free space |  |
| Cause |  | Free space in logical volume or pool may not be enough for future pair reservation for I/O operation. |  |
| Action |  | Expand storage or remove unused volumes or snapshot images to free capacity for future use. |  |
| Event ID |  | 12130006 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space utilization exceeded threshold: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Utilization Exceeded - (%lu%%) |  |
| Cause |  | Capacity utilization exceeded a default or customized threshold percentage and posted an event notice with warning severity. |  |
| Action |  | Check whether capacity is available. If not, expand storage or remove unused volumes or snapshot images to free capacity. |  |
| Event ID |  | 12130007 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space utilization below threshold: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Utilization Below - (%lu%%) |  |
| Cause |  | Capacity utilization below default or customized threshold percentage. |  |
| Action |  | 1. No action required if user initiated expansion or deletion operations to lower space utilization manually. 2. If not engaged by a user, check previous event logs for cause of utilization drop. |  |
| Event ID |  | 12130026 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space allocation exceeded #%, used:#% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Allocation Exceeded - (%lu %%)(Used=%lu %%) |  |
| Cause |  | Used data exceeded a specific percentage and posted an event notice with warning severity. It reminds user that available capacity may be insufficient in the future. |  |
| Action |  | Check available storage space. Expand storage or remove unused volumes or snapshot images to free capacity. |  |
| Event ID |  | 12130027 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space allocation below #%, used: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Allocation Below - (%lu %%)(Used=%lu %%) |  |
| Cause |  | Used data below specific percentage. |  |
| Action |  | No required action if user initiated expansion or deletion to lower space utilization manually. |  |
| Event ID |  | 12120041 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy stopped |  |
| 1st Line Message |  | ErrorCode: %d |  |
| Old Message |  | %s( VP ID:%08lx%11lx ) initial copy has stopped, event ID: %d |  |
| Cause |  | Initial copy from source to target volume for replication pair synchronization stopped due to: 1. Target Partition/VV or its LV/Pool ran out of space. 2. Connection between source and target units is unstable. |  |
| Action |  | 1. Expand storage or remove unused Partition/VV or snapshot images to free capacity. 2. Check and diagnose network status and bandwidth utilization for remote replication. 3. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 12120045 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair broken |  |
| 1st Line Message |  | ErrorCode: %d |  |
| Old Message |  | %s( VP ID:%08lx%14lx ) has broken, event ID: %d |  |
| Cause |  | Remote replication was broken due to network issues. |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. Remote replication would be recovered automatically after network issue is fixed. 2. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 1212004A |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization stopped |  |
| 1st Line Message |  | ErrorCode: %d |  |
| Old Message |  | %s( VP ID:%08lx%11lx ) pair synchronies has stopped, event ID: %d |  |
| Cause |  | Replication pair synchronization stopped as a result of: 1. Target Partition/VV or its LV/Pool ran out of space. 2. Connection between source and target units is unstable. |  |
| Action |  | 1. Expand storage or remove unused volumes or snapshot images to free capacity for future use. 2. Check and diagnose network status and bandwidth utilization for remote replication. 3. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 1212004D |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Target volume full |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) target volume full |  |
| Cause |  | The target volume for replication pair is full and failed to allocate more space. |  |
| Action |  | Expand storage or remove unused volumes or snapshot images to free capacity for future use. |  |
| Event ID |  | 120E0080 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool (ID: #) cannot connect to cloud storage, because the network error may occur. Please check your network connection, configuration or cloud service status. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Network problem |  |
| Action |  | check network setting |  |
| Event ID |  | 120E0081 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool (ID: #) cannot connect to cloud storage, because authentication failed. Please check the settings with your cloud service providers. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Authentication Key Incorrect |  |
| Action |  | check authentication setting |  |
| Event ID |  | 120E0082 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool (ID: #) cannot connect to cloud storage, because the bucket does not exist. Please check the settings of configuration UI with your cloud service providers. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Bucket is removed by unknown reason |  |
| Action |  | if the bucket is already removed, can not restore anymore |  |
| Event ID |  | 120E0083 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool (ID: #) cannot connect to cloud storage, because failed to create the bucket. Please check the settings with your cloud service providers. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Provider not allow to create bucket |  |
| Action |  | check provider |  |
| Event ID |  | 120E0084 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool (ID: #) cannot connect to cloud storage, because the bucket has been used by another pool. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | The Bucket is using by another Pool |  |
| Action |  | one bucket only can be used by one pool at the same time |  |
| Event ID |  | 120E0085 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool (ID: #) cannot connect to cloud storage, because type the wrong encryption key. Please check the correct encryption key. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Encrpytion setting incorrect |  |
| Action |  | check encryption setting |  |
| Event ID |  | 120E0086 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool (ID: #) cannot connect to cloud storage, because failed to send local data files to cloud. Please check the available free space of the cloud storage. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Provider not allow to put object |  |
| Action |  | check provider |  |
| Event ID |  | 120E0087 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool (ID: #) cannot connect to cloud storage, because all channels have been assigned to file service. Please assign at least one channel for iSCSI block device. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | All channels have been assigned to file service. |  |
| Action |  | Assign at least one channel to RAID |  |
| Event ID |  | 22080711 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Media scan aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE: CHL:%.1d ID:%.1d Media Scan Aborted |  |
| Cause |  | Media scan aborted due to: 1. Operation cancelled manually by user 2. Drive failure (global and enclosure spare only) |  |
| Action |  | 1. If operation cancelled by users, simply restart it. 2. If interrupted by drive error, check previous drive event logs for cause of interruption, and replace faulty drive. |  |
| Event ID |  | 220A0702 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Media scan aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x NOTICE: Media Scan Aborted |  |
| Cause |  | Media scan operation aborted due to: 1. User cancelled the operation manually. 2. LD has been deleted. |  |
| Action |  | 1. If the operation was cancelled by users, simply restart it. 2. If the operation was interrupted by a drive error, check event logs for cause of interruption, and replace faulty drive. |  |
| Event ID |  | 220A0704 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Unable to start media scan |  |
| 1st Line Message |  | Logical drive status is invalid for media scan |  |
| Old Message |  | ID:%lx Logical Drive WARN: Invalid Status for Media Scan |  |
| Cause |  | Media scan can only be executed when logical drive status is in "good" status. |  |
| Action |  | 1. Check current logical drive status and replace the failed member drive in LD. 2. Rebuild logical drive if necessary. 3. Restart media scan. |  |
| Event ID |  | 220A0705 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Media scan stopped. Media error: #, recovered error: # |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%lx Logical Drive WARN: Media Scan Failed (%d,%d) |  |
| Cause |  | 1. LD status changed during media scan. 2. Drive failure during media scan (uncorrectable media error / unexpected select timeout / drive offline) |  |
| Action |  | 1. Check LD status. 2. Replace failed member drive if LD status is not good. 3. Restart media scan after LD status becomes to "good". |  |
| Event ID |  | 220A0706 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Media scan aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%lx Logical Drive WARN: No spare drive available for drive error recovery procedure |  |
| Cause |  | Media scan operation aborted because of an unrecoverable error and lack of an available spare drive for replacement procedure. |  |
| Action |  | 1. Prepare and assign a spare drive. 2. Restart media scan and the drive with errors will be recovered and replaced by the spare drive. 3. Users can also engage the drive clone and replace operation manually to replace the faulty drive. |  |
| Event ID |  | 220A0707 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Unrecovered media error detected |  |
| 1st Line Message |  | Drive unrecovered media error - # |  |
| Old Message |  | CHL:.1d ID:%.1d WARN: Media Scan Unrecoverable Error Encountered - %lx%08lx |  |
| Cause |  | Drive media error was found and failed to recover. |  |
| Action |  | 1. Replace the faulty drive. 2. Restart the drive media scan operation. |  |
| Event ID |  | 22080708 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Media scan stopped |  |
| 1st Line Message |  | Media scan failed |  |
| Old Message |  | WARN: CHL:%.1d ID:%.1d Media Scan Failed |  |
| Cause |  | Drive media error found and media scan operation failed to continue. |  |
| Action |  | 1. Replace faulty drive. 2. Restart media scan. |  |
| Event ID |  | 220A0709 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Media scan stopped |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x WARN: CHL:%.1d ID:%.1d Media Scan Failed |  |
| Cause |  | Logical drive media scan operation stopped because a specific drive error was found. |  |
| Action |  | 1. Replace the faulty drive. 2. Restart logical drive media scan. |  |
| Event ID |  | 220D2101 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LICENSE |  |
| Prefix Parameter |  |  |  |
| Message |  | License key error |  |
| 1st Line Message |  | Key consistency check failed |  |
| Old Message |  | License key data consistency check failed |  |
| Cause |  | Failed to install the license key to RAID subsystem. The cause may be license key corrupt or subsystem hardware error. |  |
| Action |  | 1. Re-apply then re-install the license key. 2. If issue was not fixed, replace the backplane. 3. If it accompanied other peripheral devices fail, replace the controller. |  |
| Event ID |  | 220D2102 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LICENSE |  |
| Prefix Parameter |  |  |  |
| Message |  | License key error |  |
| 1st Line Message |  | Unsupported key for installed firmware |  |
| Old Message |  | License key not supported by installed FW |  |
| Cause |  | 1. Unsupported features found in current configurations. 2. Controller and its firmware installed into an incompatible subsystem. |  |
| Action |  | 1. Update firmware to solve incompatible firmware and configuration issues. 2. Re-apply then re-install the license key. |  |
| Event ID |  | 220B4502 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume degraded |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume ALERT: Degraded |  |
| Cause |  | One or several member logical drives in a logical volume degraded due to: 1. Logical drive failure resulting from member drive errors. 2. Specific logical drive with errors. |  |
| Action |  | 1. Check and replace failed drives, then engage logical drive rebuild operation manually or automatically to recover the logical drive. 2. Check previous drive or logical drive event logs for cause of failure. |  |
| Event ID |  | 220B4505 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume member drive missing |  |
| 1st Line Message |  | Logical volume incompleted |  |
| Old Message |  | ID:%.1x Logical Volume ALERT: Incomplete Array |  |
| Cause |  | Specific member drive missing and caused an incomplete logical drive/volume. |  |
| Action |  | Check and reinstall the missing member drive and wait for logical volume recovery. |  |
| Event ID |  | 220B4506 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume member drive missing |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume ALERT: Missing Drive(s) |  |
| Cause |  | Specific member drive missing and caused logical volume errors. |  |
| Action |  | Check and reinstall the missing member drive and wait for the logical volume recovery. |  |
| Event ID |  | 220A4508 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive degraded |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Degraded |  |
| Cause |  | Specifc member drive in a logical drive missing or failed. |  |
| Action |  | Check and replace the failed drive, then engage logical drive rebuild manually or automatically to recover the logical drive. |  |
| Event ID |  | 220A450B |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive member drive missing |  |
| 1st Line Message |  | Logical drive incompleted |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Incomplete Array |  |
| Cause |  | Too many member drives failed or missing during system bootup. |  |
| Action |  | 1. Reinstall missing drives. 2. Reboot the system and check logical drive status again. 3. Replace faulty drives and recover the logical drive manually or automatically. 4. Remove the logical drive and create a new one if data recovery is not required. |  |
| Event ID |  | 220A450C |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive member drive missing |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Missing Drive(s) |  |
| Cause |  | 1. User removed the drive or drive tray was not properly inserted. 2. 2nd or more drives failed when LD status is degrade or rebuilding. 3. Mltiple LD member drive failed at the same time |  |
| Action |  | 1. Power off RAID unit and insert missing LD member drive then power on. 2. Check event log to see which LD member drive is the last failed drive and reboor RAID subsystem with this drive inserted. If the drive is back to on-line, backup data immediately. |  |
| Event ID |  | 220A0101 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive missing |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:NA Logical Drive ALERT: CHL:%.1d ID:%.1d Drive Missing |  |
| Cause |  | Specific member drive of logical drive missing. 1. User removed the drive or drive tray not properly inserted. 2. Drive failed or contains errors. 3. Backplane connection issue. |  |
| Action |  | 1. Check or reinstall the missing member drive. 2. Replace the failed drive and engage logical drive rebuild operation manually or automatically to recover logical drive. 3. Replace the backplane. |  |
| Event ID |  | 220A0102 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:NA Logical Drive ALERT: CHL:%.1d ID:%.1d Drive Failure |  |
| Cause |  | Specific member drive of logical drive failed. 1. User removed the drive. 2. Drive failed. 3. Backplane connection issue. |  |
| Action |  | 1. Insert the drive back. 2. Replace the backplane. |  |
| Event ID |  | 220A0103 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive removed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:NA Logical Drive NOTICE: CHL:%.1d ID:%.1d Drive Removed |  |
| Cause |  | Specific drive removal detected. |  |
| Action |  | Check and reinstall the removed drive. |  |
| Event ID |  | 220A0104 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Logical drive member drive missing |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d Drive Missing |  |
| Cause |  | Specific member drive of logical drive missing. 1. User removed the drive or drive tray was not properly inserted. 2. Drive failed. 3. Backplane connection issue. |  |
| Action |  | 1.Check, reinstall or install back the missed member drive. 2. If logical drive status is fatal fail, reboot the RAID subsystem to check if the missing drive can be recognized. If not, select System -> Export System Information in SANWatch or Embedded RAI |  |
| Event ID |  | 220A0105 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Logical drive member drive failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d Drive Failure |  |
| Cause |  | Specific member drive of logical drive was failed. 1. User removed the drive. 2. Drive failed. 3. Backplane connection issue. |  |
| Action |  | 1. Insert the drive back and rebuild the Logical Drive if needed. 2. Replace the failed drive and engage the logical drive rebuild operation manually or automatically to recover logical drive. 3. If it still failed after replacing with a new drive, replac |  |
| Event ID |  | 220A0106 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Logical drive member drive removed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: CHL:%.1d ID:%.1d Drive Removed |  |
| Cause |  | Specific drive removal detected. |  |
| Action |  | Check and reinstall the removed drive. |  |
| Event ID |  | 22080601 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | SCSI parity or CRC error |  |
| Old Message |  | CHL:%.1d ID:%.1d Target WARNING: SCSI Parity/CRC Error Detected |  |
| Cause |  | Parity or CRC errors detected between drive and controller. |  |
| Action |  | 1. Wait until error clears automatically, which is usually the case. 2. If error occours frequently, contact technical support for issue escalation. |  |
| Event ID |  | 22080101 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive unexpected select timeout |  |
| Old Message |  | CHL:%.1d ID:%.1d Target WARNING: Unexpected Select Timeout |  |
| Cause |  | 1. Drive removed or not properly installed. 2. Drive failed. 3. Backplane connection issue. |  |
| Action |  | 1. Check and reinstall drive. 2. Replace the failed drive. 3. Replace the backplane. 4. If issue was not fixed, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 21080202 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Gross phase or signal error detected |  |
| Old Message |  | CHL:%.1d ID:%.1d Target WARNING: Gross Phase/Signal Error Detected |  |
| Cause |  | 1. Abnormal signal found on drive channel. This may result from drive issues. 2. If several drives encounter this problem, it could be caused by unstable backplane connection. 3. If several drives encounter this problem in the same expansion enclosure, it |  |
| Action |  | 1. Replace the faulty drive if only one reports the error. 2. For multiple error within single JBOD enclosure, replace the SAS expansion cable. 3. If (2) can not work, replace the backplane. 4. If issue was not fixed, select System -> Export System Infor |  |
| Event ID |  | 22093F02 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Drive channel failed |  |
| 1st Line Message |  | SCSI channel failed: # |  |
| Old Message |  | CHL:%.1d ID:-1 Target WARNING: SCSI Channel Failure(0x%x) |  |
| Cause |  | Drive channel error encountered. |  |
| Action |  | 1. Check cabling between enclosures. 2. Contact technical support for issue escalation. |  |
| Event ID |  | 22093F03 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel failed |  |
| 1st Line Message |  | SCSI channel failed: # |  |
| Old Message |  | CHL:%.1d ID:-1 %s Host Channel WARNING: SCSI Channel Failure(0x%x) |  |
| Cause |  | Host channel error encountered. |  |
| Action |  | 1. Check cabling between host or network switch. 2. Contact technical support for issue escalation. |  |
| Event ID |  | 22093F04 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Drive channel failed |  |
| 1st Line Message |  | SCSI channel failed |  |
| Old Message |  | CHL:%.1d ID:-1 Target WARNING: SCSI Channel Failure |  |
| Cause |  | Drive channel error encountered. |  |
| Action |  | 1. Check cabling between enclosures. 2. Contact technical support for issue escalation. |  |
| Event ID |  | 22093F05 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Host channel failed |  |
| 1st Line Message |  | SCSI channel failed |  |
| Old Message |  | CHL:%.1d ID:-1 %s Host Channel WARNING: SCSI Channel Failure |  |
| Cause |  | Host channel error encountered. |  |
| Action |  | 1. Check cabling between host or network switch. 2. Contact technical support for issue escalation. |  |
| Event ID |  | 22080501 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive I/O timeout |  |
| Old Message |  | CHL:%.1d ID:%.1d Target WARNING: Timeout Waiting for I/O to Complete |  |
| Cause |  | Drive I/O response over timeout value (default 30 seconds) due to: 1. Unstable drive. 2. Improper drive-side parameters in RAID system settings. |  |
| Action |  | 1. Replace faulty drive. 2. Check the setting and modify it to default value. |  |
| Event ID |  | 22080701 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Data overrun or underrun |  |
| Old Message |  | CHL:%.1d ID:%.1d Target WARNING: Data Overrun/Underrun Detected |  |
| Cause |  | Data overrun/underrun occurred between hard drive and controller. |  |
| Action |  | 1. Errors typically clear automatically. 2. If unresolved, replace the hard drive.. |  |
| Event ID |  | 22080802 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Invalid status or sense data received (#) |  |
| Old Message |  | CHL:%.1d ID:%.1d Target WARNING: Invalid Status/Sense Data Received (%02x/%02x/%02x) |  |
| Cause |  | Invalid status reported by drive. |  |
| Action |  | 1. Error typically clears automatically. 2. If unresolved, contact technical support for issue escalation. |  |
| Event ID |  | 22080801 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Invalid status or sense data received |  |
| Old Message |  | CHL:%.1d ID:%.1d Target WARNING: Invalid Status/Sense Data Received |  |
| Cause |  | Invalid status data reports by drive. |  |
| Action |  | 1. Error typically clears automatically. 2. If unresolved, contact technical support for issue escalation. |  |
| Event ID |  | 21081102 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive not ready detected (#) |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Unexpected Drive Not Ready (%02x/%02x/%02x) |  |
| Cause |  | Drive signal contains errors. |  |
| Action |  | 1. Replace the problematic hard drive. 2. If issue was not fixed, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 21081101 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive not ready detected |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Unexpected Drive Not Ready |  |
| Cause |  | Drive signal contains errors. |  |
| Action |  | 1. Replace the problematic hard drive. 2. If issue was not fixed, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 22081402 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL # ID # |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Unit attention received (#) |  |
| Old Message |  | CHL:%.1d ID:%.1d Target WARNING: Unit Attention Received (%02x/%02x/%02x) |  |
| Cause |  | LUN (unit) access issue reported from drive. System usually will retry and recover the drive condition. If the drive condition is bad, system retry may fail and results in drive failure or missing. |  |
| Action |  | 1. Replace drive if drive failure or missing is reported. 2. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 22081401 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL # ID # |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Unit attention received |  |
| Old Message |  | CHL:%.1d ID:%.1d Target WARNING: Unit Attention Received |  |
| Cause |  | Unit attention received on drive. |  |
| Action |  | 1. Error typically clears automatically. 2. If unresolved, contact technical support for issue escalation. |  |
| Event ID |  | 21081202 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive hardware error detected (#) |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Drive HW Error (%02x/%02x/%02x) |  |
| Cause |  | Drive hardware error detected and drive failed. |  |
| Action |  | Replace failed drive. |  |
| Event ID |  | 21081201 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive hardware error detected |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Drive HW Error |  |
| Cause |  | Drive hardware error detected and drive failed. |  |
| Action |  | Replace failed drive. |  |
| Event ID |  | 22081802 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive command aborted (#) |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Aborted Command (%02x/%02x/%02x) |  |
| Cause |  | Disk drive commands aborted due to drive failure. |  |
| Action |  | Replace faulty drive. |  |
| Event ID |  | 22081801 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive command aborted |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Aborted Command |  |
| Cause |  | Disk drive commands aborted due to drive failure. |  |
| Action |  | Replace faulty drive. |  |
| Event ID |  | 22081502 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Unexpected sense data received (#) |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Unexpected Sense Received (%02x/%02x/%02x) |  |
| Cause |  | Unexpected sense data reported from disk due to: 1. Drive issue 2. Slot issue 3. JBOD SAS cable issue |  |
| Action |  | 1. If the issue happens on only single drive, replace the drive. 2. If the issue happens on a large number of drives (ex. every drives in an enclosure), check and replace the SAS cable base on the slot A path or slot B path. 3. If the issue remains after |  |
| Event ID |  | 22081501 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Unexpected sense data received |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Unexpected Sense Received |  |
| Cause |  | Unexpected sense data reported from drive due to: 1. Disk issue 2. Slot issue 3. JBOD SAS cable issue |  |
| Action |  | 1. If the issue happens on only single drive, replace the drive. 2. If the issue happens on a large number of drives (ex. every drives in an enclosure), check and replace the SAS cable base on the slot A path or slot B path. 3. If the issue remains after |  |
| Event ID |  | 22081602 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive error block failed to reassign - # (#) |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Block Reassignment Failed - 0x%s (%02x/%02x/%02x) |  |
| Cause |  | Drive write media error reported internally inside the drive and cause data reassignment failed. |  |
| Action |  | Replace the hard drive. |  |
| Event ID |  | 22081601 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error detected |  |
| 1st Line Message |  | Drive error block failed to reassign - # |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive WARNING: Block Reassignment Failed - 0x%s |  |
| Cause |  | Write media error reported within the drive, but hard drive reassignment to avoid bad block failed. |  |
| Action |  | Replace the hard drive. |  |
| Event ID |  | 21032101 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage unstable |  |
| 1st Line Message |  | +3.3V low voltage detected (#) |  |
| Old Message |  | ALERT: +3.3V Low Voltage Detected(%s) |  |
| Cause |  | Voltage lower than threshold. 1. Power instability or power supply malfunction. 2. Voltage threshold set too high. 3. Controller error detection malfunction. |  |
| Action |  | 1. Check power status and power cord or replace the power supply unit. 2. Restore default controller settings. 3. Replace the controller if necessary. |  |
| Event ID |  | 21032102 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage unstable |  |
| 1st Line Message |  | +5V low voltage detected (#) |  |
| Old Message |  | ALERT: +5V Low Voltage Detected(%s) |  |
| Cause |  | Voltage lower than threshold. 1. Power instability or power supply malfunction. 2. Voltage threshold set too high. 3. Controller error detection malfunction. |  |
| Action |  | 1. Check power status and power cord or replace the power supply unit. 2. Restore default controller settings. 3. Replace the controller if necessary. |  |
| Event ID |  | 21032103 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage unstable |  |
| 1st Line Message |  | +12V low voltage detected (#) |  |
| Old Message |  | ALERT: +12V Low Voltage Detected(%s) |  |
| Cause |  | Voltage lower than threshold. 1. Power instability or power supply malfunction. 2. Voltage threshold set too high. 3. Controller error detection malfunction. |  |
| Action |  | 1. Check power status and power cord or replace the power supply unit. 2. Restore default controller settings. 3. Replace the controller if necessary. |  |
| Event ID |  | 21032104 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage unstable |  |
| 1st Line Message |  | +3.3V high voltage detected (#) |  |
| Old Message |  | ALERT: +3.3V High Voltage Detected(%s) |  |
| Cause |  | Voltage higher than threshold. 1. Power instability or power supply malfunction. 2. Voltage threshold set too low. 3. Controller error detection malfunction. |  |
| Action |  | 1. Check power status and power cord or replace the power supply unit. 2. Restore default controller settings. 3. Replace the controller if necessary. |  |
| Event ID |  | 21032105 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage unstable |  |
| 1st Line Message |  | +5V high voltage detected (#) |  |
| Old Message |  | ALERT: +5V High Voltage Detected(%s) |  |
| Cause |  | Voltage higher than threshold. 1. Power instability or power supply malfunction. 2. Voltage threshold set too low. 3. Controller error detection malfunction. |  |
| Action |  | 1. Check power status and power cord or replace the power supply unit. 2. Restore default controller settings. 3. Replace the controller if necessary. |  |
| Event ID |  | 21032106 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Power supply voltage unstable |  |
| 1st Line Message |  | +12V high voltage detected (#) |  |
| Old Message |  | ALERT: +12V High Voltage Detected(%s) |  |
| Cause |  | Voltage higher than threshold. 1. Power instability or power supply malfunction. 2. Voltage threshold set too low. 3. Controller error detection malfunction. |  |
| Action |  | 1. Check power status and power cord or replace the power supply unit. 2. Restore default controller settings. 3. Replace the controller if necessary. |  |
| Event ID |  | 21032107 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure power supply sensor detection failed |  |
| 1st Line Message |  | I2C device failed (#) |  |
| Old Message |  | Peripheral Device ALERT (P%02d%02d) |  |
| Cause |  | The I2C sensor on the backplane is malfunction which leads to power supply error. |  |
| Action |  | 1. Contact vendor to obtain backplane and replacement SOP. 2. If replacement backplane was not fixed, replace the controller. |  |
| Event ID |  | 21032111 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # power supply # absent |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Power Supply %d ALERT:Device Not Installed ! |  |
| Cause |  | Power supply in expansion enclosure was not installed properly. |  |
| Action |  | 1. Check power status and power cord. 2. Check and reinstall the power supply unit. |  |
| Event ID |  | 2103211E |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # power supply # failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Power Supply %d ALERT:Power Supply Failure Detected |  |
| Cause |  | Expansion enclosure power supply undetectable or in critical status. |  |
| Action |  | 1. Check power connection. 2. Reinstall power supply 3. Replace the power supply unit. 4. If problem persists, contact technical support for issue escalation. |  |
| Event ID |  | 21052201 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure fan sensor detection failed |  |
| 1st Line Message |  | I2C device failed (#) |  |
| Old Message |  | Peripheral Device ALERT (F%02d%02d) |  |
| Cause |  | The I2C sensor on the backplane is malfunction which leads to fan error. |  |
| Action |  | 1. Contact vendor to obtain backplane and replacement SOP. 2. If the issue was not fixed, replace the controller. |  |
| Event ID |  | 21052213 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # fan # absent |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Cooling Fan %d ALERT:Device Not Installed ! |  |
| Cause |  | Expansion enclosure fan module not installed properly. |  |
| Action |  | Check and reinstall fan module. |  |
| Event ID |  | 21052216 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # fan # failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Cooling Fan %d ALERT:Failure Detected |  |
| Cause |  | Fan module failure detected. |  |
| Action |  | Replace the fan module. |  |
| Event ID |  | 21072301 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | CPU low temperature detected |  |
| 1st Line Message |  | Temperature: #(C) |  |
| Old Message |  | Peripheral Device ALERT: CPU Cold Temperature Detected(%d.0(C) ) |  |
| Cause |  | Controller CPU temperature below threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21072302 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | CPU high temperature detected |  |
| 1st Line Message |  | Temperature: #(C) |  |
| Old Message |  | Peripheral Device ALERT: CPU Hot Temperature Detected(%d.0(C) ) |  |
| Cause |  | Controller CPU temperature over threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21072303 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller low temperature detected |  |
| 1st Line Message |  | ASIC temperature: #(C) |  |
| Old Message |  | Peripheral Device ALERT: Controller Cold Temperature Detected(1)(%d.0(C) ) |  |
| Cause |  | Controller ASIC temperature below threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21072304 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller high temperature detected |  |
| 1st Line Message |  | ASIC temperature: #(C) |  |
| Old Message |  | Peripheral Device ALERT: Controller Hot Temperature Detected(1)(%d.0(C) ) |  |
| Cause |  | Controller ASIC temperature over threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21072305 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller low temperature detected |  |
| 1st Line Message |  | Drive channel I/O chip temperature: #(C) |  |
| Old Message |  | Peripheral Device ALERT: Controller Cold Temperature Detected(2)(%d.0(C) ) |  |
| Cause |  | Controller drive I/O chip temperature below the threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21072306 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller high temperature detected |  |
| 1st Line Message |  | Drive channel I/O chip temperature: #(C) |  |
| Old Message |  | Peripheral Device ALERT: Controller Hot Temperature Detected(2)(%d.0(C) ) |  |
| Cause |  | Controller drive I/O chip temperature over the threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21072307 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller low temperature detected |  |
| 1st Line Message |  | Host board I/O chip temperature: #(C) |  |
| Old Message |  | Peripheral Device ALERT: Controller Cold Temperature Detected(3)(%d.0(C) ) |  |
| Cause |  | Controller host I/O chip temperature below the threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21072308 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller high temperature detected |  |
| 1st Line Message |  | Host board I/O chip temperature: #(C) |  |
| Old Message |  | Peripheral Device ALERT: Controller Hot Temperature Detected(3)(%d.0(C) ) |  |
| Cause |  | Controller host I/O chip temperature over the threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21072309 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure backplane temperature sensor detection failed |  |
| 1st Line Message |  | I2C device failed (#) |  |
| Old Message |  | Peripheral Device ALERT (T%02d%02d) |  |
| Cause |  | The I2C sensor on the backplane is malfunction which leads to temperature error. |  |
| Action |  | 1. Contact vendor to obtain backplane and replacement SOP. 2. If the issue was not fixed, replace the controller. |  |
| Event ID |  | 21010404 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller memory error detected |  |
| 1st Line Message |  | Memory ECC single bit error detected |  |
| Old Message |  | Controller WARNING: Memory ECC Single-bit Error Detected |  |
| Cause |  | The data bus of memory module or memory slot of controller contains signal errors. |  |
| Action |  | 1. Replace the memory first if user has a spare one. 2. If the issue was not fixed, replace the controller. |  |
| Event ID |  | 220A0502 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive expansion failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Expansion Failed |  |
| Cause |  | Logical drive expansion operation failed because member drive fails during off-line initialization. |  |
| Action |  | Replace the failed drive. |  |
| Event ID |  | 220A0302 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive rebuild failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Rebuild Failed |  |
| Cause |  | Logical drive rebuild operation failed due to: 1. Replacement drive contains errors. 2. Member drive failed during rebuild operation. |  |
| Action |  | 1. Replace faulty drive and restart the rebuild operation. 2. Reboot the RAID to see if the newly failed drive can be recognized again. If so, restart rebuild process. If not, clone the newly failed drive and check if cloned drive can be recognized, and |  |
| Event ID |  | 220A0401 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive parity regeneration aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Parity Regeneration Aborted |  |
| Cause |  | Logical drive parity regeneration aborted due to: 1. User cancelled the operation manually. 2. Member drive failed during regeneration operation. 3. Inconsistent parity issues found. |  |
| Action |  | 1. Users can restart the operation manually. 2. Replace failed member drive and rebuild the logical drive. 3. Start a parity regeneration operation for this logical drive manually. |  |
| Event ID |  | 220A0402 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive parity regeneration failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Parity Regeneration Failed |  |
| Cause |  | Logical drive parity regeneration failed due to: 1. Member drive failed during regeneration operation. |  |
| Action |  | 1. Replace failed member drive and rebuild the logical drive. |  |
| Event ID |  | 220A0201 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive initialization failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Initialization Failed |  |
| Cause |  | Logical drive experiences errors during initialization, possibly caused by member drive errors. |  |
| Action |  | Replace the failed drive, and rebuild the LD if needed. |  |
| Event ID |  | 220A0202 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive creation aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Creation Aborted |  |
| Cause |  | When Logical Drive is in off-line initialization state. User cancels logical drive creation. |  |
| Action |  | Re-create Logical Drive again. |  |
| Event ID |  | 220A0203 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive creation failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Creation Failed |  |
| Cause |  | Logical drive creation failed due to member drive issues: 1. Member drive removed. 2. Specific member drive failed. |  |
| Action |  | 1. Check and reinstall faulty member drive. 2. Replace failed member drive. 3. Restart the creation operation. |  |
| Event ID |  | 220A4101 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive error detected |  |
| 1st Line Message |  | Logical drive bad block count exceeded |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Logical Drive Bad Block Table FULL |  |
| Cause |  | Too many bad blocks found in a logical drive. There might be too many damaged sectors on the drive. |  |
| Action |  | 1. Replace the faulty drive. 2. Restore data from a backup source. |  |
| Event ID |  | 220A4102 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive error detected |  |
| 1st Line Message |  | Logical drive bad block table corrupted |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Logical Drive Bad Block Table BAD |  |
| Cause |  | Bad block table in logical drive not created successfully and corrupted. |  |
| Action |  | Contact technical support for issue escalation. |  |
| Event ID |  | 220A4103 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive error detected |  |
| 1st Line Message |  | Logical drive bad block table corrupted |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Logical Drive On-Line Init Table BAD |  |
| Cause |  | Media error/drive failure during LD online initiation and init table is not created successfully or corrupted, it caused data integrity issue. |  |
| Action |  | 1. Backup data on the logical drive. 2. Delete problematic logical drive and re-create it. |  |
| Event ID |  | 220A4201 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive bad data block detected and marked |  |
| 1st Line Message |  | Bad data block marked - # |  |
| Old Message |  | LG:%.1d Logical Drive ALERT: Bad Data Block Marked %bx%08lx |  |
| Cause |  | Bad blocks found in logical drive, marked bad, and corrupted during write operations. (1.multiple media error occurs in same LD stripe, which cause Bad blocks on logical drive, marked bad, and corrupted during write IO operations. 2. when LD in degraded s |  |
| Action |  | 1. Engage media scan to check all member drives in a logical drive. 2. Restore data from a backup source if necessary. (1. Engage media scan to check all member drives in a logical drive. 2. Replace the drive which media error reported by executing clone |  |
| Event ID |  | 220A4202 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Unprotected block on the logical drive detected and marked |  |
| 1st Line Message |  | Unprotected bad data block marked - # |  |
| Old Message |  | LG:%.1d Logical Drive ALERT: UNPROTECTED Block Marked %bx%08lx |  |
| Cause |  | Bad blocks were found in a logical drive and marked unprotected. |  |
| Action |  | 1. Engage media scan to check all member drives in a logical drive. 2. Restore data from a backup source if necessary. |  |
| Event ID |  | 220A4204 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive bad data block detected |  |
| 1st Line Message |  | Bad data block - # |  |
| Old Message |  | LG:%.1d Logical Drive ALERT: Bad Data Block Encountered %bx%08lx |  |
| Cause |  | Bad blocks were found in a logical drive and data may be corrupted during I/O operations. (1. Another drive media error were found in a logical drive during LD rebuilding, so data can not be recovered by parity calculation 2. Multiple media error occurs |  |
| Action |  | 1. Engage media scan to check all member drives in a logical drive. 2. Restore data from a backup source if necessary. (1. Backup the data. 2. Engage media scan to check all member drives in a logical drive. 3. Restore data from a backup source 4. Re-crea |  |
| Event ID |  | 22084302 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Unrecovered drive media error detected |  |
| 1st Line Message |  | Unrecoverable media error - # |  |
| Old Message |  | CHL:%.1d ID:%.1d ALERT:Media Error Unrecoverable-0x%s |  |
| Cause |  | Drive damaged physically and became unreliable. |  |
| Action |  | Replace faulty drives. |  |
| Event ID |  | 2108C103 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Unsupported drive detected |  |
| 1st Line Message |  | Unsupported code: # |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive NOTICE: Unsupported Drive Detected(0x%08lx) |  |
| Cause |  | Drive type (for ESVA only) or license is invalid and not supported by RAID subsystem. |  |
| Action |  | 1. Replace unrecognized drive with drive supplied by vendor. 2. If issue was not fixed, select System -> Export System Information from the latest Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 2108C104 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Unsupported drive detected |  |
| 1st Line Message |  | Incorrect drive bundle ID: # |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive NOTICE: Unsupported Drive Detected(0x%04lx) |  |
| Cause |  | Drive with incorrect bundle ID found in subsystem. |  |
| Action |  | 1. Replace unsupported drives with the one supplied by vendor. 2. If issue was not fixed, select System -> Export System Information from the latest Management GUI. Contact technical support with System information for issue escalation |  |
| Event ID |  | 2108C105 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Unsupported drive detected |  |
| 1st Line Message |  | Drive bundle code not supported. ID: #, code: # |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive NOTICE: Unsupported Drive Detected(0x%04lx,0x%04lx) |  |
| Cause |  | Drive with unsupported bundle code found in subsystem. |  |
| Action |  | 1. Replace unsupported drive with the one supplied by vendor. 2. If issue was not fixed, select System -> Export System Information from the latest Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 2208C107 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive detection failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | CHL:%.1d ID:%.1d Drive NOTICE: Scan Drive Failed |  |
| Cause |  | System fail to scan in drive due to: 1. disk hardware issue 2. enclosure hardware issue |  |
| Action |  | 1. Reinsert disk. 2. If problem still persist, or if disk failure event also occurs, replace disk. 3. If problem still persist, replace the enclosure. |  |
| Event ID |  | 22084202 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Unable to start drive error recovery procedure |  |
| 1st Line Message |  | No spare drive available |  |
| Old Message |  | ID:%lx Logical Drive WARN: No Available Spare Drive for Clone |  |
| Cause |  | Drive error found and recovery procedure halted due to lack of spare drive. |  |
| Action |  | 1. Prepare and assign a spare drive. 2. Restart media scan and the drive with errors will be recovered and replaced by the spare. 3. Users can engage drive clone and replace operation manually to replace the faulty drive. |  |
| Event ID |  | 22084205 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - DRIVE |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive error recovery procedure stopped |  |
| 1st Line Message |  | Clone failed |  |
| Old Message |  | WARNING:SMART-CH %.1d ID:%.1d Predictable Failure Detected-Clone Failed |  |
| Cause |  | Drive data clone operation stopped due to drive failure. |  |
| Action |  | 1. Replace the failed drive. 2. Restart clone operation manually. |  |
| Event ID |  | 220A1101 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Drive clone failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT:CHL:%.1d ID:%.1d Clone Failed |  |
| Cause |  | Drive data clone operation stopped in a specific logical drive because of drive failure. |  |
| Action |  | 1. Replace the failed drive. 2. Restart the clone operation manually. |  |
| Event ID |  | 220A1102 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# CHANNEL:# ID:# |  |
| Message |  | Drive clone aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT:CHL:%.1d ID:%.1d Clone Aborted |  |
| Cause |  | Drive data clone operation stopped in a specific logical drive due to: 1. User cancelled the operation manually. 2. Source drive failed during the clone operation. 3. Target drive failed during the clone operation. |  |
| Action |  | 1. Restart the operation manually. 2. If a spare drive is available, the rebuild operation will start automatically. 3. If there is no spare drive, rebuild will use the target drive as a spare before cloning. 4. Replace the failed target drive. |  |
| Event ID |  | 22093F07 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Drive channel failed |  |
| 1st Line Message |  | Redundant path error detected: # |  |
| Old Message |  | CHL:%.1d WARNING:Redundant Path for Chl:%.1d Failure Detected |  |
| Cause |  | Redundant path connection to drive channel failed. |  |
| Action |  | Check cables between enclosures and host are connected properly. |  |
| Event ID |  | 22093F09 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# ID:# |  |
| Message |  | Drive channel failed |  |
| 1st Line Message |  | Redundant path error detected: # # |  |
| Old Message |  | CHL:%.1d ID:%.1d WARNING:Redundant Path for Chl:%.1d ID:%.1d Failure Detected |  |
| Cause |  | Redundant path connection to drive channel failed. |  |
| Action |  | Check cables between enclosures and host are connected properly. |  |
| Event ID |  | 220A4401 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive inconsistent parity block detected |  |
| 1st Line Message |  | Inconsistent parity block - # |  |
| Old Message |  | LG:%.1d ALERT:Inconsistent Parity Encountered Block %bx%08lx |  |
| Cause |  | nconsistent parity data found during parity regeneration operation. |  |
| Action |  | 1. Engage parity regeneration operation for the logical drive manually. 2. If failed, restore data from a backup source. |  |
| Event ID |  | 220A4403 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive media error detected |  |
| 1st Line Message |  | Media error detected during parity check - # |  |
| Old Message |  | LG:%.1d ALERT:Media Error Encountered During Check Parity %bx%08lx |  |
| Cause |  | Media error found during parity check regeneration operation. |  |
| Action |  | 1. Check if any drive media error found and replace the drive. 2. Manually engage parity regeneration for the logical drive. 3. If failed, restore data from a backup source. |  |
| Event ID |  | 21042502 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller flash backup module (FBM) absent |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT:Controller FBM Absent |  |
| Cause |  | Failed to detect controller FBM. It may have failed or was not installed properly. |  |
| Action |  | 1. Check FBM installation. 2. If still not working, change it per FBM replacement procedure. |  |
| Event ID |  | 21042503 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller flash backup module (FBM) failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT:Controller FBM Failed |  |
| Cause |  | Controller FBM has errors. |  |
| Action |  | 1. Check FBM installation. 2. If still not working, change it per FBM replacement procedure. |  |
| Event ID |  | 21042504 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller battery backup unit (BBU) absent or failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT:Controller BBU Absent or Failed ! |  |
| Cause |  | Failed to detect controller BBU. It may have failed or was not installed properly. |  |
| Action |  | 1. Check BBU installation. 2. If still not working, change it per BBU replacement procedure. |  |
| Event ID |  | 21042505 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller battery backup unit (BBU) failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT:Controller BBU Failure Detected ! |  |
| Cause |  | Controller BBU has errors. |  |
| Action |  | 1. Check BBU installation. 2. If still not working, change it per BBU replacement procedure. |  |
| Event ID |  | 21042507 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller battery backup unit (BBU) error detected |  |
| 1st Line Message |  | BBU entered self-protection state |  |
| Old Message |  | ALERT:Controller BBU charger entered self-protection state |  |
| Cause |  | BBU malfunction. 1. BBU temperature may be over 45 C°. 2. BBU charged for over than 14 hours. |  |
| Action |  | 1. Check ambient temperatures and make sure enclosure air flow is adequate. 2. BBU entered a timer fault state and its internal timer has to be reset. Remove the BBU and reinsert it. 3. Empty BBU takes about 14.3 hours to be fully charged. Then charging w |  |
| Event ID |  | 21021101 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Invalid or conflicting enclosure ID detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT: Enclosure ID Conflict/Invalid |  |
| Cause |  | Enclosure ID invalid or duplicated and conflicting with other enclosures. |  |
| Action |  | 1. Make sure each enclosure has a unique ID. 2. Follow expansion enclosure ID adjustment procedure to modify enclosure ID and solve the issue. |  |
| Event ID |  | 21021201 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drive configuration error detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT: Invalid Drive Configuration |  |
| Cause |  | Drive type mismatch found: SAS drive installed in the SATA drive-only subsystem, or vice versa. |  |
| Action |  | Install drives of the same type or check supported drive type in the subsystem. |  |
| Event ID |  | 21021202 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # drive configuration error detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT: Enclosure ID %d Invalid Drive Configuration |  |
| Cause |  | Drive type mismatch found: SAS drive installed in the SATA drive-only expansion enclosure, or vice versa. |  |
| Action |  | Install drives of the same type in the expansion enclosure. |  |
| Event ID |  | 220B4601 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume status changed from online to offline |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume NOTICE: State change from on-line to off-line |  |
| Cause |  | Member LD in LV fatal failed. |  |
| Action |  | Check previous events if any LD issue caused LV offline. Follow the service guide to fix LD issue then LV would become online automatically. |  |
| Event ID |  | 220A4606 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive status changed from online to offline |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive NOTICE: State change from on-line to off-line |  |
| Cause |  | 1. RAID subsystem is shutdown by user operation 2. Multiple LD member drive missing caused LD not online |  |
| Action |  | 1. No action is required or user can reboot RAID system. 2. Check previous events if several LD member drives missing. Follow the service guide to fixed LD member drives missing. 3. If issue was not fixed, select System -> Export System Information in SAN |  |
| Event ID |  | 22091501 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Trunking configuration error detected (slot B) |  |
| 1st Line Message |  |  |  |
| Old Message |  | WARNING: Invalid trunking configuration for Slot B-Ch%d |  |
| Cause |  | 1. Switch ports connecting with controller B may not have been configured as trunk ports. 2. Host ports connecting with controller B may not have been configured as trunk ports. 3. Link Aggregation Control Protocol (LACP) setting on switch is invalid |  |
| Action |  | 1. Check trunking (link aggression) settings on network switch 2. Check if host ports have been configured as trunk ports. 3. Check if LACP setting on switch is disabled. |  |
| Event ID |  | 22091502 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - CHANNEL |  |
| Prefix Parameter |  | CHANNEL:# |  |
| Message |  | Trunking configuration error detected (slot A) |  |
| 1st Line Message |  |  |  |
| Old Message |  | WARNING: Invalid trunking configuration for Slot A-Ch%d |  |
| Cause |  | 1. Switch ports connecting with controller A may not have been configured as trunk ports. 2. Host ports connecting with controller A may not have been configured as trunk ports. 3. Link Aggregation Control Protocol (LACP) setting on switch is invalid |  |
| Action |  | 1. Check trunking (link aggression) settings on network switch 2. Check if host ports have been configured as trunk ports. 3. Check if LACP setting on switch is disabled |  |
| Event ID |  | 21021301 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # not supported |  |
| 1st Line Message |  |  |  |
| Old Message |  | NOTICE: Unsupported Disk Expansion Enclosure(%d) detected |  |
| Cause |  | Expansion enclosure not supported by connected subsystem. |  |
| Action |  | Verify compatiblility between subsystem and expansion enclosure. |  |
| Event ID |  | 21020103 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer # not detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | WARNING: Enclosure drawer %d not ready detected |  |
| Cause |  | Enclosure drawer not closed properly or is open. |  |
| Action |  | Check or re-install the drawer. |  |
| Event ID |  | 21020104 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - ENCLOSURE |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # drawer # not ready detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | WARNING: SES(C%d I%d)Enclosure drawer %d not ready detected |  |
| Cause |  | Expansion enclosure drawer not closed properly or is open. |  |
| Action |  | Check or re-install the drawer. |  |
| Event ID |  | 21032108 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # power supply # failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d(%d) Device ALERT:PSU0 Failure Detected |  |
| Cause |  | Power supply unit in expansion enclosure failed due to: 1. Power grid issues or insufficient power delivery. 2. Power supply unit malfunction. |  |
| Action |  | 1. Check power status and power cord. 2. Replace the power supply unit. |  |
| Event ID |  | 21032109 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure power supply # failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d Device ALERT:PSU0 Failure Detected |  |
| Cause |  | Power supply unit in expansion enclosure failed due to: 1. Power grid issues or insufficient power delivery. 2. Power supply unit malfunction. |  |
| Action |  | 1. Check power status and power cord. 2. Replace the power supply unit. |  |
| Event ID |  | 2103210C |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # power supply # absent |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d(%d) Device ALERT:PSU0 Not Present |  |
| Cause |  | Power supply unit in expansion enclosure is missing. |  |
| Action |  | 1. Check power status and power cord. 2. Check and reinstall the power supply unit. |  |
| Event ID |  | 2103210D |  |
| Severity |  | Error |  |
| Category-Module |  | HW - PSU |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure power supply # absent |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d Device ALERT:PSU0 Not Present |  |
| Cause |  | Power supply unit in enclosure is missing. |  |
| Action |  | 1. Check power status and power cord. 2. Check and reinstall the power supply unit. |  |
| Event ID |  | 2105220A |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # fan # failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d(%d) Device ALERT: Cooling Fan0 Failure Detected |  |
| Cause |  | Expansion enclosure fan module failure detected. |  |
| Action |  | Replace fan module. |  |
| Event ID |  | 2105220B |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure fan # failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d Device ALERT: Cooling Fan0 Failure Detected |  |
| Cause |  | Expansion enclosure fan module failure detected. |  |
| Action |  | Replace fan module. |  |
| Event ID |  | 2105220C |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # fan # absent |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d(%d) Device ALERT: Cooling Fan0 Not Present |  |
| Cause |  | Expansion enclosure fan module missing or not installed properly. |  |
| Action |  | 1. Check and reinstall fan module. 2. Replace fan module if needed. |  |
| Event ID |  | 2105220D |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure fan # absent |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d Device ALERT: Cooling Fan0 Not Present |  |
| Cause |  | Expansion enclosure fan module missing or not installed properly. |  |
| Action |  | 1. Check and reinstall fan module. 2. Replace fan module if needed. |  |
| Event ID |  | 2105220E |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # fan # low speed detected |  |
| 1st Line Message |  | RPM: # |  |
| Old Message |  | Peripheral Set %d(%d) Device ALERT: Cooling Fan0 Low Speed Detected(%.1d RPM) |  |
| Cause |  | Expansion enclosure fan module set to low speed or malfunctioning. |  |
| Action |  | Check or replace fan module. |  |
| Event ID |  | 21052210 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure fan # low speed detected |  |
| 1st Line Message |  | RPM: # |  |
| Old Message |  | Peripheral Set %d Device ALERT: Cooling Fan0 Low Speed Detected(%.1d RPM) |  |
| Cause |  | Expansion enclosure fan module set to low speed or malfunctioning. |  |
| Action |  | Check or replace fan module. |  |
| Event ID |  | 2105224B |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer fan # failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d Device ALERT: Drawer fan0 Failure Detected |  |
| Cause |  | Enclosure drawer fan module failure detected. |  |
| Action |  | Replace the fan module. |  |
| Event ID |  | 2105224D |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer fan # absent |  |
| 1st Line Message |  |  |  |
| Old Message |  | Peripheral Set %d Device ALERT: Drawer fan0 Not Present |  |
| Cause |  | Enclosure drawer fan module missing or not installed properly. |  |
| Action |  | 1. Check and reinstall fan module. 2. Replace fan module if needed. |  |
| Event ID |  | 21052250 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure drawer fan # low speed detected |  |
| 1st Line Message |  | RPM: # |  |
| Old Message |  | Peripheral Set %d Device ALERT: Drawer fan0 Low Speed Detected(%.1d RPM) |  |
| Cause |  | Enclosure fan module set to low speed or malfunctioning. |  |
| Action |  | Check or replace fan module. |  |
| Event ID |  | 21072316 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # backplane low temperature detected |  |
| 1st Line Message |  | Temperature: #(C) |  |
| Old Message |  | Peripheral Set %d(%d) Device ALERT: Backplane Temperature Cold Detected(%.1d(C)) |  |
| Cause |  | The temperature in expansion enclosure is too low below the threshold. It may because of environmental factor or backplane sensor malfunction. |  |
| Action |  | 1. Check ambient temperature and adjust it. 2. Replace the backplane. 3. If the issue was not fixed, contact vendor support for issue escalation. |  |
| Event ID |  | 21072317 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Expansion enclosure # backplane high temperature detected |  |
| 1st Line Message |  | Temperature: #(C) |  |
| Old Message |  | Peripheral Set %d(%d) Device ALERT: Backplane Temperature Hot Detected(%.1d(C)) |  |
| Cause |  | Temperature in enclosure over threshold due to: 1. Ambient temperature too high. 2. Enclosure cooling fan module failure. 3. Backplane sensor may be malfunctioning. |  |
| Action |  | 1. Check ambient temperature and make sure enclosure air flow is adequate. 2. Check or replace fan module. 3. Replace the backplane. If the issue was not fixed, contact technical support for issue escalation. |  |
| Event ID |  | 21072319 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Enclosure backplane high temperature detected |  |
| 1st Line Message |  | Temperature: #(C) |  |
| Old Message |  | Peripheral Set %d Device ALERT: Backplane Temperature Hot Detected(%.1d(C)) |  |
| Cause |  | Temperature in enclosure over threshold due to: 1. Ambient temperature too high. 2. Enclosure cooling fan module failure. 3. Backplane sensor may be malfunctioning. |  |
| Action |  | 1. Check ambient temperature and make sure enclosure air flow is adequate. 2. Check or replace fan module. 3. Replace the backplane. If the issue was not fixed, contact technical support for issue escalation. |  |
| Event ID |  | 22010901 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | A non-supported host board has been installed |  |
| 1st Line Message |  | A non-supported host board # has been installed |  |
| Old Message |  | A non-supported host board %d is installed(%x,%x) |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 21042508 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller Super-Capacitor absent or failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT:Controller Super-Capacitor Absent or Failed ! |  |
| Cause |  | 1. Failed to detect controller Super-Capacitor. It may have failed or was not installed properly. 2. Super-Capacitor charged for over than 10 minutes |  |
| Action |  | 1. CheckSuper-Capacitor installation. 2. If still not working, change it per Super-Capacitor replacement procedure. 3. Empty Super-Capacitor takes about 10 minutes to be fully charged. |  |
| Event ID |  | 21042509 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller Super-Capacitor failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ALERT:Controller Super-Capacitor Failure Detected ! |  |
| Cause |  | ControllerSuper-Capacitor has errors. |  |
| Action |  | 1. Check Super-Capacitor installation. 2. If still not working, change it per Super-Capacitor replacement procedure. |  |
| Event ID |  | 2104250B |  |
| Severity |  | Error |  |
| Category-Module |  | HW - BBU |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller Super-Capacitor error detected |  |
| 1st Line Message |  | Super-Capacitor entered self-protection state |  |
| Old Message |  | ALERT:Controller Super-Capacitor charger entered self-protection state |  |
| Cause |  | Super-Capacitor malfunction. 1. Super-Capacitor temperature may be over 65 C°. |  |
| Action |  | 1. Check ambient temperatures and make sure enclosure air flow is adequate. 2.Super-Capacitor entered a timer fault state and its internal timer has to be reset. Remove the BBU and reinsert it. |  |
| Event ID |  | 21062420 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - UPS |  |
| Prefix Parameter |  |  |  |
| Message |  | UPS Connection Lost |  |
| 1st Line Message |  |  |  |
| Old Message |  | UPS Connection Is Absent |  |
| Cause |  |  |  |
| Action |  | 1. Check the connection between the device and UPS. 2. Reconnect the device and UPS. |  |
| Event ID |  | 21062421 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - UPS |  |
| Prefix Parameter |  |  |  |
| Message |  | UPS AC Power Loss Detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | UPS AC Power-Loss Detected |  |
| Cause |  |  |  |
| Action |  | 1. Check AC power. 2. If the power cable of UPS has been unplugged, replug the power cable. 3. If AC power outage, shutdown the device ASAP. |  |
| Event ID |  | 21062422 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - UPS |  |
| Prefix Parameter |  |  |  |
| Message |  | UPS Low Battery Detected - #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | UPS Battery Low - %d%. |  |
| Cause |  |  |  |
| Action |  | Shutdown the device ASAP. |  |
| Event ID |  | 21062423 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - UPS |  |
| Prefix Parameter |  |  |  |
| Message |  | UPS Low Battery Detected - #%. Please shut down immediately to prevent data loss. |  |
| 1st Line Message |  |  |  |
| Old Message |  | UPS Battery Low - %d%. Please Shut down to protect data loss! |  |
| Cause |  |  |  |
| Action |  | Shutdown the device ASAP. |  |
| Event ID |  | 21072325 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller high temperature detected |  |
| 1st Line Message |  | Host board I/O chip temperature: #(C) |  |
| Old Message |  | Peripheral Device ALERT: Controller Hot Temperature Detected(4)(%d.0(C) ) |  |
| Cause |  | Controller host I/O chip temperature over the threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21010801 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Incompatibility between the controllers has been found. |  |
| 1st Line Message |  | Incompatibility between the controllers has been found (Inconsistent Controller Board ID. Loc:0x#, Alt:0x#). |  |
| Old Message |  | Inconsistent Controller Board ID Number.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |  |
| Action |  | Contact technical support for changing the controller. |  |
| Event ID |  | 21010802 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Incompatibility between the controllers has been found. |  |
| 1st Line Message |  | Incompatibility between the controllers has been found (Inconsistent Controller Board Rev Number. Loc:0x0#, Alt:0x#). |  |
| Old Message |  | Inconsistent Controller Board Rev Number.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |  |
| Action |  | Contact technical support for changing the controller. |  |
| Event ID |  | 21010803 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Invalid hardware settings have been detected. |  |
| 1st Line Message |  |  |  |
| Old Message |  | Main Board HW Settings not found! |  |
| Cause |  |  |  |
| Action |  | Contact technical support for changing the controller. |  |
| Event ID |  | 21010804 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Incompatibility between the controllers has been found. |  |
| 1st Line Message |  | Incompatibility between the controllers has been found (Inconsistent HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message |  | Inconsistent Main Board HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |  |
| Action |  | Contact technical support for changing the controller. |  |
| Event ID |  | 21010805 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Incompatibility between the controllers has been found. |  |
| 1st Line Message |  | Incompatibility between the controllers has been found (Inconsistent Host Board 1 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message |  | Inconsistent Host Board 1 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |  |
| Action |  | Contact technical support for changing the controller. |  |
| Event ID |  | 21010806 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Incompatibility between the controllers has been found. |  |
| 1st Line Message |  | Incompatibility between the controllers has been found (Inconsistent Host Board 2 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message |  | Inconsistent Host Board 2 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |  |
| Action |  | Contact technical support for changing the controller. |  |
| Event ID |  | 21010807 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Incompatibility between the controllers has been found. |  |
| 1st Line Message |  | Incompatibility between the controllers has been found (Inconsistent DRAM Size. Loc: #MB, Alt:#MB). |  |
| Old Message |  | Inconsistent DRAM Size.(Loc:%dMB,Alt:%dMB) |  |
| Cause |  |  |  |
| Action |  | Contact technical support for changing the controller. |  |
| Event ID |  | 21010808 |  |
| Severity |  | Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Incompatibility between the controllers has been found. |  |
| 1st Line Message |  | Incompatibility between the controllers has been found (Inconsistent NVRAM size. Loc:0x#, Alt:0x#). |  |
| Old Message |  | Inconsistent NVRAM Size.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |  |
| Action |  | Contact technical support for changing the controller. |  |
| Event ID |  | 2107231A |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | The temperature sensor # of expansion enclosure # is not supported. |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Temp Sensor %d ERROR:Device Not Supported ! |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 2107231B |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | The temperature sensor # of expansion enclosure # is not installed. |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Temp Sensor %d ERROR:Device Not Installed ! |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 2107231C |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Unknown status of the temperature sensor # of expansion enclosure # has been detected. |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Temp Sensor %d ERROR:Device Unknown Status ! |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 2107231D |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | The temperature sensor # of expansion enclosure # is not available. |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Temp Sensor %d ERROR:Device Not Available ! |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 2107231E |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | Low temperature has been detected by sensor # for expansion enclosure #. |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Temp Sensor %d ERROR:Under Temperature Failure |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 2107231F |  |
| Severity |  | Error |  |
| Category-Module |  | HW - SENSOR |  |
| Prefix Parameter |  |  |  |
| Message |  | High temperature has been detected by sensor # for expansion enclosure #. |  |
| 1st Line Message |  |  |  |
| Old Message |  | SES(C%d I%d)Temp Sensor %d ERROR:Over Temperature Failure |  |
| Cause |  |  |  |
| Action |  | Check the fan status and speed. |  |
| Event ID |  | 2105221B |  |
| Severity |  | Error |  |
| Category-Module |  | HW - FAN |  |
| Prefix Parameter |  |  |  |
| Message |  | CPU FAN # Failure Detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | CPU FAN %d Failure Detected |  |
| Cause |  |  |  |
| Action |  | Contact support for changing the CPU fan. |  |
| Event ID |  | 220E0002 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pool creation failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Creation Failed |  |
| Cause |  | Specific pool element encountered errors during pool creation. |  |
| Action |  | 1. Check previous drive or pool element/logical drive event logs for cause of pool element issues. 2. Check if scale-out member pool elements in another subsystem encounter errors resulting from connection loss or member element failure. 3. Replace faulty drive or resolve connection issue, and recreate pool again. |  |
| Event ID |  | 220E0004 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pool expansion failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Expand Failed |  |
| Cause |  | Pool data migration across all elements failed due to pool element or logical drive issues. |  |
| Action |  | 1. Check previous drive or pool element/logical drive event logs for cause of pool element issues. 2. Check if scale-out member pool elements in another subsystem encounter errors resulting from connection loss or member element failure. 3. Replace faulty drive or resolve connection issue, and restart the expansion operation again. |  |
| Event ID |  | 220E000C |  |
| Severity |  | Error |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pool migration failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Migration Fail |  |
| Cause |  | The operation of pool data migration across all elements was failed result from the pool element or drive issues. |  |
| Action |  | 1. Check previous drive or pool element/logical drive event logs for cause of pool element issues. 2.Replace any faulty drives, or address element/logical drive errors with corresponding event actions. |  |
| Event ID |  | 220F000E |  |
| Severity |  | Error |  |
| Category-Module |  | FW - VV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Volume creation failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Creation Failed |  |
| Cause |  | Volume creation failed due to: 1. Internal pool I/O error caused by pool element error. 2. Insufficient available space in pool. |  |
| Action |  | 1. Check previous pool element/logical drive event logs for cause of pool issues. Remove faulty pool element and add again if necessary. 2. Check pool space threshold related events or capacity usage if pool space is insufficient. Expand storage or remove unused volume / snapshot. |  |
| Event ID |  | 220F0010 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - VV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Volume deletion failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Deleted Failed |  |
| Cause |  | Volume deletion failed due to: 1. Internal pool I/O error. 2. Failed to delete specific volume as it was still mapped to host. 3. The volume was the source volume of specific replication. 4. The volume still contains snapshot images. |  |
| Action |  | 1. Check previous pool element/logical drive event logs for cause of pool issues. Replace any faulty drives. 2. Unmap volume before trying to delete it. 3. Remove the replication pair first before volume deletion. 4. Delete snapshot images. |  |
| Event ID |  | 22120016 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair creation failed |  |
| 1st Line Message |  | Volume set: # |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Creation Failed in %s( VS SrcID:%08x%08x ) |  |
| Cause |  | Replication pair creation failed due to: 1. Source or target Partition/VV encountered errors because of logical drive error. 2. Network connection issues prevented remote replication pair creation. 3. Source volume pool capacity insufficient for pair crea |  |
| Action |  | 1. Check previous logical drive error event for cause of pool and volume issues. 2. Check and diagnose network status and bandwidth utilization for remote replication. 3. Fix capacity issue by pool expansion, remove unused volume or snapshot images to free more capacity for pair creation. 4. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 2212001D |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | ID:# |  |
| Message |  | Synchronous pair split after %d second timeout |  |
| 1st Line Message |  | Volume set: # |  |
| Old Message |  | Data in Volume ID:%08lx%08x of Pair ID:%08lx%08lx failed to sync %d Timeout, pair broken |  |
| Cause |  | Replication pair synchronization encountered errors because of network issues. |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Enhance network tolerance by raising the remote timeout threshold. |  |
| Event ID |  | 220B0029 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Logical volume creation failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( LV ID:%08lx%08lx ) Creation Failed |  |
| Cause |  | Specific logical drive encountered errors during logical volume creation operation. |  |
| Action |  | Check previous events if any Logical Drive issue caused Logical Volume creation failed. Follow the service guide to fixed Logical Drive issue then recreate the Logical Volume. |  |
| Event ID |  | 220B002B |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Logical volume expansion failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( LV ID:%08lx%08lx ) Expand Failed |  |
| Cause |  | Specific logical drive encountered errors during logical volume expansion operation. |  |
| Action |  | Check previous events if any Logical Drive issue caused Logical Volume expansion failed. Follow the service guide to fixed Logical Drive issue then restart Logical Volume expansion. |  |
| Event ID |  | 220C002E |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Partition creation failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( PT ID:%08lx%08lx ) Creation Failed |  |
| Cause |  | Partition creation failed due to: 1. Logical volume internal I/O error. 2. Insufficient available space in logical volume. |  |
| Action |  | 1. Check previous events if any logical drive issue caused Partition creation failed. Follow the service guide to fix the drive issue then recreate the Partition. 2. Expand LV then recreate the Partition. |  |
| Event ID |  | 220C0030 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Partition deletion failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( PT ID:%08lx%08lx ) Deleted Failed |  |
| Cause |  | Partition deletion failed due to: 1. Partition is still mapped to host. 2. Partition snapshot image exists. 3. Partition is used for replication pair. 4. Logical volume internal I/O error |  |
| Action |  | 1. Check previous events if any logical drive issue caused Partition deletion failed. Follow the service guide to fixed the drive issue then re-delete the Partition. 2. Umap the specific Partition. 3. Delete the snapshot images for the specific Partition. 4. Remove replication pair before Partition deletion. |  |
| Event ID |  | 22160038 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - VV/DATASERVICE |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Insufficient free space for data allocation |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) error due to insufficient space |  |
| Cause |  | Insufficient capacity for data service or I/O operation space reservation. |  |
| Action |  | 1. Check space threshold related events or capacity usage if free space was not enough. 2. Expand storage or remove unused volumes or snapshot images to free capacity. |  |
| Event ID |  | 2212003A |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) error, pair synchronization aborted |  |
| Cause |  | Replication pair synchronization encountered errors and aborted due to remote pair metadata synchronize error. |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Restart pair synchronization manually. |  |
| Event ID |  | 2212003B |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization failed and split |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) error, synchronization pair split |  |
| Cause |  | Replication pair synchronization encountered errors, and pair was split to lower the host I/O impact. The cause may be unstable connection between source and target units. |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. After the network issue fixed, re-sync the pair synchronization manually. |  |
| Event ID |  | 23100002 |  |
| Severity |  | Error |  |
| Category-Module |  | SW - SI |  |
| Prefix Parameter |  | NAME:# |  |
| Message |  | Snapshot image creation failed |  |
| 1st Line Message |  | Status: # |  |
| Old Message |  |  |  |
| Cause |  | Failed to create snapshot image due to: 1. Free space not enough for creating a new snapshot image. 2. Snapshot image count exceeds license limitation. 3. Logical drive I/O error and cannot complete snapshot image creation. |  |
| Action |  | 1. Expand storage capacity or remove unused partitions or snapshot images to free capacity. 2. Delete snapshot image to affordable count 3. Check logical drive event for possible cause of snapshot creation failure. |  |
| Event ID |  | 23100004 |  |
| Severity |  | Error |  |
| Category-Module |  | SW - SI |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Snapshot image deletion failed |  |
| 1st Line Message |  | Status: # |  |
| Old Message |  |  |  |
| Cause |  | Failed to delete snapshot image due to: 1. Internal pool I/O error caused by drive error. 2. The snapshot image was still mapped to host. 3. The snapshot image was the source volume of a specific replication. |  |
| Action |  | 1. Check previous volume event logs for cause of deletion failure. 2. Check and unmap snapshot images before deletion. 3. Remove the replication pair before snapshot deletion. |  |
| Event ID |  | 22130006 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space utilization exceeded threshold: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Utilization Exceeded - (%lu%%) |  |
| Cause |  | Capacity utilization exceeded a default or customized threshold percentage and posted an event notice with error severity. |  |
| Action |  | Check capacity and verify available space. Expand storage or remove unused volumes or snapshot images to free capacity. |  |
| Event ID |  | 22130007 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space utilization dropped below threshold: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Utilization Below - (%lu%%) |  |
| Cause |  | Capacity utilization below threshold setting. |  |
| Action |  | No required action if users expanded storage or deleted unused data manually to lower capacity utilization. |  |
| Event ID |  | 22130026 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space allocation exceeded #%, used:#% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Allocation Exceeded - (%lu %%)(Used=%lu %%) |  |
| Cause |  | Used data exceeded a specific percentage and posted an event notice with error severity. |  |
| Action |  | Check capacity usage and verify available space. Expand storage or remove unused volumes or snapshot images to free capacity or shrink the reservation size. |  |
| Event ID |  | 22130027 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space allocation was below #%, used:#% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Allocation Below - (%lu %%)(Used=%lu %%) |  |
| Cause |  | Used data below a specific percentage. |  |
| Action |  | No required action if users expanded storage or deleted unused data manually to lower capacity utilization. |  |
| Event ID |  | 2213003C |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Forced offline by errors |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) force offline |  |
| Cause |  | Logical volume or pool forced offline because of system metadata updating errors. |  |
| Action |  | 1. Check any logical drive event and correct it. 2. Check network connection connectivity. 3. Metadata write failed due to logical drive/pool element failure. Check previous event logs for cause. |  |
| Event ID |  | 2212003E |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy failed to start |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) initial copy has started failed |  |
| Cause |  | Initial copy from source to target Partition/VV for replication pair synchronization encountered errors. 1. Network unstable 2. Logical drive error |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Check status of logical volume or pool for source and target volume. If errors occurred, there will be correponding logical drive event and related action. Follow troubleshooting guide to fix logical drive event. 3. Recreate remote pair. |  |
| Event ID |  | 22120043 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy failed to resume |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) initial copy has resumed failed |  |
| Cause |  | Failed to resume a initial copy operation for replication pair synchronization due to following errors: 1. Network unstable 2. Logical Drive error |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Check status of logical volume or pool for source and target volume. If errors occurred, there will be correponding logical drive event and related action. Follow troubleshooting guide to fix logical drive event. 3. Resume inital copy of remote pair. |  |
| Event ID |  | 22120047 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization failed to start |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) pair synchronies has started failed |  |
| Cause |  | Failed to start pair synchronization due to following errors: 1. Network unstable 2. Logical Drive error |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Check status of logical volume or pool for source and target volume. If errors occurred, there will be correponding logical drive event and related action. Follow troubleshooting guide to fix logical drive event. 3. Resume pair synchronization. |  |
| Event ID |  | 2212004C |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization failed to resume |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) pair synchronies has resumed failed |  |
| Cause |  | Failed to resume paused pair synchronization due to following errors: 1. Network unstable 2. Logical Drive error |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Check status of logical volume or pool for source and target volume. If errors occurred, there will be correponding logical drive event and related action. Follow troubleshooting guide to fix logical drive event. 3. Resume pair synchronization of remote pair. |  |
| Event ID |  | 22140053 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Volume expansion failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Expand Failed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 22120054 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Bad block found in source volume, pair synchronization aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Bad block found in source volume, pair synchronization aborted |  |
| Cause |  | Replication pair synchronization aborted due to bad block(s) found in the source volume. |  |
| Action |  | 1. Delete the pair and fix the bad block issue in the source volume. 2. Recreate the pair manually. |  |
| Event ID |  | 220E0055 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Bad block found in deleted ME, migration aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Bad block found in deleted ME, migration aborted |  |
| Cause |  | Pool migration aborted due to bad block(s) found in deleted ME. |  |
| Action |  | 1. Fix the bad block issue in the ME. 2. Restart pool migration. |  |
| Event ID |  | 320B4503 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Volume ALERT: Fatal Fail |  |
| Cause |  | Critical issues in logical volume or its member logical drives. |  |
| Action |  | 1. Delete failed logical volume if no longer needed. 2. Restart the RAID unit for Logical Volume recovery. 3. If not, contact technical support for issue escalation and possible data recovery. |  |
| Event ID |  | 320B4504 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - LV |  |
| Prefix Parameter |  | LV:# |  |
| Message |  | Logical volume failed |  |
| 1st Line Message |  | Logical volume invalid |  |
| Old Message |  | ID:%.1x Logical Volume ALERT: Invalid Array |  |
| Cause |  | Critical issues in logical volume or its member logical drives. |  |
| Action |  | 1. Delete invalid logical volume if no longer needed. 2. Select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 320A4509 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Fatal Fail |  |
| Cause |  | 1. User removed the drive or drive tray was not properly inserted. 2. 2nd or more LD member drives failed when LD status is degrade or rebuilding. 3. Multiple LD member drive failure at the same time. |  |
| Action |  | 1. Power off RAID unit and insert missing LD member drive then power on. 2. Check event log to see which LD member drive is the last failed drive and reboor RAID subsystem with this drive inserted. If the drive is back to on-line, backup data immediately. |  |
| Event ID |  | 320A450A |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - LD |  |
| Prefix Parameter |  | LD:# |  |
| Message |  | Logical drive failed |  |
| 1st Line Message |  | Logical drive invalid |  |
| Old Message |  | ID:%.1x Logical Drive ALERT: Invalid Array |  |
| Cause |  | 1. Too many drive errors occurred at the same time. 2. Too many member drives failed or missing for automatic rebuild operation. |  |
| Action |  | 1. Replace faulty drives. 2. Delete and recreate logical drive if no longer needed. 3. Restore data from a backup source. 4. Contact technical support for issue escalation and possible data recovery. |  |
| Event ID |  | 32011101 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller failure or shutdown detected |  |
| 1st Line Message |  |  |  |
| Old Message |  | Redundant controller failure or shutdown was detected. Please contact technical support. |  |
| Cause |  | Redundant controller failure or shutdown detected. |  |
| Action |  | 1. Check the 7-segment value from the controller rear. 2. Run deassert failed controller or re-insert the failed controller to check if it becomes on-line. 3. If it's not on-line, contact technical support with controller 7-segment value. |  |
| Event ID |  | 32011102 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller has shut down |  |
| 1st Line Message |  |  |  |
| Old Message |  | Redundant controller has shut down. Please contact technical support. |  |
| Cause |  | Redundant controller has shut down due to a specific firmware issue. |  |
| Action |  | 1. Run deassert failed controller or re-insert the failed controller to initialize the controller. 2. Select System -> Export System Information in Management GUI. 3. Run "Memory dump tool" to collect dump. 4. Contact technical support wit |  |
| Event ID |  | 31011103 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | HW - CONTROLLER |  |
| Prefix Parameter |  |  |  |
| Message |  | Controller hardware failed (#) |  |
| 1st Line Message |  |  |  |
| Old Message |  | Redundant controller hardware failure was detected. Please replace the controller. |  |
| Cause |  | Redundant controller hardware failure detected. |  |
| Action |  | 1. Replace the controller. 2. If the issue was not fixed, select System -> Export System Information in Management GUI. 3. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 32130006 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space utilization exceeded threshold: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Utilization Exceeded - (%lu%%) |  |
| Cause |  | Capacity utilization exceeded a default or customized threshold percentage and posted an event notice with critical error severity. |  |
| Action |  | Check capacity usage for available space. Expand storage or remove unused volumes or snapshot images to free capacity |  |
| Event ID |  | 32130007 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space utilization dropped below threshold: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Utilization Below - (%lu%%) |  |
| Cause |  | Capacity utilization below threshold setting. |  |
| Action |  | No required action if users expanded storage or deleted unused data manually to lower capacity utilization. |  |
| Event ID |  | 32130026 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space allocation exceeded #%, used: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Allocation Exceeded - (%lu %%)(Used=%lu %%) |  |
| Cause |  | Used data exceeded specific percentage and posted an event notice with critical error severity. |  |
| Action |  | Check capacity usage and verify available space. Expand storage or remove unused volumes or snapshot images to free capacity or shrink the reservation size. |  |
| Event ID |  | 32130027 |  |
| Severity |  | Critical Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Space allocation below #%, used: #% |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Space Allocation Below - (%lu %%)(Used=%lu %%) |  |
| Cause |  | Used data is freed and it is below specific percentage. |  |
| Action |  | No required action if users expanded storage or deleted unused data manually to lower capacity utilization. |  |
| Event ID |  | 2213003C |  |
| Severity |  | Error |  |
| Category-Module |  | FW - LV/POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | The pool has been foreced offline because error detected. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) force offline |  |
| Cause |  | Logical volume or pool forced offline because of system metadata updating errors. |  |
| Action |  | 1. Check any logical drive event and correct it. 2. Check network connection connectivity. 3. Metadata write failed due to logical drive/pool element failure. Check previous event logs for cause. |  |
| Event ID |  | 0212003D |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy started |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 0212003E |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Failed to start the Initial copy. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) initial copy has started failed |  |
| Cause |  | Initial copy from source to target Partition/VV for replication pair synchronization encountered errors. 1. Network unstable 2. Logical drive error |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Check status of logical volume or pool for source and target volume. If errors occurred, there will be correponding logical drive event and related action. Follow troubleshooting guide to fix logical drive event. 3. Recreate remote pair. |  |
| Event ID |  | 0212003F |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy has completed. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%09lx ) initial copy complete size %d%s transfer rate %d%s |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120040 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy has continued |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%10lx ) initial copy has continued |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 12120041 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy has been stopped |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%11lx ) initial copy has stopped, event ID: %d |  |
| Cause |  | Initial copy from source to target volume for replication pair synchronization stopped due to: 1. Target Partition/VV or its LV/Pool ran out of space. 2. Connection between source and target units is unstable. |  |
| Action |  | 1. Expand storage or remove unused Partition/VV or snapshot images to free capacity. 2. Check and diagnose network status and bandwidth utilization for remote replication. 3. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 02120042 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Initial copy has been resumed. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) initial copy has resumed success |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 22120043 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Failed to resume the initial copy |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) initial copy has resumed failed |  |
| Cause |  | Failed to resume a initial copy operation for replication pair synchronization due to following errors: 1. Network unstable 2. Logical Drive error |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Check status of logical volume or pool for source and target volume. If errors occurred, there will be correponding logical drive event and related action. Follow troubleshooting guide to fix logical drive event. 3. Resume inital copy of remote pair. |  |
| Event ID |  | 02120044 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair recovered |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%13lx ) has recovered |  |
| Cause |  | 1. Pair recovered because the network connection was recovered. 2. Reboot the controller of the source volume and the pair will be recovered automatically. |  |
| Action |  |  |  |
| Event ID |  | 12120045 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair broken |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%14lx ) has broken, event ID: %d |  |
| Cause |  | Remote replication was broken due to network issues. |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. Remote replication would be recovered automatically after network issue is fixed. 2. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 02120046 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization has started. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) pair synchronies has started success |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 22120047 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Failed to start the pair synchronization. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) pair synchronies has started failed |  |
| Cause |  | Failed to start pair synchronization due to following errors: 1. Network unstable 2. Logical Drive error |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Check status of logical volume or pool for source and target volume. If errors occurred, there will be correponding logical drive event and related action. Follow troubleshooting guide to fix logical drive event. 3. Resume pair synchronization. |  |
| Event ID |  | 02120048 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization completed. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%09lx ) pair synchronies complete size %d%s transfer rate %d%s |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120049 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization has continued. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%10lx ) pair synchronies has continued |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1212004A |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization has been stopped. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%11lx ) pair synchronies has stopped, event ID: %d |  |
| Cause |  | Replication pair synchronization stopped as a result of: 1. Target Partition/VV or its LV/Pool ran out of space. 2. Connection between source and target units is unstable. |  |
| Action |  | 1. Expand storage or remove unused volumes or snapshot images to free capacity for future use. 2. Check and diagnose network status and bandwidth utilization for remote replication. 3. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID |  | 0212004B |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Pair synchronization has been resumed. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) pair synchronies has resumed success |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 2212004C |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Failed to resume the pair synchronization. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) pair synchronies has resumed failed |  |
| Cause |  | Failed to resume paused pair synchronization due to following errors: 1. Network unstable 2. Logical Drive error |  |
| Action |  | 1. Check and diagnose network status and bandwidth utilization for remote replication. 2. Check status of logical volume or pool for source and target volume. If errors occurred, there will be correponding logical drive event and related action. Follow troubleshooting guide to fix logical drive event. 3. Resume pair synchronization of remote pair. |  |
| Event ID |  | 1212004D |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Target volume full |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%12lx ) target volume full |  |
| Cause |  | The target volume for replication pair is full and failed to allocate more space. |  |
| Action |  | Expand storage or remove unused volumes or snapshot images to free capacity for future use. |  |
| Event ID |  | 0212004E |  |
| Severity |  | Information |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  |  |  |
| Message |  | Pair synchronization is in progress. |  |
| 1st Line Message |  |  |  |
| Old Message |  | (%d.%d%s, %dms, %d.%d%%)(%d.%d%s, %dms, %d.%d%%)(%d.%d%s, %dms, %d.%d%%)(%d.%d%s, %dms, %d.%d%%) |  |
| Cause |  | Event used to describe data transfer rate, latency, and packet loss information for data replication during pair synchronization of specific period. |  |
| Action |  | Expand storage or remove unused volumes or snapshot images to free capacity for future use. |  |
| Event ID |  | 0212004F |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Tier migration has been started. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Tier Migration Start |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120050 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Tier migration has completed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Tier Migration Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120051 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Tier migration has been aborted. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Tier Migration Abort |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 02120052 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Volume expansion has completed. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Expand Complete |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 22120053 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Volume expansion failed |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( SI ID:%08lx%08lx ) Expand Failed |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 22120054 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - PAIR |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Bad block found in source volume, pair synchronization aborted. |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( VP ID:%08lx%08lx ) Bad block found in source volume, pair synchronization aborted |  |
| Cause |  | Replication pair synchronization aborted due to bad block(s) found in the source volume. |  |
| Action |  | 1. Delete the pair and fix the bad block issue in the source volume. 2. Recreate the pair manually. |  |
| Event ID |  | 220E0055 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | Bad block found in deleted ME, migration aborted |  |
| 1st Line Message |  |  |  |
| Old Message |  | %s( DC ID:%08lx%08lx ) Bad block found in deleted ME, migration abortedsynchronization aborted |  |
| Cause |  | Pool migration aborted due to bad block(s) found in deleted ME. |  |
| Action |  | 1. Fix the bad block issue in the ME. 2. Restart pool migration. |  |
| Event ID |  | 02120080 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - SI |  |
| Prefix Parameter |  |  |  |
| Message |  | The snapshot images has been backed up to cloud. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 120E0080 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool cannot connect to cloud, because the network error. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Network problem |  |
| Action |  | check network setting |  |
| Event ID |  | 120E0081 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool cannot connect to cloud, because authentication failed. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Authentication Key Incorrect |  |
| Action |  | check authentication setting |  |
| Event ID |  | 120E0082 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool cannot connect to cloud, because the bucket not exist. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Bucket is removed by unknown reason |  |
| Action |  | if the bucket is already removed, can not restore anymore |  |
| Event ID |  | 120E0083 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool cannot connect to cloud, because failed create bucket.. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Provider not allow to create bucket |  |
| Action |  | check provider |  |
| Event ID |  | 120E0084 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool cannot connect to cloud. The bucket has been used. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | The Bucket is using by another Pool |  |
| Action |  | one bucket only can be used by one pool at the same time |  |
| Event ID |  | 120E0085 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | TThe pool cannot connect to cloud. Wrong encryption key. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Encrpytion setting incorrect |  |
| Action |  | check encryption setting |  |
| Event ID |  | 120E0086 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool has failed to upload data to cloud. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Provider not allow to put object |  |
| Action |  | check provider |  |
| Event ID |  | 120E0087 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool cannot connect to cloud. No channel for iSCSI device. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | All channels have been assigned to file service. |  |
| Action |  | Assign at least one channel to RAID |  |
| Event ID |  | 120E0088 |  |
| Severity |  | Information |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The pool has been deleted. The cloud storage is not deleted. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Can not connect to provider |  |
| Action |  | User can delete the bucket from provider later |  |
| Event ID |  | 220E0089 |  |
| Severity |  | Error |  |
| Category-Module |  | FW - POOL |  |
| Prefix Parameter |  |  |  |
| Message |  | The data of the pool saved in the bucket is corrupted. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Should not happened |  |
| Action |  | Should not happened |  |
| Event ID |  | 1214008A |  |
| Severity |  | Warning |  |
| Category-Module |  | FW - VV/PARTITION |  |
| Prefix Parameter |  | NAME:# ID:# |  |
| Message |  | The system has been unable to satisfy QoS policy for 15 min. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | The system has been unable to satisfy the QoS policy for 15 minutes. |  |
| Action |  | You may want to adjust the policy configurations. |  |
| Event ID |  | 1318008B |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot license expired. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318008C |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Failed to take snapshot of pair target volume. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318008D |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | The exception of the snapshot schedule prune rule occurred. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318008E |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Maximum snapshot amount of the volume reached. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318008F |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Maximum snapshot amount of the volume reached. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180090 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched, flush agents cannot be connected. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180091 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched. Host volume disk can not be locked. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180092 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched. Host cache flush failed. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180093 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched. Host volume disk has been locked. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180094 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched. Host cache data flush has timed out. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180095 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched. Host flush database cache failed. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180096 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched. File system does not mount volume. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180097 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched. Volume has not been mapped to host. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180098 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | snapshot schedule launched. Exception has occurred. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 13180099 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | snapshot schedule failed. The volume has not been mapped. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318009A |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | The snapshot schedule has failed to unlock the host volume disk. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318009B |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | The snapshot schedule has failed to resume the host database. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318009C |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule failed. Flush settings have not configured. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318009D |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched but failed to backup to cloud. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318009E |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched but exception has occurred. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 1318009F |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Snapshot schedule launched, but device can not be connected. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A0 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | The storage tiering license of the device has expired. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A1 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Tier migration schedule failed. Previous process is processing. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A2 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Tier migration schedule failed. Specified volumes are not found. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A3 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Tier migration schedule rejected. The storage has one tier. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A4 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Tier migration schedule failed. Exception has occurred. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A5 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Volume replication schedule launched, but target volume mapped. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A6 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | The volume mirror license of the device has expired. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A7 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Volume mirror schedule failed. Exception(s) have occurred. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A8 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Volume mirror schedule failed. Exception has occurred. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800A9 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Volume mirror schedule launched. Source volume is not mapped. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800AA |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Volume mirror schedule launched. Flush agent(s) not connected. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800AB |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | The volume copy license of the device has expired. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800AC |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Volume copy schedule failed. Exception has occurred. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800AD |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SETTING |  |
| Prefix Parameter |  |  |  |
| Message |  | The SMTP server has not been configured yet. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 121800AE |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-VV/PARTITION |  |
| Prefix Parameter |  |  |  |
| Message |  | Sync cloud storage stopped, due to insufficient pool space. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 021600AF |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-VV/DATASERVICE |  |
| Prefix Parameter |  |  |  |
| Message |  | Deduplication throughput optimization process has started. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 021600B0 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-VV/DATASERVICE |  |
| Prefix Parameter |  |  |  |
| Message |  | Deduplication throughput optimization process has stopped. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 021600B1 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-VV/DATASERVICE |  |
| Prefix Parameter |  |  |  |
| Message |  | The restoration of cloud-integrated volume has been terminated. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800B2 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | The maximum number of snapshot images has been reached. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 231800B3 |  |
| Severity |  | Error |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | The snapshot image container is abnormal. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Failed to create SI because BAS mount fail |  |
| Action |  | Call support |  |
| Event ID |  | 031800B4 |  |
| Severity |  | Information |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Service manager has submitted device status report. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800B5 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-SCHEDULE |  |
| Prefix Parameter |  |  |  |
| Message |  | Serivce failed to submit device status report. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 131800B6 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | Network error. Bucket entry cannot connect to the cloud. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Network problem |  |
| Action |  | check network setting |  |
| Event ID |  | 131800B7 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | Authentication failed. Bucket entry cannot connect to cloud. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Authentication Key Incorrect |  |
| Action |  | check authentication setting |  |
| Event ID |  | 131800B8 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | Cloud bucket not found. Bucket entry cannot connect to cloud. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Bucket is removed by unknown reason |  |
| Action |  | if the bucket is already removed, cannot restore anymore |  |
| Event ID |  | 131800BA |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | Bucketentry cannot connect to the used cloud bucket. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | The Bucket is using by another Pool |  |
| Action |  | one bucket only can be used by one pool at the same time |  |
| Event ID |  | 131800BB |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | Bucket entry cannot connect to cloud with wrong encryption key. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Encrpytion setting incorrect |  |
| Action |  | check encryption setting |  |
| Event ID |  | 131800BC |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | Failed to upload data via the bucket entry. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Provider not allow to put object |  |
| Action |  | check provider |  |
| Event ID |  | 131800BD |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | No available iSCSI channels can be used to connect to cloud. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | All channels have been assigned to file service. |  |
| Action |  | Assign at least one channel to RAID |  |
| Event ID |  | 021A00BE |  |
| Severity |  | Information |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | The bucket entry is deleted. Cloud bucket remains available. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Can not connect to provider |  |
| Action |  | User can delete the bucket from provider later |  |
| Event ID |  | 221A00BF |  |
| Severity |  | Error |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | The data sent via the bucketentry are corrupted in cloud bucket. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Should not happened |  |
| Action |  | hould not happened |  |
| Event ID |  | 121A00C0 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | The data sent via the bucketentry are corrupted in cloud bucket. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Should not happened |  |
| Action |  | hould not happened |  |
| Event ID |  | 121A00C1 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-BUCKET |  |
| Prefix Parameter |  |  |  |
| Message |  | Authentication failed. The cloud entry cannot connect to cloud. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Authentication Key Incorrect |  |
| Action |  | check authentication setting |  |
| Event ID |  | 121400C2 |  |
| Severity |  | Warning |  |
| Category-Module |  | FW-VV/PARTITION |  |
| Prefix Parameter |  |  |  |
| Message |  | Sync cloud storage stopped, due to insufficient pool space. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 031B00C3 |  |
| Severity |  | Information |  |
| Category-Module |  | SW-RESTful |  |
| Prefix Parameter |  |  |  |
| Message |  | The RESTful API is enabled. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | RESTful API enabled |  |
| Action |  | Do nothing |  |
| Event ID |  | 031B00C4 |  |
| Severity |  | Information |  |
| Category-Module |  | SW-RESTful |  |
| Prefix Parameter |  |  |  |
| Message |  | The RESTful API is disabled. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | RESTful API enabled |  |
| Action |  | Do nothing |  |
| Event ID |  | 031B00C5 |  |
| Severity |  | Information |  |
| Category-Module |  | SW-RESTful |  |
| Prefix Parameter |  |  |  |
| Message |  | The RESTful API client authenticated and received certificate. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | Client has been authenticated. |  |
| Action |  | Do nothing |  |
| Event ID |  | 031B00C6 |  |
| Severity |  | Information |  |
| Category-Module |  | SW-RESTful |  |
| Prefix Parameter |  |  |  |
| Message |  | The RESTful API client failed the authentication. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | The RESTful API client failed the authentication. |  |
| Action |  | Check the username and password; try again later. |  |
| Event ID |  | 031B00C7 |  |
| Severity |  | Information |  |
| Category-Module |  | SW-RESTful |  |
| Prefix Parameter |  |  |  |
| Message |  | The RESTful API client has an outdated certificate. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  | The RESTful API client failed the authentication. |  |
| Action |  | Try again later. |  |
| Event ID |  | 031B00C8 |  |
| Severity |  | Warning |  |
| Category-Module |  | SW-PAIR |  |
| Prefix Parameter |  |  |  |
| Message |  | No available network channel to run remote volume replication. |  |
| 1st Line Message |  |  |  |
| Old Message |  |  |  |
| Cause |  |  |  |
| Action |  |  |  |
| Event ID |  | 21072321 |  |
| Severity |  | Error |  |
| Category-Module |  | HW-SENSOR |  |
| Prefix Parameter |  | CPU # low temperature detected. |  |
| Message |  |  |  |
| 1st Line Message |  | Temperature: #(C). |  |
| Old Message |  | Peripheral Device ALERT: CPU Cold Temperature Detected(%d.0(C)). |  |
| Cause |  | Controller CPU temperature below threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 21072322 |  |
| Severity |  | Error |  |
| Category-Module |  | HW-SENSOR |  |
| Prefix Parameter |  | CPU # high temperature detected. |  |
| Message |  |  |  |
| 1st Line Message |  | Temperature: #(C). |  |
| Old Message |  | Peripheral Device ALERT: CPU Cold Temperature Detected(%d.0(C)). |  |
| Cause |  | Controller CPU temperature below threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID |  | 0107A321 |  |
| Severity |  | Information |  |
| Category-Module |  | HW-SENSOR |  |
| Prefix Parameter |  | CPU # temperature back to normal. |  |
| Message |  |  |  |
| 1st Line Message |  | Low temperature raised to #(C). |  |
| Old Message |  | NOTICE:CPU Cold Temperature Back To Normal(%d.0(C)). |  |
| Cause |  | CPU low temperature back within normal levels. |  |
| Action |  |  |  |
| Event ID |  | 0107A322 |  |
| Severity |  | Information |  |
| Category-Module |  | HW-SENSOR |  |
| Prefix Parameter |  | CPU # temperature back to normal. |  |
| Message |  |  |  |
| 1st Line Message |  | High temperature down to #(C). |  |
| Old Message |  | NOTICE:CPU Hot Temperature Back To Normal(%d.0(C)). |  |
| Cause |  | CPU low temperature back within normal levels. |  |
| Action |  |  |  |
| Event ID |  | 21072326 |  |
| Severity |  | Error |  |
| Category-Module |  | HW-SENSOR |  |
| Prefix Parameter |  | I/O module temperature: #(C). |  |
| Message |  | I/O module # low temperature detected. |  |
| 1st Line Message |  | I/O module temperature: #(C). |  |
| Old Message |  | Peripheral Device ALERT: I/O module Cold Temperature Detected(%d.0(C)). |  |
| Cause |  | Controller I/O module temperature below threshold. |  |
| Action |  | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID | 21072327 |  |
| Severity | Error |  |
| Category-Module | HW-SENSOR |  |
| Prefix Parameter | I/O module temperature: #(C). |  |
| Message |  |  |
| 1st Line Message | I/O module # high temperature detected. |  |
| Old Message | Peripheral Device ALERT: I/O module Hot Temperature Detected(%d.0(C)). |  |
| Cause | Controller I/O module temperature over threshold. |  |
| Action | 1. Check ambient temperature. 2. Check fan module. If there are errors, check event logs for generated event notices that offer corresponding actions to address the issue. |  |
| Event ID | 0107A314 |  |
| Severity | Information |  |
| Category-Module | HW-SENSOR |  |
| Prefix Parameter | I/O module # temperature back to normal . |  |
| Message |  |  |
| 1st Line Message | Low temperature raised to #(C). |  |
| Old Message | NOTICE: I/O module Cold Temperature Back To Normal(%d.0(C)). |  |
| Cause | I/O module low temperature back within normal levels. |  |
| Action |  |  |
| Event ID | 0107A315 |  |
| Severity | Information |  |
| Category-Module | HW-SENSOR |  |
| Prefix Parameter | I/O module # temperature back to normal . |  |
| Message |  |  |
| 1st Line Message | High temperature down to #(C). |  |
| Old Message | NOTICE: I/O module Hot Temperature Back To Normal(%d.0(C)). |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 2101080A |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter | Incompatibility between the controllers has been found. |  |
| Message |  |  |
| 1st Line Message | Incompatibility between the controllers has been found (Inconsistent Host Board 3 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message | Inconsistent Host Board 3 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |
| Action | Contact technical support for changing the controller |  |
| Event ID | 2101080B |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter | Incompatibility between the controllers has been found. |  |
| Message |  |  |
| 1st Line Message | Incompatibility between the controllers has been found (Inconsistent Host Board 4 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message | Inconsistent Host Board 4 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |
| Action | Contact technical support for changing the controller |  |
| Event ID | 2101080C |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter | Incompatibility between the controllers has been found. |  |
| Message |  |  |
| 1st Line Message | Incompatibility between the controllers has been found (Inconsistent Host Board 5 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message | Inconsistent Host Board 5 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |
| Action | Contact technical support for changing the controller |  |
| Event ID | 2101080D |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter | Incompatibility between the controllers has been found. |  |
| Message |  |  |
| 1st Line Message | Incompatibility between the controllers has been found (Inconsistent Host Board 6 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message | Inconsistent Host Board 6 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |
| Action | Contact technical support for changing the controller |  |
| Event ID | 2101080E |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter | Incompatibility between the controllers has been found. |  |
| Message |  |  |
| 1st Line Message | Incompatibility between the controllers has been found (Inconsistent Host Board 7 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message | Inconsistent Host Board 7 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |
| Action | Contact technical support for changing the controller |  |
| Event ID | 2101080F |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter | Incompatibility between the controllers has been found. |  |
| Message |  |  |
| 1st Line Message | Incompatibility between the controllers has been found (Inconsistent Host Board 8 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message | Inconsistent Host Board 8 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |
| Action | Contact technical support for changing the controller |  |
| Event ID | 21010810 |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter | Incompatibility between the controllers has been found. |  |
| Message |  |  |
| 1st Line Message | Incompatibility between the controllers has been found (Inconsistent Host Board 9 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message | Inconsistent Host Board 9 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |
| Action | Contact technical support for changing the controller |  |
| Event ID | 21010811 |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter | Incompatibility between the controllers has been found. |  |
| Message |  |  |
| 1st Line Message | Incompatibility between the controllers has been found (Inconsistent Host Board 10 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message | Inconsistent Host Board 10 HW Settings ID.(Loc:0x%x,Alt:0x%x) |  |
| Cause |  |  |
| Action | Contact technical support for changing the controller |  |
| Event ID | 31084312 |  |
| Severity | Critical Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter | At least one or more SSDs that the remaining life are less than # month(s), have been detected. |  |
| Message |  |  |
| 1st Line Message | Incompatibility between the controllers has been found (Inconsistent Host Board 10 HW setting ID. Loc:0x#, Alt:0x#). |  |
| Old Message | ALERT: The SSD(s), which remaining life are less than threshold, have been detected. |  |
| Cause |  |  |
| Action | Copy and replace the SSD which remaining life is less than the threshold |  |
| Event ID | 12018901 |  |
| Severity | Warning |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The password of the device has been reset. |  |
| 1st Line Message |  |  |
| Old Message | The device password has been reset to default. |  |
| Cause | Controller password has been reset by default button. |  |
| Action | do nothing or change the device password |  |
| Event ID | 020A8B41 |  |
| Severity | information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Media scan started |  |
| 1st Line Message |  |  |
| Old Message | NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Starting Media Scan |  |
| Cause | Media scan on a disk drive started. |  |
| Action |  |  |
| Event ID | 020A8B81 |  |
| Severity | information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Media scan started |  |
| 1st Line Message |  |  |
| Old Message | NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Starting Media Scan |  |
| Cause | Media scan on a disk drive started. |  |
| Action |  |  |
| Event ID | 020A8B43 |  |
| Severity | information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Media scan started |  |
| 1st Line Message |  |  |
| Old Message | NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Starting Media Scan |  |
| Cause | Media scan on a disk drive started. |  |
| Action |  |  |
| Event ID | 020A8B83 |  |
| Severity | information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Media scan started |  |
| 1st Line Message |  |  |
| Old Message | NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Starting Media Scan |  |
| Cause | Media scan on a disk drive started. |  |
| Action |  |  |
| Event ID | 02088C41 |  |
| Severity | information |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Media scan completed |  |
| 1st Line Message |  |  |
| Old Message | NOTICE: Media Scan of CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Completed |  |
| Cause | 'Media scan on a disk drive completed. |  |
| Action |  |  |
| Event ID | 02088C81 |  |
| Severity | information |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Media scan completed |  |
| 1st Line Message |  |  |
| Old Message | NOTICE: Media Scan of CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Completed |  |
| Cause | 'Media scan on a disk drive completed. |  |
| Action |  |  |
| Event ID | 020A8C43 |  |
| Severity | information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | The media scan has been completed. Recovered error: #, Unrecovered error: # |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x NOTICE: Media Scan of CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Completed (%d,%d) |  |
| Cause | Media scan on a disk drive completed. |  |
| Action |  |  |
| Event ID | 020A8C83 |  |
| Severity | information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | The media scan has been completed. Recovered error: #, Unrecovered error: # |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x NOTICE: Media Scan of CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Completed (%d,%d) |  |
| Cause | Media scan on a disk drive completed. |  |
| Action |  |  |
| Event ID | 22080751 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | The media scan has been aborted. |  |
| 1st Line Message |  |  |
| Old Message | NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Media Scan Aborted |  |
| Cause | Media scan aborted due to: 1. Operation cancelled manually by user 2. Drive failure (global and enclosure spare only) |  |
| Action | 1. If operation cancelled by users, simply restart it. 2. If interrupted by drive error, check previous drive event logs for cause of interruption, and replace faulty drive. |  |
| Event ID | 22080791 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | The media scan has been aborted. |  |
| 1st Line Message |  |  |
| Old Message | NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Media Scan Aborted |  |
| Cause | Media scan aborted due to: 1. Operation cancelled manually by user 2. Drive failure (global and enclosure spare only) |  |
| Action | 1. If operation cancelled by users, simply restart it. 2. If interrupted by drive error, check previous drive event logs for cause of interruption, and replace faulty drive. |  |
| Event ID | 220A0747 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter | Drive unrecovered media error - # |  |
| Message | Unrecovered media error detected |  |
| 1st Line Message |  |  |
| Old Message | CHL:.1d ID:%.1d (JBODId:%d SlotNum:%d) WARN: Media Scan Unrecoverable Error Encountered - %lx%08lx |  |
| Cause | Drive media error was found and failed to recover. |  |
| Action | 1. Replace the faulty drive. 2. Restart the drive media scan operation. |  |
| Event ID | 220A0787 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter | Drive unrecovered media error - # |  |
| Message | Unrecovered media error detected |  |
| 1st Line Message |  |  |
| Old Message | CHL:.1d ID:%.1d (JBODId:%d SlotNum:%d) WARN: Media Scan Unrecoverable Error Encountered - %lx%08lx |  |
| Cause | Drive media error was found and failed to recover. |  |
| Action | 1. Replace the faulty drive. 2. Restart the drive media scan operation. |  |
| Event ID | 22080748 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Media scan stopped |  |
| 1st Line Message | Media scan failed |  |
| Old Message | WARN: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Media Scan Failed |  |
| Cause | Drive media error found and media scan operation failed to continue. |  |
| Action | 1. Replace faulty drive. 2. Restart media scan. |  |
| Event ID | 22080788 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Media scan stopped |  |
| 1st Line Message | Media scan failed |  |
| Old Message | WARN: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Media Scan Failed |  |
| Cause | Drive media error found and media scan operation failed to continue. |  |
| Action | 1. Replace faulty drive. 2. Restart media scan. |  |
| Event ID | 220A0749 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Media scan stopped |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x WARN: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Media Scan Failed |  |
| Cause | Logical drive media scan operation stopped because a specific drive error was found. |  |
| Action | 1. Replace the faulty drive. 2. Restart logical drive media scan. |  |
| Event ID | 220A0789 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Media scan stopped |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x WARN: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Media Scan Failed |  |
| Cause | Logical drive media scan operation stopped because a specific drive error was found. |  |
| Action | 1. Replace the faulty drive. 2. Restart logical drive media scan. |  |
| Event ID | 02081342 |  |
| Severity | Information |  |
| Category-Module | FW-DRVIE |  |
| Prefix Parameter |  |  |
| Message |  |  |
| 1st Line Message | Drive media error - # (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Media Error Encountered - 0x%s (%02x/%02x/%02x) |  |
| Cause | 'Media errors found in disk drives. |  |
| Action | If this message occurs frequently, replace the disk drive. |  |
| Event ID | 02081382 |  |
| Severity | Information |  |
| Category-Module | FW-DRVIE |  |
| Prefix Parameter |  |  |
| Message |  |  |
| 1st Line Message | Drive media error - # (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Media Error Encountered - 0x%s (%02x/%02x/%02x) |  |
| Cause | 'Media errors found in disk drives. |  |
| Action | If this message occurs frequently, replace the disk drive. |  |
| Event ID | 02081341 |  |
| Severity | Information |  |
| Category-Module | FW-DRVIE |  |
| Prefix Parameter |  |  |
| Message |  |  |
| 1st Line Message | Drive media error - # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Media Error Encountered - 0x%s (%02x/%02x/%02x) |  |
| Cause | 'Media errors found in disk drives. |  |
| Action | If this message occurs frequently, replace the disk drive. |  |
| Event ID | 02081381 |  |
| Severity | Information |  |
| Category-Module | FW-DRVIE |  |
| Prefix Parameter |  |  |
| Message |  |  |
| 1st Line Message | Drive media error - # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Media Error Encountered - 0x%s (%02x/%02x/%02x) |  |
| Cause | 'Media errors found in disk drives. |  |
| Action | If this message occurs frequently, replace the disk drive. |  |
| Event ID | 01081942 |  |
| Severity | Information |  |
| Category-Module | HW-DRVIE |  |
| Prefix Parameter |  |  |
| Message |  |  |
| 1st Line Message | Drive error recovered - # (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Drive Recovered Error - 0x%s (%02x/%02x/%02x) |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 01081982 |  |
| Severity | Information |  |
| Category-Module | HW-DRVIE |  |
| Prefix Parameter |  |  |
| Message |  |  |
| 1st Line Message | Drive error recovered - # (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Drive Recovered Error - 0x%s (%02x/%02x/%02x) |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 220A0141 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive missing |  |
| 1st Line Message |  |  |
| Old Message | ID:NA Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Missing |  |
| Cause | Specific member drive of logical drive missing. 1. User removed the drive or drive tray not properly inserted. 2. Drive failed or contains errors. 3. Backplane connection issue. |  |
| Action | 1. Check or reinstall the missing member drive. 2. Replace the failed drive and engage logical drive rebuild operation manually or automatically to recover logical drive. 3. Replace the backplane. |  |
| Event ID | 220A0181 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive missing |  |
| 1st Line Message |  |  |
| Old Message | ID:NA Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Missing |  |
| Cause | Specific member drive of logical drive missing. 1. User removed the drive or drive tray not properly inserted. 2. Drive failed or contains errors. 3. Backplane connection issue. |  |
| Action | 1. Check or reinstall the missing member drive. 2. Replace the failed drive and engage logical drive rebuild operation manually or automatically to recover logical drive. 3. Replace the backplane. |  |
| Event ID | 220A0142 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive failed |  |
| 1st Line Message |  |  |
| Old Message | ID:NA Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Failure |  |
| Cause | Specific member drive of logical drive failed. 1. User removed the drive. 2. Drive failed. 3. Backplane connection issue. |  |
| Action | 1. Insert the drive back. 2. Replace the backplane. |  |
| Event ID | 220A0182 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive failed |  |
| 1st Line Message |  |  |
| Old Message | ID:NA Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Failure |  |
| Cause | Specific member drive of logical drive failed. 1. User removed the drive. 2. Drive failed. 3. Backplane connection issue. |  |
| Action | 1. Insert the drive back. 2. Replace the backplane. |  |
| Event ID | 220A0143 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive removed |  |
| 1st Line Message |  |  |
| Old Message | ID:NA Logical Drive NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Removed |  |
| Cause | Specific drive removal detected. |  |
| Action | Check and reinstall the removed drive. |  |

| Event ID | 220A0183 |
| --- | --- |
| Severity | Error |
| Category-Module | FW-LD |
| Prefix Parameter |  |
| Message | Drive removed |
| 1st Line Message |  |
| Old Message | ID:NA Logical Drive NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Removed |
| Cause | Specific drive removal detected. |
| Action | Check and reinstall the removed drive. |
| Event ID | 220A0144 |
| Severity | Error |
| Category-Module | FW-LD |
| Prefix Parameter |  |
| Message | A second or third LD member drive failed. |
| 1st Line Message |  |
| Old Message | ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Missing |
| Cause | Possible cause: 1. User removed the drive or the drive tray was not properly inserted. 2. The drive failed. 3. Backplane connection issue. |
| Action | 1. Insert the drive back and rebuild the Logical Drive if needed. 2. Replace the failed drive and rebuild the logical drive. 3. Replace the backplane. |
| Event ID | 220A0184 |
| Severity | Error |
| Category-Module | FW-LD |
| Prefix Parameter |  |
| Message | A second or third LD member drive failed. |
| 1st Line Message |  |
| Old Message | ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Missing |
| Cause | Possible cause: 1. User removed the drive or the drive tray was not properly inserted. 2. The drive failed. 3. Backplane connection issue. |
| Action | 1. Insert the drive back and rebuild the Logical Drive if needed. 2. Replace the failed drive and rebuild the logical drive. 3. Replace the backplane. |
| Event ID | 220A0145 |
| Severity | Error |
| Category-Module | FW-LD |
| Prefix Parameter |  |
| Message | The first LD member drive failed. |
| 1st Line Message |  |
| Old Message | ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Failure |
| Cause | Possible cause: 1. User removed the drive or the drive tray was not properly inserted. 2. The drive failed. 3. Backplane connection issue. |
| Action | 1. Insert the drive back and rebuild the Logical Drive if needed. 2. Replace the failed drive and rebuild the logical drive. 3. Replace the backplane. |
| Event ID | 220A0185 |
| Severity | Error |
| Category-Module | FW-LD |
| Prefix Parameter |  |
| Message | The first LD member drive failed. |
| 1st Line Message |  |
| Old Message | ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Failure |
| Cause | Possible cause: 1. User removed the drive or the drive tray was not properly inserted. 2. The drive failed. 3. Backplane connection issue. |
| Action | 1. Insert the drive back and rebuild the Logical Drive if needed. 2. Replace the failed drive and rebuild the logical drive. 3. Replace the backplane. |
| Event ID | 220A0146 |
| Severity | Error |
| Category-Module | FW-LD |
| Prefix Parameter |  |
| Message | Logical drive member drive removed |
| 1st Line Message |  |
| Old Message | ID:%.1x Logical Drive NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Removed |
| Cause | Specific drive removal detected. |
| Action | Check and reinstall the removed drive. |
| Event ID | 220A0186 |
| Severity | Error |
| Category-Module | FW-LD |
| Prefix Parameter |  |
| Message | Logical drive member drive removed |
| 1st Line Message |  |
| Old Message | ID:%.1x Logical Drive NOTICE: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Removed |
| Cause | Specific drive removal detected. |
| Action | Check and reinstall the removed drive. |
| Event ID | 22080641 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | SCSI parity or CRC error |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: SCSI Parity/CRC Error Detected |
| Cause | Parity or CRC errors detected between drive and controller. |
| Action | 1. Wait until error clears automatically, which is usually the case. 2. If error occours frequently, contact technical support for issue escalation. |
| Event ID | 22080681 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | SCSI parity or CRC error |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: SCSI Parity/CRC Error Detected |
| Cause | Parity or CRC errors detected between drive and controller. |
| Action | 1. Wait until error clears automatically, which is usually the case. 2. If error occours frequently, contact technical support for issue escalation. |
| Event ID | 22080141 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Drive unexpected select timeout |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Unexpected Select Timeout |
| Cause | 1. Drive removed or not properly installed. 2. Drive failed. 3. Backplane connection issue. |
| Action | 1. Check and reinstall drive. 2. Replace the failed drive. 3. Replace the backplane. 4. If issue was not fixed, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |
| Event ID | 22080181 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Drive unexpected select timeout |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Unexpected Select Timeout |
| Cause | 1. Drive removed or not properly installed. 2. Drive failed. 3. Backplane connection issue. |
| Action | 1. Check and reinstall drive. 2. Replace the failed drive. 3. Replace the backplane. 4. If issue was not fixed, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |
| Event ID | 21080242 |
| Severity | Error |
| Category-Module | HW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Gross phase or signal error detected |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Gross Phase/Signal Error Detected |
| Cause | 1. Abnormal signal found on drive channel. This may result from drive issues. 2. If several drives encounter this problem, it could be caused by unstable backplane connection. 3. If several drives encounter this problem in the same expansion enclosure, it could be caused by unstable SAS expansion cable. |
| Action | 1. Replace the faulty drive if only one reports the error. 2. For multiple error within single JBOD enclosure, replace the SAS expansion cable. 3. If (2) can not work, replace the backplane. 4. If issue was not fixed, select System -> Export System Information in SANWatch. Contact technical support with System Information for issue escalation. |
| Event ID | 21080282 |
| Severity | Error |
| Category-Module | HW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Gross phase or signal error detected |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Gross Phase/Signal Error Detected |
| Cause | 1. Abnormal signal found on drive channel. This may result from drive issues. 2. If several drives encounter this problem, it could be caused by unstable backplane connection. 3. If several drives encounter this problem in the same expansion enclosure, it could be caused by unstable SAS expansion cable. |
| Action | 1. Replace the faulty drive if only one reports the error. 2. For multiple error within single JBOD enclosure, replace the SAS expansion cable. 3. If (2) can not work, replace the backplane. 4. If issue was not fixed, select System -> Export System Information in SANWatch. Contact technical support with System Information for issue escalation. |
| Event ID | 22080541 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Drive I/O timeout |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Timeout Waiting for I/O to Complete |
| Cause | Drive I/O response over timeout value (default 30 seconds) due to: 1. Unstable drive. 2. Improper drive-side parameters in RAID system settings. |
| Action | 1. Replace faulty drive. 2. Check the setting and modify it to default value. |
| Event ID | 22080581 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Drive I/O timeout |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Timeout Waiting for I/O to Complete |
| Cause | Drive I/O response over timeout value (default 30 seconds) due to: 1. Unstable drive. 2. Improper drive-side parameters in RAID system settings. |
| Action | 1. Replace faulty drive. 2. Check the setting and modify it to default value. |
| Event ID | 22080741 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Data overrun or underrun |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Data Overrun/Underrun Detected |
| Cause | Data overrun/underrun occurred between hard drive and controller. |
| Action | 1. Errors typically clear automatically. 2. If unresolved, replace the hard drive.. |
| Event ID | 22080781 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Data overrun or underrun |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Data Overrun/Underrun Detected |
| Cause | Data overrun/underrun occurred between hard drive and controller. |
| Action | 1. Errors typically clear automatically. 2. If unresolved, replace the hard drive.. |
| Event ID | 22080842 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Invalid status or sense data received (#) |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Invalid Status/Sense Data Received (%02x/%02x/%02x) |
| Cause | Invalid status reported by drive. |
| Action | 1. Error typically clears automatically. 2. If unresolved, contact technical support for issue escalation. |
| Event ID | 22080882 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Invalid status or sense data received (#) |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Invalid Status/Sense Data Received (%02x/%02x/%02x) |
| Cause | Invalid status reported by drive. |
| Action | 1. Error typically clears automatically. 2. If unresolved, contact technical support for issue escalation. |
| Event ID | 22080841 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Invalid status or sense data received |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Invalid Status/Sense Data Received |
| Cause | Invalid status data reports by drive. |
| Action | 1. Error typically clears automatically. 2. If unresolved, contact technical support for issue escalation. |
| Event ID | 22080881 |
| Severity | Error |
| Category-Module | FW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Invalid status or sense data received |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Invalid Status/Sense Data Received |
| Cause | Invalid status data reports by drive. |
| Action | 1. Error typically clears automatically. 2. If unresolved, contact technical support for issue escalation. |
| Event ID | 22081142 |
| Severity | Error |
| Category-Module | HW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Drive not ready detected (#) |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Unexpected Drive Not Ready (%02x/%02x/%02x) |
| Cause | Drive signal contains errors. |
| Action | 1. Replace the problematic hard drive. 2. If issue was not fixed, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |
| Event ID | 22081182 |
| Severity | Error |
| Category-Module | HW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Drive not ready detected (#) |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Unexpected Drive Not Ready (%02x/%02x/%02x) |
| Cause | Drive signal contains errors. |
| Action | 1. Replace the problematic hard drive. 2. If issue was not fixed, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |
| Event ID | 22081141 |
| Severity | Error |
| Category-Module | HW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Drive not ready detected |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Unexpected Drive Not Ready |
| Cause | Drive signal contains errors. |
| Action | 1. Replace the problematic hard drive. 2. If issue was not fixed, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |

| Event ID | 22081181 |
| --- | --- |
| Severity | Error |
| Category-Module | HW-DRIVE |
| Prefix Parameter |  |
| Message | Drive error detected |
| 1st Line Message | Drive not ready detected |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Unexpected Drive Not Ready |
| Cause | Drive signal contains errors. |
| Action | 1. Replace the problematic hard drive. 2. If issue was not fixed, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |

| Event ID | 22081442 |  |
| --- | --- | --- |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Unit attention received (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Unit Attention Received (%02x/%02x/%02x) |  |
| Cause | LUN (unit) access issue reported from drive. System usually will retry and recover the drive condition. If the drive condition is bad, system retry may fail and results in drive failure or missing. |  |
| Action | 1. Replace drive if drive failure or missing is reported. 2. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID | 22081482 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Unit attention received (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Unit Attention Received (%02x/%02x/%02x) |  |
| Cause | LUN (unit) access issue reported from drive. System usually will retry and recover the drive condition. If the drive condition is bad, system retry may fail and results in drive failure or missing. |  |
| Action | 1. Replace drive if drive failure or missing is reported. 2. If issue was not fixed, select System -> Export System Information in Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID | 22081441 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Unit attention received |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Unit Attention Received (%02x/%02x/%02x) |  |
| Cause | Unit attention received on drive. |  |
| Action | 1. Error typically clears automatically. 2. If unresolved, contact technical support for issue escalation. |  |
| Event ID | 22081481 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Unit attention received |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Target WARNING: Unit Attention Received (%02x/%02x/%02x) |  |
| Cause | Unit attention received on drive. |  |
| Action | 1. Error typically clears automatically. 2. If unresolved, contact technical support for issue escalation. |  |
| Event ID | 22081242 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive failed. |  |
| 1st Line Message | Drive hardware error detected (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Drive HW Error (%02x/%02x/%02x) |  |
| Cause | Drive diagnostic reported failure. |  |
| Action | Replace the failed drive. |  |
| Event ID | 22081282 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive failed. |  |
| 1st Line Message | Drive hardware error detected (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Drive HW Error (%02x/%02x/%02x) |  |
| Cause | Drive diagnostic reported failure. |  |
| Action | Replace the failed drive. |  |
| Event ID | 22081241 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive failed. |  |
| 1st Line Message | Drive hardware error detected (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Drive HW Error (%02x/%02x/%02x) |  |
| Cause | Drive diagnostic reported failure. |  |
| Action | Replace the failed drive. |  |
| Event ID | 22081281 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive failed. |  |
| 1st Line Message | Drive hardware error detected (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Drive HW Error (%02x/%02x/%02x) |  |
| Cause | Drive diagnostic reported failure. |  |
| Action | Replace the failed drive. |  |
| Event ID | 22081842 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive command aborted (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Aborted Command (%02x/%02x/%02x) |  |
| Cause | Disk drive commands aborted due to drive failure. |  |
| Action | Replace the failed drive. |  |
| Event ID | 22081882 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive command aborted (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Aborted Command (%02x/%02x/%02x) |  |
| Cause | Disk drive commands aborted due to drive failure. |  |
| Action | Replace the failed drive. |  |
| Event ID | 22081841 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive command aborted |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Aborted Command |  |
| Cause | Disk drive commands aborted due to drive failure. |  |
| Action | Replace the failed drive. |  |
| Event ID | 22081881 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive command aborted |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Aborted Command |  |
| Cause | Disk drive commands aborted due to drive failure. |  |
| Action | Replace the failed drive. |  |
| Event ID | 22081542 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Unexpected sense data received (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Unexpected Sense Received (%02x/%02x/%02x) |  |
| Cause | Unexpected sense data reported from disk due to: 1. Drive issue 2. Slot issue 3. JBOD SAS cable issue |  |
| Action | 1. If the issue happens on only single drive, replace the drive. 2. If the issue happens on a large number of drives (ex. every drives in an enclosure), check and replace the SAS cable base on the slot A path or slot B path. 3. If the issue remains after disk drive or SAS cable have been replaced, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |  |
| Event ID | 22081582 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Unexpected sense data received (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Unexpected Sense Received (%02x/%02x/%02x) |  |
| Cause | Unexpected sense data reported from disk due to: 1. Drive issue 2. Slot issue 3. JBOD SAS cable issue |  |
| Action | 1. If the issue happens on only single drive, replace the drive. 2. If the issue happens on a large number of drives (ex. every drives in an enclosure), check and replace the SAS cable base on the slot A path or slot B path. 3. If the issue remains after disk drive or SAS cable have been replaced, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |  |
| Event ID | 22081541 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Unexpected sense data received (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Unexpected Sense Received |  |
| Cause | Unexpected sense data reported from disk due to: 1. Drive issue 2. Slot issue 3. JBOD SAS cable issue |  |
| Action | 1. If the issue happens on only single drive, replace the drive. 2. If the issue happens on a large number of drives (ex. every drives in an enclosure), check and replace the SAS cable base on the slot A path or slot B path. 3. If the issue remains after disk drive or SAS cable have been replaced, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |  |
| Event ID | 22081581 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Unexpected sense data received (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Unexpected Sense Received |  |
| Cause | Unexpected sense data reported from disk due to: 1. Drive issue 2. Slot issue 3. JBOD SAS cable issue |  |
| Action | 1. If the issue happens on only single drive, replace the drive. 2. If the issue happens on a large number of drives (ex. every drives in an enclosure), check and replace the SAS cable base on the slot A path or slot B path. 3. If the issue remains after disk drive or SAS cable have been replaced, select System -> Export System Information from the latest SANWatch. Contact technical support with System information for issue escalation. |  |
| Event ID | 02081741 |  |
| Severity | Information |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive error block reassigned - # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Block Successfully Reassigned - 0x%s |  |
| Cause | 'Writing to drives reassigned due to bad blocks. Media errors found in disk drives. |  |
| Action | If this message occurs frequently, replace the disk drive |  |
| Event ID | 02081781 |  |
| Severity | Information |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive error block reassigned - # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Block Successfully Reassigned - 0x%s |  |
| Cause | 'Writing to drives reassigned due to bad blocks. Media errors found in disk drives. |  |
| Action | If this message occurs frequently, replace the disk drive |  |
| Event ID | 22081642 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive error block failed to reassign - # (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Block Reassignment Failed - 0x%s (%02x/%02x/%02x) |  |
| Cause | Drive write media error reported internally inside the drive and cause data reassignment failed. |  |
| Action | Replace the hard drive. |  |
| Event ID | 22081682 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive error block failed to reassign - # (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Block Reassignment Failed - 0x%s (%02x/%02x/%02x) |  |
| Cause | Drive write media error reported internally inside the drive and cause data reassignment failed. |  |
| Action | Replace the hard drive. |  |
| Event ID | 22081641 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive error block failed to reassign - # (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Block Reassignment Failed - 0x%s |  |
| Cause | Write media error reported within the drive, but hard drive reassignment to avoid bad block failed. |  |
| Action | Replace the hard drive. |  |
| Event ID | 22081681 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Drive error block failed to reassign - # (#) |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive WARNING: Block Reassignment Failed - 0x%s |  |
| Cause | Write media error reported within the drive, but hard drive reassignment to avoid bad block failed. |  |
| Action | Replace the hard drive. |  |
| Event ID | 02084341 |  |
| Severity | Information |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message |  |  |
| 1st Line Message | Drive media error recovered - # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) NOTICE:Media Error Recovered-0x%s |  |
| Cause | Data on bad blocks restored. |  |
| Action |  |  |
| Event ID | 02084381 |  |
| Severity | Information |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message |  |  |
| 1st Line Message | Drive media error recovered - # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) NOTICE:Media Error Recovered-0x%s |  |
| Cause | Data on bad blocks restored. |  |
| Action |  |  |
| Event ID | 22084342 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Unrecovered drive media error detected |  |
| 1st Line Message | Unrecoverable media error - # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) ALERT:Media Error Unrecoverable-0x%s |  |
| Cause | Drive damaged physically and became unreliable. |  |
| Action | Replace faulty drives. |  |
| Event ID | 22084382 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Unrecovered drive media error detected |  |
| 1st Line Message | Unrecoverable media error - # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) ALERT:Media Error Unrecoverable-0x%s |  |
| Cause | Drive damaged physically and became unreliable. |  |
| Action | Replace faulty drives. |  |
| Event ID | 0208C141 |  |
| Severity | Information |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive scanned |  |
| 1st Line Message |  |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Scan Drive Successful |  |
| Cause | 'Scanning disk drive completed without errors. |  |
| Action | 'No action required. |  |
| Event ID | 0208C181 |  |
| Severity | Information |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive scanned |  |
| 1st Line Message |  |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Scan Drive Successful |  |
| Cause | 'Scanning disk drive completed without errors. |  |
| Action | 'No action required. |  |
| Event ID | 1208C142 |  |
| Severity | Warning |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Exiled drive detected |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Exiled Drive Detected |  |
| Cause | Unstable LD member drive used to be failed once would become Exiled drive after rebooting RAID subsystem or re-inserting the failed drive. |  |
| Action | Replace the Exiled drive with a new drive for LD rebuilding. |  |
| Event ID | 1208C182 |  |
| Severity | Warning |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error detected |  |
| 1st Line Message | Exiled drive detected |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Exiled Drive Detected |  |
| Cause | Unstable LD member drive used to be failed once would become Exiled drive after rebooting RAID subsystem or re-inserting the failed drive. |  |
| Action | Replace the Exiled drive with a new drive for LD rebuilding. |  |
| Event ID | 2108C143 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Unsupported drive detected |  |
| 1st Line Message | Unsupported code: # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Unsupported Drive Detected(0x%08lx) |  |
| Cause | Drive type (for ESVA only) or license is invalid and not supported by RAID subsystem. |  |
| Action | 1. Replace unrecognized drive with drive supplied by vendor. 2. If issue was not fixed, select System -> Export System Information from the latest Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID | 2108C183 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Unsupported drive detected |  |
| 1st Line Message | Unsupported code: # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Unsupported Drive Detected(0x%08lx) |  |
| Cause | Drive type (for ESVA only) or license is invalid and not supported by RAID subsystem. |  |
| Action | 1. Replace unrecognized drive with drive supplied by vendor. 2. If issue was not fixed, select System -> Export System Information from the latest Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID | 2108C144 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Unsupported drive detected |  |
| 1st Line Message | Incorrect drive bundle ID: # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Unsupported Drive Detected(0x%04lx) |  |
| Cause | Drive with incorrect bundle ID found in subsystem. |  |
| Action | 1. Replace unsupported drives with the one supplied by vendor. 2. If issue was not fixed, select System -> Export System Information from the latest Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID | 2108C184 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Unsupported drive detected |  |
| 1st Line Message | Incorrect drive bundle ID: # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Unsupported Drive Detected(0x%04lx) |  |
| Cause | Drive with incorrect bundle ID found in subsystem. |  |
| Action | 1. Replace unsupported drives with the one supplied by vendor. 2. If issue was not fixed, select System -> Export System Information from the latest Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID | 2108C145 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Unsupported drive detected |  |
| 1st Line Message | Drive bundle code not supported. ID: #, code: # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Unsupported Drive Detected(0x%04lx,0x%04lx) |  |
| Cause | Drive with unsupported bundle code found in subsystem. |  |
| Action | 1. Replace unsupported drive with the one supplied by vendor. 2. If issue was not fixed, select System -> Export System Information from the latest Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID | 2108C185 |  |
| Severity | Error |  |
| Category-Module | HW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Unsupported drive detected |  |
| 1st Line Message | Drive bundle code not supported. ID: #, code: # |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Unsupported Drive Detected(0x%04lx,0x%04lx) |  |
| Cause | Drive with unsupported bundle code found in subsystem. |  |
| Action | 1. Replace unsupported drive with the one supplied by vendor. 2. If issue was not fixed, select System -> Export System Information from the latest Management GUI. Contact technical support with System information for issue escalation. |  |
| Event ID | 2108C147 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive detection failed |  |
| 1st Line Message |  |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Scan Drive Failed |  |
| Cause | System fail to scan in drive due to: 1. disk hardware issue 2. enclosure hardware issue |  |
| Action | 1. Reinsert disk. 2. If problem still persist, or if disk failure event also occurs, replace disk. 3. If problem still persist, replace the enclosure. |  |
| Event ID | 2108C187 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive detection failed |  |
| 1st Line Message |  |  |
| Old Message | CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive NOTICE: Scan Drive Failed |  |
| Cause | System fail to scan in drive due to: 1. disk hardware issue 2. enclosure hardware issue |  |
| Action | 1. Reinsert disk. 2. If problem still persist, or if disk failure event also occurs, replace disk. 3. If problem still persist, replace the enclosure. |  |
| Event ID | 12084243 |  |
| Severity | Warning |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive SMART error state detected |  |
| 1st Line Message |  |  |
| Old Message | WARNING:SMART-CH %.1d ID:%.1d (JBODId:%d SlotNum:%d) Predictable Failure Detected |  |
| Cause | Drive SMART error threshold exceeded. |  |
| Action | 1. Clone and replace problematic drive. 2. Replace the original cloned drive with new one. |  |
| Event ID | 12084283 |  |
| Severity | Warning |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive SMART error state detected |  |
| 1st Line Message |  |  |
| Old Message | WARNING:SMART-CH %.1d ID:%.1d (JBODId:%d SlotNum:%d) Predictable Failure Detected |  |
| Cause | Drive SMART error threshold exceeded. |  |
| Action | 1. Clone and replace problematic drive. 2. Replace the original cloned drive with new one. |  |
| Event ID | 12084244 |  |
| Severity | Warning |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error recovery procedure started |  |
| 1st Line Message | Drive contains SMART errors |  |
| Old Message | WARNING:SMART-CH %.1d ID:%.1d (JBODId:%d SlotNum:%d) Predictable Failure Detected-Starting Clone |  |
| Cause | Drive SMART errors was found and trigger the clone opearion for replacement. |  |
| Action | No action required except for replacing the cloned disk that has errors when recovery procedure is completed. |  |
| Event ID | 12084284 |  |
| Severity | Warning |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error recovery procedure started |  |
| 1st Line Message | Drive contains SMART errors |  |
| Old Message | WARNING:SMART-CH %.1d ID:%.1d (JBODId:%d SlotNum:%d) Predictable Failure Detected-Starting Clone |  |
| Cause | Drive SMART errors was found and trigger the clone opearion for replacement. |  |
| Action | No action required except for replacing the cloned disk that has errors when recovery procedure is completed. |  |
| Event ID | 22084245 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error recovery procedure started |  |
| 1st Line Message | Clone failed |  |
| Old Message | WARNING:SMART-CH %.1d ID:%.1d (JBODId:%d SlotNum:%d) Predictable Failure Detected-Clone Failed |  |
| Cause | Drive data clone operation stopped due to drive failure. |  |
| Action | 1. Replace the failed drive. 2. Restart clone operation manually. |  |
| Event ID | 22084285 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | Drive error recovery procedure started |  |
| 1st Line Message | Clone failed |  |
| Old Message | WARNING:SMART-CH %.1d ID:%.1d (JBODId:%d SlotNum:%d) Predictable Failure Detected-Clone Failed |  |
| Cause | Drive data clone operation stopped due to drive failure. |  |
| Action | 1. Replace the failed drive. 2. Restart clone operation manually. |  |
| Event ID | 020AA141 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone resumed |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Continue Clone Operation |  |
| Cause | Cloning a drive in a logical drive ongoing. |  |
| Action |  |  |
| Event ID | 020AA181 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone resumed |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Continue Clone Operation |  |
| Cause | Cloning a drive in a logical drive ongoing. |  |
| Action |  |  |
| Event ID | 020AA142 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone started |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Starting Clone |  |
| Cause | Cloning a drive in a logical drive started. |  |
| Action |  |  |
| Event ID | 020AA182 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone started |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Starting Clone |  |
| Cause | Cloning a drive in a logical drive started. |  |
| Action |  |  |
| Event ID | 020AA241 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive copy and replace completed |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Copy and Replace Completed |  |
| Cause | Copying and replacing a drive in a logical drive completed. |  |
| Action |  |  |
| Event ID | 020AA281 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive copy and replace completed |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Copy and Replace Completed |  |
| Cause | Copying and replacing a drive in a logical drive completed. |  |
| Action |  |  |
| Event ID | 020AA242 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone completed |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Clone Completed |  |
| Cause | Cloning a drive in a logical drive completed. |  |
| Action |  |  |
| Event ID | 020AA282 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone completed |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Clone Completed |  |
| Cause | Cloning a drive in a logical drive completed. |  |
| Action |  |  |
| Event ID | 220A1141 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone failed |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive ALERT:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Clone Failed |  |
| Cause | Drive data clone operation stopped in a specific logical drive because of drive failure. |  |
| Action | 1. Replace the failed drive. 2. Restart the clone operation manually. |  |
| Event ID | 220A1181 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone failed |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive ALERT:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Clone Failed |  |
| Cause | Drive data clone operation stopped in a specific logical drive because of drive failure. |  |
| Action | 1. Replace the failed drive. 2. Restart the clone operation manually. |  |
| Event ID | 220A1142 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone aborted |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive ALERT:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Clone Aborted |  |
| Cause | Drive data clone operation stopped in a specific logical drive due to: 1. User cancelled the operation manually. 2. Source drive failed during the clone operation. 3. Target drive failed during the clone operation. |  |
| Action | 1. Restart the operation manually. 2. If a spare drive is available, the rebuild operation will start automatically. 3. If there is no spare drive, rebuild will use the target drive as a spare before cloning. 4. Replace the failed target drive. |  |
| Event ID | 220A1182 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Drive clone aborted |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive ALERT:CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Clone Aborted |  |
| Cause | Drive data clone operation stopped in a specific logical drive due to: 1. User cancelled the operation manually. 2. Source drive failed during the clone operation. 3. Target drive failed during the clone operation. |  |
| Action | 1. Restart the operation manually. 2. If a spare drive is available, the rebuild operation will start automatically. 3. If there is no spare drive, rebuild will use the target drive as a spare before cloning. 4. Replace the failed target drive. |  |
| Event ID | 21011001 |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The secondary controller is incompatible and it is not allowed to boot up. |  |
| 1st Line Message |  |  |
| Old Message | The secondary controller is incompatible and it is not allowed to boot up. |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 21011002 |  |
| Severity | Error |  |
| Category-Module | HW-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The size of memory installed in the secondary controller is inconsistent. |  |
| 1st Line Message |  |  |
| Old Message | The size of memory installed in the secondary controller is inconsistent. |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 20011003 |  |
| Severity | Error |  |
| Category-Module | Generic-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The secondary controller is not for the enclosure, and it contains unsaved write cache data. |  |
| 1st Line Message |  |  |
| Old Message | The secondary controller is not for the enclosure, and it contains unsaved write cache data. |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 22011004 |  |
| Severity | Error |  |
| Category-Module | FW-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The firmware of the secondary controller is incompatible. |  |
| 1st Line Message |  |  |
| Old Message | The firmware of the secondary controller is incompatible. |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 22011005 |  |
| Severity | Error |  |
| Category-Module | FW-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The cache memory range of the secondary controller is incompatible. |  |
| 1st Line Message |  |  |
| Old Message | The cache memory range of the secondary controller is incompatible. |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 20011001 |  |
| Severity | Error |  |
| Category-Module | Generic-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The secondary controller is not allowed to boot up because the fatal failed LD contains unsaved write cache data. |  |
| 1st Line Message |  |  |
| Old Message | The secondary controller is not allowed to boot up because the fatal failed LD contains unsaved write cache data. |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 20011002 |  |
| Severity | Error |  |
| Category-Module | Generic-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The secondary controller is not allowed to boot up and is waiting for the write cache data recovery. |  |
| 1st Line Message |  |  |
| Old Message | The secondary controller is not allowed to boot up and is waiting for the write cache data recovery. |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 20011006 |  |
| Severity | Error |  |
| Category-Module | Generic-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The secondary controller is not allowed to boot up because it contains backed-up data which are not found in primary controller. |  |
| 1st Line Message |  |  |
| Old Message | The secondary controller is not allowed to boot up because the backed-up cache data can not be found in primary controller. |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 120B0802 |  |
| Severity | Warning |  |
| Category-Module | FW-LV |  |
| Prefix Parameter |  |  |
| Message | Logical volume cache data purged |  |
| 1st Line Message |  |  |
| Old Message | LV:%d Logical Volume WARNING: Cache Data Purged |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 120A0803 |  |
| Severity | Warning |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive cache data purged |  |
| 1st Line Message |  |  |
| Old Message | LG:%d Logical Drive WARNING: Cache Data Purged |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 220A4205 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive bad data block detected and marked |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 220A4206 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Unprotected block on the logical drive detected and marked |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 120A4207 |  |
| Severity | Warning |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Unprotected block on the logical drive detected and marked |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 220A4208 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Unprotected block on the logical drive detected and marked |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 220A4405 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive inconsistent parity block detected |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 020A4406 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive inconsistent parity block recovered |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 220A4407 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive media error detected |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 120A4408 |  |
| Severity | Warning |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive media error recovered |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 2208C148 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | The 4K sector drive is unsupported. |  |
| 1st Line Message |  |  |
| Old Message | CHL:%.1d ID:%.1d Drive NOTICE: Scan Drive Failed |  |
| Cause | System fail to scan in drive due to: 1. disk hardware issue 2. enclosure hardware issue |  |
| Action | 1. Reinsert disk. 2. If problem still persist, or if disk failure event also occurs, replace disk. 3. If problem still persist, replace the enclosure. |  |
| Event ID | 2208C188 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | The 4K sector drive is unsupported. |  |
| 1st Line Message |  |  |
| Old Message | CHL:%.1d ID:%.1d Drive NOTICE: Scan Drive Failed |  |
| Cause | System fail to scan in drive due to: 1. disk hardware issue 2. enclosure hardware issue |  |
| Action | 1. Reinsert disk. 2. If problem still persist, or if disk failure event also occurs, replace disk. 3. If problem still persist, replace the enclosure. |  |
| Event ID | 10190001 |  |
| Severity | Warning |  |
| Category-Module | Generic-AGENT |  |
| Prefix Parameter |  |  |
| Message | User # failed to logged in from # via # due to authorization failure. |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause | user logged in with incorrect username or password |  |
| Action |  |  |
| Event ID | 00190002 |  |
| Severity | Information |  |
| Category-Module | Generic-AGENT |  |
| Prefix Parameter |  |  |
| Message | User # logged in from # via #. |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause | user has logged in |  |
| Action |  |  |
| Event ID | 020D1501 |  |
| Severity | Information |  |
| Category-Module | FW-LICENSE |  |
| Prefix Parameter |  |  |
| Message | The trial function of EonCloud Gateway will expire and be disabled in # day(s). |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 01091E01 |  |
| Severity | Information |  |
| Category-Module | HW-CHANNEL |  |
| Prefix Parameter |  |  |
| Message | The configured channel speed (# Gb) does not match the SFP speed (# Gb). |  |
| 1st Line Message |  |  |
| Old Message | CH:%d Invalid %dG SFP installation detected while host board mode is %dG |  |
| Cause |  |  |
| Action | Change the SFP with compatible speed. |  |
| Event ID | 32091F01 |  |
| Severity | Critical Error |  |
| Category-Module | FW-CHANNEL |  |
| Prefix Parameter |  |  |
| Message | Channel # is down due to unexpected critical errors. |  |
| 1st Line Message |  |  |
| Old Message | Channel %d is down due to unexpected critical errors. (pci reset fail, port is disabled. statusReg=%d errReg1=%d errReg2=%d) |  |
| Cause | The chip firmware may encounter some error and unable to recover. |  |
| Action | Reboot to recover port function. |  |
| Event ID | 220A0147 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | A second or third LD member drive failed. |  |
| 1st Line Message |  |  |
| Old Message | Name:%s Id:%s Pool Name:%s ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Missing |  |
| Cause | Possible cause: 1. User removed the drive or the drive tray was not properly inserted. 2. The drive failed. 3. Backplane connection issue. |  |
| Action | 1. Insert the drive back and rebuild the Logical Drive if needed. 2. Replace the failed drive and rebuild the logical drive. 3. Replace the backplane. |  |
| Event ID | 220A0187 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | A second or third LD member drive failed. |  |
| 1st Line Message |  |  |
| Old Message | Name:%s Id:%s Pool Name:%s ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Missing |  |
| Cause | Possible cause: 1. User removed the drive or the drive tray was not properly inserted. 2. The drive failed. 3. Backplane connection issue. |  |
| Action | 1. Insert the drive back and rebuild the Logical Drive if needed. 2. Replace the failed drive and rebuild the logical drive. 3. Replace the backplane. |  |
| Event ID | 220A0148 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | The first LD member drive failed. |  |
| 1st Line Message |  |  |
| Old Message | Name:%s Id:%s Pool Name:%s ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Failure |  |
| Cause | Possible cause: 1. User removed the drive or the drive tray was not properly inserted. 2. The drive failed. 3. Backplane connection issue. |  |
| Action | 1. Insert the drive back and rebuild the Logical Drive if needed. 2. Replace the failed drive and rebuild the logical drive. 3. Replace the backplane. |  |
| Event ID | 220A0188 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | The first LD member drive failed. |  |
| 1st Line Message |  |  |
| Old Message | Name:%s Id:%s Pool Name:%s ID:%.1x Logical Drive ALERT: CHL:%.1d ID:%.1d (JBODId:%d SlotNum:%d) Drive Failure |  |
| Cause | Possible cause: 1. User removed the drive or the drive tray was not properly inserted. 2. The drive failed. 3. Backplane connection issue. |  |
| Action | 1. Insert the drive back and rebuild the Logical Drive if needed. 2. Replace the failed drive and rebuild the logical drive. 3. Replace the backplane. |  |
| Event ID | 020A8305 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive rebuild started |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE: Starting Rebuild |  |
| Cause | 'Rebuilding a logical drive started |  |
| Action |  |  |
| Event ID | 020A8306 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive rebuild started |  |
| 1st Line Message |  |  |
| Old Message | ID:%.1x Logical Drive NOTICE: Starting Rebuild |  |
| Cause | 'Rebuilding a logical drive started |  |
| Action |  |  |
| Event ID | 020A8307 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive rebuild started |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 020A8308 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive rebuild started |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 020A8309 |  |
| Severity | Information |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Logical drive rebuild started |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 121C0101 |  |
| Severity | Warning |  |
| Category-Module | FW-SNMP |  |
| Prefix Parameter |  |  |
| Message | Failed to load the SNMP settings. The system has applied the default SNMP settings. |  |
| 1st Line Message |  |  |
| Old Message | Failed to load the SNMP settings. The system has applied the default SNMP settings. (Flag: %x) |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 121C0102 |  |
| Severity | Warning |  |
| Category-Module | FW-SNMP |  |
| Prefix Parameter |  |  |
| Message | Failed to load the SNMP settings. The system has applied the default SNMP settings. |  |
| 1st Line Message |  |  |
| Old Message | Failed to load the SNMP settings. The system has applied the default SNMP settings. (Flag: %x) |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 02190003 |  |
| Severity | Information |  |
| Category-Module | FW-AGENT |  |
| Prefix Parameter |  |  |
| Message | Firmware updated. The storage device password is synced to this embedded system. |  |
| 1st Line Message |  |  |
| Old Message |  |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 220A0901 |  |
| Severity | Error |  |
| Category-Module | FW-LD |  |
| Prefix Parameter |  |  |
| Message | Unable to create a logical drive. The system is busy initializing created logical drives. |  |
| 1st Line Message |  |  |
| Old Message | Unable to create a logical drive. The system is busy initializing created logical drives. (reason: %x, lg idx: %x, ld id: %x) |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 22081B01 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | The MUX board for the SATA drive is not connected, or a drive PHY error is detected. |  |
| 1st Line Message |  |  |
| Old Message | Enc Ctrl B SATA Drive Chl:%x Slot:%x Expdr(%x) redundant phy is error |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 22081B02 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | The MUX board for the SATA drive is not connected, or a drive PHY error is detected. |  |
| 1st Line Message |  |  |
| Old Message | JBOD:%x Ctrl B SATA Drive Chl:%x Slot:%x Expdr(%x) redundant phy is error |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 22081B03 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | A drive PHY error is detected. |  |
| 1st Line Message |  |  |
| Old Message | Enc Ctrl %x, Drive type:%x, Drive Chl:%x Slot:%x Expdr(%x) redundant phy is error |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 22081B04 |  |
| Severity | Error |  |
| Category-Module | FW-DRIVE |  |
| Prefix Parameter |  |  |
| Message | A drive PHY error is detected. |  |
| 1st Line Message |  |  |
| Old Message | JBOD:%x Ctrl %x Drive type:%x, Drive Chl:%x Slot:%x Expdr(%x) redundant phy is error |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 12011601 |  |
| Severity | Warning |  |
| Category-Module | FW-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | Jumbo frame enabled. The system has run out of memory and disabled the second Ethernet board. |  |
| 1st Line Message | Host board: # |  |
| Old Message | Jumbo frame enabled. The system has run out of memory and disabled the second Ethernet board. |  |
| Cause |  |  |
| Action |  |  |
| Event ID | 12011602 |  |
| Severity | Warning |  |
| Category-Module | FW-CONTROLLER |  |
| Prefix Parameter |  |  |
| Message | The system is rebooted to enable jumbo frame. The system has run out of memory and disabled the second Ethernet board. |  |
| 1st Line Message | Host board: # |  |
| Old Message | The system is rebooted to enable jumbo frame. The system has run out of memory and disabled the second Ethernet board. |  |
| Cause |  |  |
| Action |  |  |
| Event ID |  | 0x82090401 |  |
| Severity |  | Information |  |
| Category-Module |  | Account - User |  |
| Message |  | {0}'s password has been changed by {1}. |  |
| 1st Line Message |  | {0}'s password has been changed by {1}. |  |
| Cause |  | User's password has been changed by himself or someone. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x82090402 |  |
| Severity |  | Information |  |
| Category-Module |  | Account - User |  |
| Message |  | {0}'s password has expired. |  |
| 1st Line Message |  | {0}'s password has expired. |  |
| Cause |  | Someone's password expired. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x82020403 |  |
| Severity |  | Information |  |
| Category-Module |  | System |  |
| Message |  | The password policy has been enabled. |  |
| 1st Line Message |  | The password policy has been enabled. |  |
| Cause |  | The password policy has been enabled by someone. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x82020404 |  |
| Severity |  | Information |  |
| Category-Module |  | System |  |
| Message |  | The settings of the password policy have been changed. |  |
| 1st Line Message |  | The settings of the password policy have been changed. |  |
| Cause |  | The settings of password policy have been changed by someone. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x82020405 |  |
| Severity |  | Information |  |
| Category-Module |  | System |  |
| Message |  | The password policy has been disabled. |  |
| 1st Line Message |  | The password policy has been disabled. |  |
| Cause |  | The password policy has been disabled by someone. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x82050407 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | The {0} service status has returned to normal. |  |
| 1st Line Message |  | The {0} service status has returned to normal. |  |
| Cause |  | service is back to normal |  |
| Action |  | do nothing |  |
| Event ID |  | 0x82410408 |  |
| Severity |  | Warning |  |
| Category-Module |  | HWMonitor_Service |  |
| Message |  | Service {0} will be reactivated in a few minutes because its status is abnormal. |  |
| 1st Line Message |  | Service {0} will be reactivated in a few minutes because its status is abnormal. |  |
| Cause |  | Restart the service. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x8245041A |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Failed to restart the {0} service. |  |
| 1st Line Message |  | Failed to restart the {0} service. |  |
| Cause |  | Failed to restart the specific service. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x8209040B |  |
| Severity |  | Information |  |
| Category-Module |  | Account - User |  |
| Message |  | The user {0} has been created. |  |
| 1st Line Message |  | The user {0} has been created. |  |
| Cause |  | Create a user account |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8209040C |  |
| Severity |  | Information |  |
| Category-Module |  | Account - User |  |
| Message |  | The user {0} has been assigned to the following group(s): {1} |  |
| 1st Line Message |  | The user {0} has been assigned to the following group(s): {1} |  |
| Cause |  | The user has been assigned to group(s) |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8209040D |  |
| Severity |  | Information |  |
| Category-Module |  | Account - User |  |
| Message |  | The superuser privilege has been assigned to the user {0}. |  |
| 1st Line Message |  | The superuser privilege has been assigned to the user {0}. |  |
| Cause |  | The superuser privilege has been assigned to a user |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8209040E |  |
| Severity |  | Information |  |
| Category-Module |  | Account - User |  |
| Message |  | The superuser privilege for user {0} has been stopped. |  |
| 1st Line Message |  | The superuser privilege for user {0} has been stopped. |  |
| Cause |  | Superuser privilege has been stopped |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8209040F |  |
| Severity |  | Information |  |
| Category-Module |  | Account - User |  |
| Message |  | The user {0} has been deleted. |  |
| 1st Line Message |  | The user {0} has been deleted. |  |
| Cause |  | User account has been delete |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820A0410 |  |
| Severity |  | Information |  |
| Category-Module |  | Account - Group |  |
| Message |  | The group {0} has been created. |  |
| 1st Line Message |  | The group {0} has been created. |  |
| Cause |  | Add a new group |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820A0411 |  |
| Severity |  | Information |  |
| Category-Module |  | Account - Group |  |
| Message |  | The group {0} has been deleted. |  |
| 1st Line Message |  | The group {0} has been deleted. |  |
| Cause |  | Delete a group |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820A0412 |  |
| Severity |  | Information |  |
| Category-Module |  | Account - Group |  |
| Message |  | The group {0} added the following user(s): {1} |  |
| 1st Line Message |  | The group {0} added the following user(s): {1} |  |
| Cause |  | Add user(s) to a group |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820A0413 |  |
| Severity |  | Information |  |
| Category-Module |  | Account - Group |  |
| Message |  | The group {0} removed the following user(s): {1} |  |
| 1st Line Message |  | The group {0} removed the following user(s): {1} |  |
| Cause |  | Remove user(s) from a group |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82050414 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | The {0} service has been started successfully. |  |
| 1st Line Message |  | The {0} service has been started successfully. |  |
| Cause |  | The specific service has been started |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82050415 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | The {0} service has been restarted successfully. |  |
| 1st Line Message |  | The {0} service has been restarted successfully. |  |
| Cause |  | The specific service has been restarted |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82050416 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | The {0} service has been stopped successfully. |  |
| 1st Line Message |  | The {0} service has been stopped successfully. |  |
| Cause |  | The specific service has been stopped |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82050417 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | The configuration of {0} service has been applied successfully. |  |
| 1st Line Message |  | The configuration of {0} service has been applied successfully. |  |
| Cause |  | The configuration of specific service has been applied. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82450418 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Failed to start the {0} service. |  |
| 1st Line Message |  | Failed to start the {0} service. |  |
| Cause |  | Failed to start the specific service. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x82850419 |  |
| Severity |  | Error |  |
| Category-Module |  | Service |  |
| Message |  | Failed to start the {0} service. |  |
| 1st Line Message |  | Failed to start the {0} service. |  |
| Cause |  | Failed to start the specific service. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x8245041A |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Failed to restart the {0} service. |  |
| 1st Line Message |  | Failed to restart the {0} service. |  |
| Cause |  | Failed to restart the specific service. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x8285041B |  |
| Severity |  | Error |  |
| Category-Module |  | Service |  |
| Message |  | Failed to restart the {0} service. |  |
| 1st Line Message |  | Failed to restart the {0} service. |  |
| Cause |  | Failed to restart the specific service. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x8245041C |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Failed to stop the {0} service. |  |
| 1st Line Message |  | Failed to stop the {0} service. |  |
| Cause |  | Failed to stop the specific service. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x8245041E |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Failed to set the configuration of {0} service. |  |
| 1st Line Message |  | Failed to set the configuration of {0} service. |  |
| Cause |  | Fail to set the configuration of specific service. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x8285041D |  |
| Severity |  | Error |  |
| Category-Module |  | Service |  |
| Message |  | Failed to stop the {0} service. |  |
| 1st Line Message |  | Failed to stop the {0} service. |  |
| Cause |  | Failed to stop the specific service. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x8285041F |  |
| Severity |  | Error |  |
| Category-Module |  | Service |  |
| Message |  | Failed to set the configuration of {0} service. |  |
| 1st Line Message |  | Failed to set the configuration of {0} service. |  |
| Cause |  | Fail to set the configuration of specific service. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x82070420 |  |
| Severity |  | Information |  |
| Category-Module |  | Share |  |
| Message |  | The {0} has been added into share configuration of {1} protocol successfully. |  |
| 1st Line Message |  | The {0} has been added into share configuration of {1} protocol successfully. |  |
| Cause |  | The share path via specific protocol has been created. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82070421 |  |
| Severity |  | Information |  |
| Category-Module |  | Share |  |
| Message |  | The {0} has been removed from share configuration of {1} protocol successfully. |  |
| 1st Line Message |  | The {0} has been removed from share configuration of {1} protocol successfully. |  |
| Cause |  | The share path via specific protocol has been removed. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82070422 |  |
| Severity |  | Information |  |
| Category-Module |  | Share |  |
| Message |  | The share configuration of {0} via {1} protocol has been applied successfully. |  |
| 1st Line Message |  | The share configuration of {0} via {1} protocol has been applied successfully. |  |
| Cause |  | The configuration of the share path via specific protocol has been applied. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82470423 |  |
| Severity |  | Warning |  |
| Category-Module |  | Share |  |
| Message |  | Failed to add {0} into share configuration of {1} protocol. |  |
| 1st Line Message |  | Failed to add {0} into share configuration of {1} protocol. |  |
| Cause |  | Fail to create the share path via specific protocol. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x82450424 |  |
| Severity |  | Warning |  |
| Category-Module |  | Share |  |
| Message |  | Failed to remove {0} from share configuration of {1} protocol. |  |
| 1st Line Message |  | Failed to remove {0} from share configuration of {1} protocol. |  |
| Cause |  | Fail to remove the share path via specific protocol. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x82450425 |  |
| Severity |  | Warning |  |
| Category-Module |  | Share |  |
| Message |  | Failed to apply the share configuration of {0} via {1} protocol. |  |
| 1st Line Message |  | Failed to apply the share configuration of {0} via {1} protocol. |  |
| Cause |  | Fail to apply the configuration of the share path via specific protocol. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824B0426 |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Remote replication task {1} test failed. Target directory {0} invalid. |  |
| 1st Line Message |  | Remote replication task {1} test failed. Target directory {0} invalid. |  |
| Cause |  | Verify the settings of remote replication task. The target folder is not existed. |  |
| Action |  | The target folder may be removed. User should assign a valid folder as target. 1. Edit the Remote Replication task, or 2. Create the folder in remote site. |  |
| Event ID |  | 0x824B0427 |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Remote replication task {1} test failed. Host {0} non-responsive. |  |
| 1st Line Message |  | Remote replication task {1} test failed. Host {0} non-responsive. |  |
| Cause |  | Verify the settings of remote replication task . Remote site is no responding. |  |
| Action |  | Check the status of remote site. (On/off) Check the network setting and status. |  |
| Event ID |  | 0x824B0428 |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Remote replication task {1} test failed. Invalid username or password. |  |
| 1st Line Message |  | Remote replication task {1} test failed. Invalid username or password. |  |
| Cause |  | Verify the settings of remote replication task . Username or password to login remote site is incorrect. |  |
| Action |  | 1. Check the username and password. 2. The setting of remote site may have changed. |  |
| Event ID |  | 0x820B0429 |  |
| Severity |  | Information |  |
| Category-Module |  | RR |  |
| Message |  | Remote replication task from {0} to {1}, has been deleted. |  |
| 1st Line Message |  | Remote replication task from {0} to {1}, has been deleted. |  |
| Cause |  | RR task has been deleted |  |
| Action |  | N/A |  |
| Event ID |  | 0x820B042B |  |
| Severity |  | Information |  |
| Category-Module |  | RR |  |
| Message |  | The backup operation of the remote replication task from {0} to {1} has started. |  |
| 1st Line Message |  | The backup operation of the remote replication task from {0} to {1} has started. |  |
| Cause |  | RR task has been started |  |
| Action |  | N/A |  |
| Event ID |  | 0x820B042D |  |
| Severity |  | Information |  |
| Category-Module |  | RR |  |
| Message |  | The remote replication task, from {0} to {1} has stopped. |  |
| 1st Line Message |  | The remote replication task, from {0} to {1} has stopped. |  |
| Cause |  | RR task has been stoped |  |
| Action |  | N/A |  |
| Event ID |  | 0x820B042F |  |
| Severity |  | Information |  |
| Category-Module |  | RR |  |
| Message |  | The restoration of remote replication task from {1} to {0} has begun. |  |
| 1st Line Message |  | The restoration of remote replication task from {1} to {0} has begun. |  |
| Cause |  | RR task has start to restore |  |
| Action |  | N/A |  |
| Event ID |  | 0x820B0431 |  |
| Severity |  | Information |  |
| Category-Module |  | RR |  |
| Message |  | Remote replication task from {0} to {1} has been restored. |  |
| 1st Line Message |  | Remote replication task from {0} to {1} has been restored. |  |
| Cause |  | Restore RR task has been complete |  |
| Action |  | N/A |  |
| Event ID |  | 0x820B0433 |  |
| Severity |  | Information |  |
| Category-Module |  | RR |  |
| Message |  | Remote replication from {0} to {1} has been completed. |  |
| 1st Line Message |  | Remote replication from {0} to {1} has been completed. |  |
| Cause |  | Execute RR task has been complete |  |
| Action |  | N/A |  |
| Event ID |  | 0x824B042A |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Fail to delete remote replication task from {0} to {1}. |  |
| 1st Line Message |  | Fail to delete remote replication task from {0} to {1}. |  |
| Cause |  | code exception |  |
| Action |  | Check log of HAServer |  |
| Event ID |  | 0x824B042C |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Failed to backup the remote replication task from {0} to {1}. |  |
| 1st Line Message |  | Failed to backup the remote replication task from {0} to {1}. |  |
| Cause |  | 1. Target IP is invalid 2. Username/password has been changed. 3. Network has been borken. |  |
| Action |  | 1. Check network status. 2. Check target server IP. 3. Check username/password and permission |  |
| Event ID |  | 0x824B042E |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Failed to stop the remote replication task from {0} to {1}. |  |
| 1st Line Message |  | Failed to stop the remote replication task from {0} to {1}. |  |
| Cause |  | code exception |  |
| Action |  | Check log of HAServer |  |
| Event ID |  | 0x824B0430 |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Fail to restore remote replication task from {1} to {0}. |  |
| 1st Line Message |  | Fail to restore remote replication task from {1} to {0}. |  |
| Cause |  | 1. Target IP is invalid 2. Username/password has been changed. 3. Network has been borken. |  |
| Action |  | 1. Check network status. 2. Check target server IP. 3. Check username/password and permission |  |
| Event ID |  | 0x824B0432 |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Fail to restore remote replication task from {0} to {1}. |  |
| 1st Line Message |  | Fail to restore remote replication task from {0} to {1}. |  |
| Cause |  | 1. Target IP is invalid 2. Username/password has been changed. 3. Network has been borken. |  |
| Action |  | 1. Check network status. 2. Check target server IP. 3. Check username/password and permission |  |
| Event ID |  | 0x824B0434 |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Failed to replicate data from {0} to {1}. |  |
| 1st Line Message |  | Failed to replicate data from {0} to {1}. |  |
| Cause |  | 1. Target IP is invalid 2. Username/password has been changed. 3. Network has been borken. |  |
| Action |  | 1. Check network status. 2. Check target server IP. 3. Check username/password and permission |  |
| Event ID |  | 0x824B0435 |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Remote replication task {0} target folder has insufficient storage capacity. |  |
| 1st Line Message |  | Remote replication task {0} target folder has insufficient storage capacity. |  |
| Cause |  | The quota of destination folder is full. |  |
| Action |  | Check the quota of destination folder |  |
| Event ID |  | 0x824B0436 |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Network timeout, remote replication from {0} to {1} failed to start. |  |
| 1st Line Message |  | Network timeout, remote replication from {0} to {1} failed to start. |  |
| Cause |  | 1. io is too slow 2. network problem |  |
| Action |  | Check the system status , and network situation. |  |
| Event ID |  | 0x824B0438 |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Failed to create remote replication task from {0} to {1}. |  |
| 1st Line Message |  | Failed to create remote replication task from {0} to {1}. |  |
| Cause |  | code exception |  |
| Action |  | Check log of HAServer |  |
| Event ID |  | 0x824C043A |  |
| Severity |  | Warning |  |
| Category-Module |  | Schedule |  |
| Message |  | Failed to create schedule {0}. |  |
| 1st Line Message |  | Failed to create schedule {0}. |  |
| Cause |  | code exception |  |
| Action |  | Check log of HAServer |  |
| Event ID |  | 0x824C043C |  |
| Severity |  | Warning |  |
| Category-Module |  | Schedule |  |
| Message |  | Failed to enable schedule {0} |  |
| 1st Line Message |  | Failed to enable schedule {0} |  |
| Cause |  | code exception |  |
| Action |  | Check log of HAServer |  |
| Event ID |  | 0x824C043E |  |
| Severity |  | Warning |  |
| Category-Module |  | Schedule |  |
| Message |  | Failed to disable schedule {0} . |  |
| 1st Line Message |  | Failed to disable schedule {0} . |  |
| Cause |  | code exception |  |
| Action |  | Check log of HAServer |  |
| Event ID |  | 0x824C0440 |  |
| Severity |  | Warning |  |
| Category-Module |  | Schedule |  |
| Message |  | Failed to delete schedule {0}. |  |
| 1st Line Message |  | Failed to delete schedule {0}. |  |
| Cause |  | ceased |  |
| Action |  | N/A |  |
| Event ID |  | 0x824C0441 |  |
| Severity |  | Warning |  |
| Category-Module |  | Schedule |  |
| Message |  | Scheduled task {0} is still running, the task will run on the next schedule. |  |
| 1st Line Message |  | Scheduled task {0} is still running, the task will run on the next schedule. |  |
| Cause |  | rsync process is still in progress. |  |
| Action |  | 1. Wait 2. Change the period |  |
| Event ID |  | 0x82460445 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | Failed to create file system {0}. |  |
| 1st Line Message |  | Failed to create file system {0}. |  |
| Cause |  | Fail to create a new pool |  |
| Action |  | N/A |  |
| Event ID |  | 0x82460447 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | Failed to delete file system {0}. |  |
| 1st Line Message |  | Failed to delete file system {0}. |  |
| Cause |  | Fail to delete pool |  |
| Action |  | Try again later |  |
| Event ID |  | 0x82460449 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | Fail to create folder {0} in the file system {1}. |  |
| 1st Line Message |  | Fail to create folder {0} in the file system {1}. |  |
| Cause |  | Fail to delete folder |  |
| Action |  | N/A |  |
| Event ID |  | 0x820B0437 |  |
| Severity |  | Information |  |
| Category-Module |  | RR |  |
| Message |  | The remote replication task from {0} to {1} has been created. |  |
| 1st Line Message |  | The remote replication task from {0} to {1} has been created. |  |
| Cause |  | RR task has been created |  |
| Action |  | N/A |  |
| Event ID |  | 0x820C0439 |  |
| Severity |  | Information |  |
| Category-Module |  | Schedule |  |
| Message |  | Schedule {0} has been created. |  |
| 1st Line Message |  | Schedule {0} has been created. |  |
| Cause |  | Create a schedule. |  |
| Action |  | N/A |  |
| Event ID |  | 0x820C043B |  |
| Severity |  | Information |  |
| Category-Module |  | Schedule |  |
| Message |  | Schedule {0} has been enabled. |  |
| 1st Line Message |  | Schedule {0} has been enabled. |  |
| Cause |  | Enable a scheduled backup task. |  |
| Action |  | N/A |  |
| Event ID |  | 0x820C043D |  |
| Severity |  | Information |  |
| Category-Module |  | Schedule |  |
| Message |  | Schedule {0} has been disabled. |  |
| 1st Line Message |  | Schedule {0} has been disabled. |  |
| Cause |  | Disable a scheduled backup task |  |
| Action |  | N/A |  |
| Event ID |  | 0x820C043F |  |
| Severity |  | Information |  |
| Category-Module |  | Schedule |  |
| Message |  | Schedule {0} has been deleted. |  |
| 1st Line Message |  | Schedule {0} has been deleted. |  |
| Cause |  | Delete a schedule. |  |
| Action |  | N/A |  |
| Event ID |  | 0x82060442 |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | {0} Drive inserted. |  |
| 1st Line Message |  | {0} Drive inserted. |  |
| Cause |  | RAID map a new volume to NAS, |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82060443 |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | {0} Drive unplugged |  |
| 1st Line Message |  | {0} Drive unplugged |  |
| Cause |  | the drive in the slot has been removed. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82060444 |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | The file system {0} has been created. |  |
| 1st Line Message |  | The file system {0} has been created. |  |
| Cause |  | Create a new pool |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82060446 |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | The file system {0} has been deleted. |  |
| 1st Line Message |  | The file system {0} has been deleted. |  |
| Cause |  | A pool has been deleted. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82060448 |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | The folder {0} has been created in the file system {1}. |  |
| 1st Line Message |  | The folder {0} has been created in the file system {1}. |  |
| Cause |  | A folder has been created. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8206044A |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | The folder {0} of the file system {1} has been deleted. |  |
| 1st Line Message |  | The folder {0} of the file system {1} has been deleted. |  |
| Cause |  | The folder has been deleted. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8203044C |  |
| Severity |  | Information |  |
| Category-Module |  | Controller |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | System has been downgraded to single mode. |  |
| Action |  | Insert another controller or do nothing |  |
| Event ID |  | 0x8203044D |  |
| Severity |  | Information |  |
| Category-Module |  | Controller |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | Controller has been booted |  |
| Action |  | N/A |  |
| Event ID |  | 0x82020450 |  |
| Severity |  | Information |  |
| Category-Module |  | System |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | A controller failure has been detected. The controller failover process will be launched. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82020451 |  |
| Severity |  | Information |  |
| Category-Module |  | System |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | The controller failover process has been completed. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82020452 |  |
| Severity |  | Information |  |
| Category-Module |  | System |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | The failed controller has been recovered. The failback process will be launched. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82020453 |  |
| Severity |  | Information |  |
| Category-Module |  | System |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | The controller failback process has been completed. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82020455 |  |
| Severity |  | Information |  |
| Category-Module |  | Controller |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | Controller has been inserted |  |
| Action |  | N/A |  |
| Event ID |  | 0x82040456 |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | Network connection {0} has been restored. |  |
| 1st Line Message |  | Network connection {0} has been restored. |  |
| Cause |  | NIC has been restored |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82040458 |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | Interface {0} from aggregation group {2} has been restored. |  |
| 1st Line Message |  | Interface {0} from aggregation group {2} has been restored. |  |
| Cause |  | Trunk port has been restored |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8204045A |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | The address mode of {0} has been set to {2}. The IP address is {3}. |  |
| 1st Line Message |  | The address mode of {0} has been set to {2}. The IP address is {3}. |  |
| Cause |  | Success to set IP config |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8246044B |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | Fail to delete folder {0} of the file system {1}. |  |
| 1st Line Message |  | Fail to delete folder {0} of the file system {1}. |  |
| Cause |  | Fail to delete folder |  |
| Action |  | N/A |  |
| Event ID |  | 0x8243044E |  |
| Severity |  | Warning |  |
| Category-Module |  | Controller |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | controller unplug or heartbeat error |  |
| Action |  | wait remote controller reboot |  |
| Event ID |  | 0x8242044F |  |
| Severity |  | Warning |  |
| Category-Module |  | System |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | power interruption |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82420454 |  |
| Severity |  | Warning |  |
| Category-Module |  | Controller |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | Controller has been unplugged |  |
| Action |  | Plug the controller back |  |
| Event ID |  | 0x82440457 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Network connection {0} is disconnected. |  |
| 1st Line Message |  | Network connection {0} is disconnected. |  |
| Cause |  | NIC failed to restore |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82440459 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Interface {0} from aggregation group {2} is abnormal. |  |
| 1st Line Message |  | Interface {0} from aggregation group {2} is abnormal. |  |
| Cause |  | Trunk port failed to restore |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8204045A |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | The address mode of {0} has been set to {2}. The IP address is {3}. |  |
| 1st Line Message |  | The address mode of {0} has been set to {2}. The IP address is {3}. |  |
| Cause |  | Success to set IP config |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8204045B |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | The MTU size of {1} has been set to {0}. |  |
| 1st Line Message |  | The MTU size of {1} has been set to {0}. |  |
| Cause |  | Success to set jumbo frame |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8204045C |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | The DNS server {0} has been added. |  |
| 1st Line Message |  | The DNS server {0} has been added. |  |
| Cause |  | Success to add dns entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8204045D |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | The DNS server {0} has been removed. |  |
| 1st Line Message |  | The DNS server {0} has been removed. |  |
| Cause |  | Success to remove dns entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8204045E |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | The DNS suffix {0} has been added. |  |
| 1st Line Message |  | The DNS suffix {0} has been added. |  |
| Cause |  | Success to add dns suffix entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8204045F |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | The DNS suffix {0} has been removed. |  |
| 1st Line Message |  | The DNS suffix {0} has been removed. |  |
| Cause |  | Success to remove dns suffix entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82040460 |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | The port aggregation group {0} has been created. |  |
| 1st Line Message |  | The port aggregation group {0} has been created. |  |
| Cause |  | Success to add trunk port. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82040461 |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | The port aggregation group {0} has been remmoved. |  |
| 1st Line Message |  | The port aggregation group {0} has been removed. |  |
| Cause |  | Success to remove trunk port. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82040462 |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | Route rule has been added successfully. |  |
| 1st Line Message |  | Route rule has been added successfully. |  |
| Cause |  | Success to add an route entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82040463 |  |
| Severity |  | Information |  |
| Category-Module |  | Network |  |
| Message |  | Route rule has been removed successfully. |  |
| 1st Line Message |  | Route rule has been removed successfully. |  |
| Cause |  | Success to remove an route entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82440464 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to set IP config on interface {0} |  |
| 1st Line Message |  | Failed to set IP config on interface {0} |  |
| Cause |  | Failed to set IP config |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82440465 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to set MTU size to {0} on interface {1} |  |
| 1st Line Message |  | Failed to set MTU size to {0} on interface {1} |  |
| Cause |  | Failed to set jumbo frame |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82440466 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to add DNS server {0}. |  |
| 1st Line Message |  | Failed to add DNS server{0}. |  |
| Cause |  | Failed to add dns entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82440467 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to remove DNS server {0}. |  |
| 1st Line Message |  | Failed to remove DNS server {0}. |  |
| Cause |  | Failed to remove dns entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82440468 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to add DNS suffix {0}. |  |
| 1st Line Message |  | Failed to add DNS suffix {0}. |  |
| Cause |  | Failed to add dns suffix entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82440469 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to remove DNS suffix {0}. |  |
| 1st Line Message |  | Failed to remove DNS suffix {0}. |  |
| Cause |  | Failed to remove dns suffix entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8244046A |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to create the port aggregation group {0}. |  |
| 1st Line Message |  | Failed to create the port aggregation group {0}. |  |
| Cause |  | Failed to add trunk port. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8244046B |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to remove the port aggregation group {0}. |  |
| 1st Line Message |  | Failed to remove the port aggregation group {0}. |  |
| Cause |  | Failed to remove trunk port. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8244046C |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to add an Route rule. |  |
| 1st Line Message |  | Failed to add an Route rule. |  |
| Cause |  | Failed to add an route entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8244046D |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Failed to remove an Route rule. |  |
| 1st Line Message |  | Failed to remove an Route rule. |  |
| Cause |  | Failed to remove an route entry |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8242046E |  |
| Severity |  | Warning |  |
| Category-Module |  | System |  |
| Message |  | The usage of the coredump folder is over 90%. Please backup the data as soon as possible. |  |
| 1st Line Message |  | The usage of the coredump folder is over 90%. Please backup the data as soon as possible. |  |
| Cause |  | Usage of coredump folder is over 90% |  |
| Action |  | Export the coredump file |  |
| Event ID |  | 0x8246046F |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | The space used by the folder {0} has exceeded the monitoring threshold: {1}. |  |
| 1st Line Message |  | The space used by the folder {0} has exceeded the monitoring threshold: {1}. |  |
| Cause |  | The space used by the folder has exceeded the monitoring threshold. |  |
| Action |  | Expand the volume to get more capacity or do nothing |  |
| Event ID |  | 0x820D0470 |  |
| Severity |  | Information |  |
| Category-Module |  | Network - Security |  |
| Message |  | The ipblock configuration has been applied. |  |
| 1st Line Message |  | The ipblock configuration has been applied. |  |
| Cause |  | The ipblock configuration has been appilied. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820D0472 |  |
| Severity |  | Information |  |
| Category-Module |  | Network - Security |  |
| Message |  | The selected ip address has been removed. |  |
| 1st Line Message |  | The selected ip address has been removed. |  |
| Cause |  | The selected ip address has been removed from ipblock configuration. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824D0471 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network - Security |  |
| Message |  | Fail to apply the ipblock configuration |  |
| 1st Line Message |  | Fail to apply the ipblock configuration |  |
| Cause |  | Fail to apply the ipblock configuration. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824D0473 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network - Security |  |
| Message |  | Fail to remove the ip address from ipblock configuration. |  |
| 1st Line Message |  | Fail to remove the ip address from ipblock configuration. |  |
| Cause |  | Fail to removed selected ip address from ipblock configuration.. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824D0474 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network - Security |  |
| Message |  | {0} was blocked via {1} due to the maximum login attempts exceeded. |  |
| 1st Line Message |  | {0} was blocked via {1} due to the maximum login attempts exceeded. |  |
| Cause |  | The ip address has been banned due to configuration compliance |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824D0475 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network - Security |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | Fail to ban ip address on other Controller |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824C0478 |  |
| Severity |  | Warning |  |
| Category-Module |  | Schedule |  |
| Message |  | Failed to start the schedule {0}. |  |
| 1st Line Message |  | Failed to start the schedule task {0}. |  |
| Cause |  | failed to start the schedule task. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824D0479 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network - Security |  |
| Message |  | Failed to restore the whitelist/blacklist. |  |
| 1st Line Message |  | Failed to restore the whitelist/blacklist. |  |
| Cause |  | Failed to restore the black/white list. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8245047B |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | The LDAP server failed to add the user {0} when importing the users’ list from the CSV file. |  |
| 1st Line Message |  | The LDAP server failed to add the user {0} when importing the users’ list from the CSV file. |  |
| Cause |  | Failed to add user to LDAP server by import CSV file. |  |
| Action |  | Check the user account |  |
| Event ID |  | 0x8249047C |  |
| Severity |  | Warning |  |
| Category-Module |  | Account - User |  |
| Message |  | The user {0} has nearly reached the quota limit on the volume {1}. |  |
| 1st Line Message |  | The user {0} has nearly reached the quota limit on the volume {1}. |  |
| Cause |  | User quota has nearly reached the quota limit on the specific volume. |  |
| Action |  | 1. Expand user quota, or 2. Notify the user. |  |
| Event ID |  | 0x8245047D |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | The LDAP server failed to add user(s) in batch mode. (User name prefix: {0}; User name start number: {1}; Number of users: {2}) You can delete the created user(s), or try again with overwriting the existing user(s) later. |  |
| 1st Line Message |  | The LDAP server failed to add user(s) in batch mode. (User name prefix: {0}; User name start number: {1}; Number of users: {2}) You can delete the created user(s), or try again with overwriting the existing user(s) later. |  |
| Cause |  | Failed to add user by creating user(s) in batch mode. |  |
| Action |  | 1. Check the user list. 2. Delete the created user(s) and try again. 3. Or, try again and overwite the existed user(s) |  |
| Event ID |  | 0x8245047E |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | The AD/LDAP server has been disconnected. Please check the network connection and server status. |  |
| 1st Line Message |  | The AD/LDAP server has been disconnected. Please check the network connection and server status. |  |
| Cause |  | AD/LDAP server connection has been disconnected. |  |
| Action |  | Check the network status between the storage device and AD/LDAP server. |  |
| Event ID |  | 0x820D0476 |  |
| Severity |  | Information |  |
| Category-Module |  | Network - Security |  |
| Message |  | Detect the ip {0} has been unbanned by system |  |
| 1st Line Message |  | Detect the ip {0} has been unbanned by system |  |
| Cause |  | The ip address has been unbanned due to configuration compliance |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820C0477 |  |
| Severity |  | Information |  |
| Category-Module |  | Schedule |  |
| Message |  | The schedule {0} has started |  |
| 1st Line Message |  | The schedule task {0} has started |  |
| Cause |  | schedule task started. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820D047A |  |
| Severity |  | Information |  |
| Category-Module |  | Network - Security |  |
| Message |  | User {0} failed to log in from {1} via {2} |  |
| 1st Line Message |  | User {0} failed to log in from {1} via {2} |  |
| Cause |  | Detect from client ip user login failed via specific services |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x8205047F |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | The AD/LDAP server connection has been restored. |  |
| 1st Line Message |  | The AD/LDAP server connection has been restored. |  |
| Cause |  | AD/LDAP server connection has been restored. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82460480 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | All volumes have been deactivated. |  |
| 1st Line Message |  | All volumes have been deactivated. |  |
| Cause |  | deactivate NAS pools |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x82450482 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | The setting of port {0} for the service {1} has a conflict with another service. Please check the port settings of the services. |  |
| 1st Line Message |  | The setting of port {0} for the service {1} has a conflict with another service. Please check the port settings of the services. |  |
| Cause |  | Port conclict |  |
| Action |  | Check the service settings |  |
| Event ID |  | 0x820E0483 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The {0} server has been started successfully. |  |
| 1st Line Message |  | The {0} server has been started successfully. |  |
| Cause |  | Application server has been started. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820E0484 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The {0} server has been restarted successfully. |  |
| 1st Line Message |  | The {0} server has been restarted successfully. |  |
| Cause |  | Application server has been restarted. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820E0485 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The {0} server has been stopped successfully. |  |
| 1st Line Message |  | The {0} server has been stopped successfully. |  |
| Cause |  | Application server has been stopped. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820E0486 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The {0} server has been configured successfully. |  |
| 1st Line Message |  | The {0} server has been configured successfully. |  |
| Cause |  | Application server has been configured. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824E0487 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to start the {0} server. |  |
| 1st Line Message |  | Failed to start the {0} server. |  |
| Cause |  | Failed to start the application server. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824E0488 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to restart the {0} server. |  |
| 1st Line Message |  | Failed to restart the {0} server. |  |
| Cause |  | Failed to restart the application server. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824E0489 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to stop the {0} server. |  |
| 1st Line Message |  | Failed to stop the {0} server. |  |
| Cause |  | Failed to stop the application server |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824E048A |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to configure the {0} server. |  |
| 1st Line Message |  | Failed to configure the {0} server. |  |
| Cause |  | Failed to configure the application server |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824E048B |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | The {0} server has been stopped because the folder for saving temporary data is full or has been deleted. Please check the folder status of {1}. |  |
| 1st Line Message |  | The {0} server has been stopped because the folder for saving temporary data is full or has been deleted. Please check the folder status of {1}. |  |
| Cause |  | Application has been stopped because of the cache folder has been deleted. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824E048C |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to back up the LDAP server database because the folder for saving the data may have been deleted. Please check the LDAP server settings. |  |
| 1st Line Message |  | Failed to back up the LDAP server database because the folder for saving the data may have been deleted. Please check the LDAP server settings. |  |
| Cause |  | The folder for backing-up LDAP server database does not exist. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824E048D |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | N/A |  |
| 1st Line Message |  | N/A |  |
| Cause |  | The folder for backing-up LDAP server database does not exist on primary controller. |  |
| Action |  | Check the settings and try again. |  |
| Event ID |  | 0x824E048F |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to start SyncCloud service. |  |
| 1st Line Message |  | Failed to start SyncCloud service. |  |
| Cause |  | Failed to start SyncCloud service. |  |
| Action |  | Check settings of the service and netwrok status. Try again later. |  |
| Event ID |  | 0x824E0491 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to stop SyncCloud service. |  |
| 1st Line Message |  | Failed to stop SyncCloud service. |  |
| Cause |  | System busy. |  |
| Action |  | try again. |  |
| Event ID |  | 0x820E048E |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | SyncCloud service started successfully. |  |
| 1st Line Message |  | SyncCloud service started successfully. |  |
| Cause |  | SyncCloud service started successfully. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820E0490 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | SyncCloud service stopped successfully. |  |
| 1st Line Message |  | SyncCloud service stopped successfully. |  |
| Cause |  | SyncCloud service stopped successfully. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x824E0492 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to fetch SyncCloud config database from {0}. Sync Cloud service has been automatically disabled by system. |  |
| 1st Line Message |  | Failed to fetch SyncCloud config database from {0}. Sync Cloud service has been automatically disabled by system. |  |
| Cause |  | The folder can not be accessed. Or the database crashed. |  |
| Action |  | Check the database location. |  |
| Event ID |  | 0x82460493 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | The volume {0} (ID: {1}) for file service cannot be mounted. There is another volume with the same pool/volume name already. Please check the pool/volume name. |  |
| 1st Line Message |  | The volume {0} (ID: {1}) for file service cannot be mounted. There is another volume with the same pool/volume name already. Please check the pool/volume name. |  |
| Cause |  | There is another volume with the same pool/volume name already. |  |
| Action |  | Check and change pool/volume name. |  |
| Event ID |  | 0x82460494 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | The folder {0} size has exceeded the alert threshold of its quota. |  |
| 1st Line Message |  | The folder {0} size has exceeded the alert threshold of its quota. |  |
| Cause |  | The folder used size exceeds the threshold. |  |
| Action |  | 1. Delete some files/subfolders. 2. extend the quota size. |  |
| Event ID |  | 0x82460495 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | IP address conflict detected, conflicted IP(v4): {0} |  |
| 1st Line Message |  | IP address conflict detected, conflicted IP(v4): {0} |  |
| Cause |  | IPv4 address conflict detected. |  |
| Action |  | Change the IP setting |  |
| Event ID |  | 0x82460496 |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | IP address conflict detected, conflicted IP(v6): {0} |  |
| 1st Line Message |  | IP address conflict detected, conflicted IP(v6): {0} |  |
| Cause |  | IPv6 address conflict detected. |  |
| Action |  | Change the IP setting |  |
| Event ID |  | 0x824E0497 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to init SyncCloud config database {0}. |  |
| 1st Line Message |  | Failed to init SyncCloud config database {0}. |  |
| Cause |  | Failed to initial the SyncCloud database. |  |
| Action |  | Check working folder capacity and try again. |  |
| Event ID |  | 0x824E0498 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | The SyncCloud task, which is owned by {3}, between local folder {0} and remote folder {1} of the provider {2} failed due to network problems. |  |
| 1st Line Message |  | The SyncCloud task, which is owned by {3}, between local folder {0} and remote folder {1} of the provider {2} failed due to network problems. |  |
| Cause |  | Network problem |  |
| Action |  | 1. Check network connection |  |
| Event ID |  | 0x824E0499 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | File system {0}, {1} percent capacity used. |  |
| 1st Line Message |  | File system {0}, {1} percent capacity used. |  |
| Cause |  | File system capacity usage over threshold |  |
| Action |  | 1. Delete some files 2. Expand the file system |  |
| Event ID |  | 0x82C6049A |  |
| Severity |  | Critical |  |
| Category-Module |  | Storage |  |
| Message |  | File system {0}, {1} percent capacity used. |  |
| 1st Line Message |  | File system {0}, {1} percent capacity used. |  |
| Cause |  | File system capacity usage over threshold |  |
| Action |  | 1. Delete some files 2. Expand the file system |  |
| Event ID |  | 0x8246049B |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | The file system of the volume {0}, which has failed to be mounted, has been repaired. |  |
| 1st Line Message |  | The file system of the volume {0}, which has failed to be mounted, has been repaired. |  |
| Cause |  | The file system of the volume, which has failed to be mounted, has been repaired. |  |
| Action |  | Check the file in the folder “lost & found” |  |
| Event ID |  | 0x8246049C |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | Failed to repair the file system of the volume {0}, which has failed to be mounted. |  |
| 1st Line Message |  | Failed to repair the file system of the volume {0}, which has failed to be mounted. |  |
| Cause |  | Failed to repair the file system of the volume. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824E049D |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to upgrade the LDAP server service. |  |
| 1st Line Message |  | Failed to upgrade the LDAP server service. |  |
| Cause |  | Failed to upgrade the LDAP server service. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824E049E |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The LDAP server service has been upgraded. |  |
| 1st Line Message |  | The LDAP server service has been upgraded. |  |
| Cause |  | LDAP server service has been upgraded. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824E049F |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | The LDAP server service has been disabled because the folder for saving LDAP server data is not found. |  |
| 1st Line Message |  | The LDAP server service has been disabled because the folder for saving LDAP server data is not found. |  |
| Cause |  | Working folder is not found. |  |
| Action |  | Configure the settings and activate the service again. |  |
| Event ID |  | 0x824E04A0 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | The LDAP server service has been disabled because the settings are incomplete or not found. |  |
| 1st Line Message |  | The LDAP server service has been disabled because the settings are incomplete or not found. |  |
| Cause |  | The configuration of the LDAP server service is not found. |  |
| Action |  | Check the settings of the LDAP server service. |  |
| Event ID |  | 0x824E04A1 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | The LDAP server service has been disabled because the database was corrupted. |  |
| 1st Line Message |  | The LDAP server service has been disabled because the database was corrupted. |  |
| Cause |  | The LDAP server database corrupted. |  |
| Action |  | Reset the LDAP server |  |
| Event ID |  | 0x824504A2 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Failed to connect to the AD server because the username or password is incorrect. |  |
| 1st Line Message |  | Failed to connect to the AD server because the username or password is incorrect. |  |
| Cause |  | The username or password is incorrect. |  |
| Action |  | Check the username and passowrd and try again later. |  |
| Event ID |  | 0x824504A3 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Failed to connect to the AD server because the KDC server is not found. |  |
| 1st Line Message |  | Failed to connect to the AD server because the KDC server is not found. |  |
| Cause |  | DNS entry error: The KDC server is not found. |  |
| Action |  | Check the DNS server settings. |  |
| Event ID |  | 0x824504A4 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | The AD server is unreachable. |  |
| 1st Line Message |  | The AD server is unreachable. |  |
| Cause |  | The AD server is unreachable. |  |
| Action |  | Check the AD server IP address and try again later. |  |
| Event ID |  | 0x824504A5 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Not enough privilege for the AD user {0} to join the domain. |  |
| 1st Line Message |  | Not enough privilege for the AD user {0} to join the domain. |  |
| Cause |  | Privilege is not enough to join the domain. |  |
| Action |  | Use another user to join the domain. |  |
| Event ID |  | 0x824504A6 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Failed to connect to the LDAP server because the username or password is incorrect. |  |
| 1st Line Message |  | Failed to connect to the LDAP server because the username or password is incorrect. |  |
| Cause |  | The username or password is incorrect. |  |
| Action |  | Check the username and passowrd and try again later. |  |
| Event ID |  | 0x824504A8 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | The LDAP server is unreachable. |  |
| 1st Line Message |  | The LDAP server is unreachable. |  |
| Cause |  | The LDAP server is unreachable. |  |
| Action |  | Check the LDAP server IP address and try again later. |  |
| Event ID |  | 0x824504A9 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Failed to connect to the LDAP server because the base DN is incorrect. |  |
| 1st Line Message |  | Failed to connect to the LDAP server because the base DN is incorrect. |  |
| Cause |  | The base DN of the LDAP server is incorrect. |  |
| Action |  | Check the LDAP server IP address and try again later. |  |
| Event ID |  | 0x820E04AA |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | {0} service started successfully. |  |
| 1st Line Message |  | {0} service started successfully. |  |
| Cause |  | SyncCloud service started successfully. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824E04AB |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to start {0} service. |  |
| 1st Line Message |  | Failed to start {0} service. |  |
| Cause |  | Failed to start SyncCloud service. |  |
| Action |  | Check settings of the service and netwrok status. |  |
| Event ID |  | 0x820E04AC |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | {0} service stopped successfully. |  |
| 1st Line Message |  | {0} service stopped successfully. |  |
| Cause |  | SyncCloud service stopped successfully. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x824E04AD |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to stop {0} service. |  |
| 1st Line Message |  | Failed to start {0} service. |  |
| Cause |  | System busy. |  |
| Action |  | Try again |  |
| Event ID |  | 0x824E04AE |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to fetch {1} config database from {0}. {2} service has been automatically disabled by system. |  |
| 1st Line Message |  | Failed to fetch {1} config database from {0}. {2} service has been automatically disabled by system. |  |
| Cause |  | The folder can not be accessed. Or the database crashed. |  |
| Action |  | Check the database location. |  |
| Event ID |  | 0x824E04AF |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to init {1} config database {0}. |  |
| 1st Line Message |  | Failed to init {1} config database {0}. |  |
| Cause |  | Failed to initial the SyncCloud database. |  |
| Action |  | Check working folder capacity and try again. |  |
| Event ID |  | 0x824E04B0 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | The {4} task, which is owned by {3}, between local folder {0} and remote folder {1} of the provider {2} failed due to network problems. |  |
| 1st Line Message |  | The {4} task, which is owned by {3}, between local folder {0} and remote folder {1} of the provider {2} failed due to network problems. |  |
| Cause |  | Network problem |  |
| Action |  | 1. Check network connection 2. Try again later |  |
| Event ID |  | 0x820E04B1 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The antivirus scan job {0} has been stopped. |  |
| 1st Line Message |  | The antivirus scan job {0} has been stopped. |  |
| Cause |  | The scan job has been stopped |  |
| Action |  | do nothing or try again later. |  |
| Event ID |  | 0x820E04B2 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The antivirus scheduled scan job {0} has been completed. |  |
| 1st Line Message |  | The antivirus scheduled scan job {0} has been completed. |  |
| Cause |  | The scheduled scan job has been completed. |  |
| Action |  | Do nothing or check the log |  |
| Event ID |  | 0x824E04B3 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | The NVR server crash has been detected and the system will start the recovery process. |  |
| 1st Line Message |  | The NVR server crash has been detected and the system will start the recovery process. |  |
| Cause |  | NVR server has creashed. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820E04B4 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The NVR server has been recovered. |  |
| 1st Line Message |  | The NVR server has been recovered. |  |
| Cause |  | NVR server has been recovered. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x824E04B5 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to recover the NVR server. Please contact the technical support. |  |
| 1st Line Message |  | Failed to recover the NVR server. Please contact the technical support. |  |
| Cause |  | Failed to recover NVR server |  |
| Action |  | Contact spport |  |
| Event ID |  | 0x824E04B6 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | There are too many files in the folder {0} queuing to upload to the cloud. |  |
| 1st Line Message |  | There are too many files in the folder {0} queuing to upload to the cloud. |  |
| Cause |  | There are too many dirty data in the folder that needs to be updated to the cloud. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x824E04B7 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | There are too many dirty data in the folder {0} that needs to be updated to the cloud. |  |
| 1st Line Message |  | There are too many dirty data in the folder {0} that needs to be updated to the cloud. |  |
| Cause |  | There are too many dirty data in the folder that needs to be updated to the cloud. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820F04B8 |  |
| Severity |  | Information |  |
| Category-Module |  | Cloud gateway |  |
| Message |  | The privilege of the directory {0} in cloud storage has been changed. |  |
| 1st Line Message |  | The privilege of the directory {0} in cloud storage has been changed. |  |
| Cause |  | Applying privilege changes from requests of access control management |  |
| Action |  | do nothing |  |
| Event ID |  | 0x82CF04B9 |  |
| Severity |  | Critical |  |
| Category-Module |  | Cloud gateway |  |
| Message |  | Prepopulate is ceased because cache size reaches 80% of the quota. |  |
| 1st Line Message |  | Prepopulate is ceased because cache size reaches 80% of the quota. |  |
| Cause |  | Cache size reaches 80% of the quota. |  |
| Action |  | 1. Expand the cache size 2. Disable prepopulate |  |
| Event ID |  | 0x828F04BA |  |
| Severity |  | Error |  |
| Category-Module |  | Cloud gateway |  |
| Message |  | The time difference between local time and remote time is more than 15 minutes. |  |
| 1st Line Message |  | The time difference between local time and remote time is more than 15 minutes. |  |
| Cause |  | Clock is too skew |  |
| Action |  | Adjust the time settings. (or resync with NTP server) |  |
| Event ID |  | 0x820F04BB |  |
| Severity |  | Information |  |
| Category-Module |  | Cloud gateway |  |
| Message |  | The privilege to remote folder {0} is changed from {1} to {2}. |  |
| 1st Line Message |  | The privilege to remote folder {0} is changed from {1} to {2}. |  |
| Cause |  | The privilege to remote folder has been changed. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820E04BC |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | Some {0} initialization process has been skipped. |  |
| 1st Line Message |  | Some {0} initialization process has been skipped. |  |
| Cause |  | initialization process skipped. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820F04BD |  |
| Severity |  | Information |  |
| Category-Module |  | Cloud gateway |  |
| Message |  | Some {0} task initialization process has been skipped. |  |
| 1st Line Message |  | Some {0} task initialization process has been skipped. |  |
| Cause |  | initialization process skipped. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820F04BE |  |
| Severity |  | Information |  |
| Category-Module |  | Cloud gateway |  |
| Message |  | {0} fails to sync data due to out of space of local ({1}) or remote ({2}) folders. |  |
| 1st Line Message |  | {0} fails to sync data due to out of space of local ({1}) or remote ({2}) folders. |  |
| Cause |  | Failed to sync data because local or remote folders are out of space. |  |
| Action |  | Check the quota settings of the local and remote folders. |  |
| Event ID |  | 0x820E04BF |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | {0} fails to sync data due to out of space of local ({1}) or remote ({2}) folders. |  |
| 1st Line Message |  | {0} fails to sync data due to out of space of local ({1}) or remote ({2}) folders. |  |
| Cause |  | Failed to sync data because local or remote folders are out of space. |  |
| Action |  | Check the quota settings of the local and remote folders. |  |
| Event ID |  | 0x820604C0 |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | The usage of the volume {0} in the pool {1} has exceeded the threshold {2}. |  |
| 1st Line Message |  | The usage of the volume {0} in the pool {1} has exceeded the threshold {2}. |  |
| Cause |  | The usage of the volume exceeds the threshold. |  |
| Action |  | 1. Expand the volume 2. Delete content. |  |
| Event ID |  | 0x824604C1 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | The usage of the volume {0} in the pool {1} has exceeded the threshold {2}. |  |
| 1st Line Message |  | The usage of the volume {0} in the pool {1} has exceeded the threshold {2}. |  |
| Cause |  | The usage of the volume exceeds the threshold. |  |
| Action |  | 1. Expand the volume 2. Delete content. |  |
| Event ID |  | 0x828604C2 |  |
| Severity |  | Error |  |
| Category-Module |  | Storage |  |
| Message |  | The usage of the volume {0} in the pool {1} has exceeded the threshold {2}. |  |
| 1st Line Message |  | The usage of the volume {0} in the pool {1} has exceeded the threshold {2}. |  |
| Cause |  | The usage of the volume exceeds the threshold. |  |
| Action |  | 1. Expand the volume 2. Delete content. |  |
| Event ID |  | 0x82C604C3 |  |
| Severity |  | Critical |  |
| Category-Module |  | Storage |  |
| Message |  | The usage of the volume {0} in the pool {1} has exceeded the threshold {2}. |  |
| 1st Line Message |  | The usage of the volume {0} in the pool {1} has exceeded the threshold {2}. |  |
| Cause |  | The usage of the volume exceeds the threshold. |  |
| Action |  | 1. Expand the volume 2. Delete content. |  |
| Event ID |  | 0x824E04C4 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | {0} fails to connect to cloud provider. |  |
| 1st Line Message |  | {0} fails to connect to cloud provider. |  |
| Cause |  | SyncCloud failed to connect to cloud provider. |  |
| Action |  | 1. check network status 2. Try again later |  |
| Event ID |  | 0x824F04C5 |  |
| Severity |  | Warning |  |
| Category-Module |  | Cloud gateway |  |
| Message |  | {0} fails to connect to cloud provider. |  |
| 1st Line Message |  | {0} fails to connect to cloud provider. |  |
| Cause |  | cloud storage gateway failed to connect to cloud provider. |  |
| Action |  | 1. check network status 2. Try again later |  |
| Event ID |  | 0x828404C6 |  |
| Severity |  | Error |  |
| Category-Module |  | Network |  |
| Message |  | CHANNEL {0} Trunking configuration error detected. |  |
| 1st Line Message |  | CHANNEL {0} Trunking configuration error detected. |  |
| Cause |  | Trunk error has been detected because switch configuration is improper. (Controller A) |  |
| Action |  | 1. Check switch configuration 2. Try again |  |
| Event ID |  | 0x828404C7 |  |
| Severity |  | Error |  |
| Category-Module |  | Network |  |
| Message |  | CHANNEL {0} Trunking configuration error detected. |  |
| 1st Line Message |  | CHANNEL {0} Trunking configuration error detected. |  |
| Cause |  | Trunk error has been detected because switch configuration is improper. (Controller B) |  |
| Action |  | 1. Check switch configuration 2. Try again |  |
| Event ID |  | 0x824E04C8 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Docker service crashed due to unexpected errors. |  |
| 1st Line Message |  | Docker service crashed due to unexpected errors. |  |
| Cause |  | Docker service crashed due to unexpected errors. |  |
| Action |  | 1. do nothing (or waiting for service auto recover) 2. restart docker service |  |
| Event ID |  | 0x820E04C9 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | Docker service has resumed from crash. |  |
| 1st Line Message |  | Docker service has resumed from crash. |  |
| Cause |  | Docker service has resumed from crash. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824E04CA |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Docker service failed to resume from crash. |  |
| 1st Line Message |  | Docker service failed to resume from crash. |  |
| Cause |  | Docker service failed to resume from crash. |  |
| Action |  | 1. do nothing (or waiting for service auto recover) 2. restart docker service |  |
| Event ID |  | 0x820E04CB |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | Docker has been disabled by user {0}. |  |
| 1st Line Message |  | Docker has been disabled by user {0}. |  |
| Cause |  | Docker service has been disabled. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820E04CC |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | Docker has been enabled by user {0}. |  |
| 1st Line Message |  | Docker has been enabled by user {0}. |  |
| Cause |  | Docker service has been enabled. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824E04CD |  |
| Severity |  | Warning |  |
| Category-Module |  | Cloud gateway |  |
| Message |  | Cloud integrated folders stop working because the database is lost. |  |
| 1st Line Message |  | Cloud integrated folders stop working because the database is lost. |  |
| Cause |  | The folder for saving cloud gateway database has been deleted. |  |
| Action |  | Configure Cloud gateway service again. |  |
| Event ID |  | 0x824404CE |  |
| Severity |  | Warning |  |
| Category-Module |  | Network |  |
| Message |  | Several connections to Channel {0} have been terminated by {1} administrator. |  |
| 1st Line Message |  | Several connections to Channel {0} have been terminated by {1} administrator. |  |
| Cause |  | Connnection(s) have been terminated by admin |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820504CF |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | User Log for {0} has been enabled. |  |
| 1st Line Message |  | User Log for {0} has been enabled. |  |
| Cause |  | Enable data access log for specific protocol. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820504D0 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | User Log for {0} has been disabled. |  |
| 1st Line Message |  | User Log for {0} has been disabled. |  |
| Cause |  | Disable data access log for specific protocol. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820504D1 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | Failed to write log for {0} due to storage errors or short of spaces. |  |
| 1st Line Message |  | Failed to write log for {0} due to storage errors or short of spaces. |  |
| Cause |  | The space is not enough to save log data. |  |
| Action |  | 1. expand volume/ folder quota 2. Change folder for database |  |
| Event ID |  | 0x820504D2 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | Failed to write log for {0} because folder [{1}] is missed. |  |
| 1st Line Message |  | Failed to write log for {0} because folder [{1}] is missed. |  |
| Cause |  | Folder missing |  |
| Action |  | Change folder for database. |  |
| Event ID |  | 0x820504D3 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | The database for file transfer audit log has been crashed. |  |
| 1st Line Message |  | The database for file transfer audit log has been crashed. |  |
| Cause |  | The database for file transfer audit log has been crashed because of incorrect operation for database |  |
| Action |  | delete crashed database |  |
| Event ID |  | 0x820504D4 |  |
| Severity |  | Information |  |
| Category-Module |  | Service |  |
| Message |  | Data access logs are not available because its database crashes or disappears. |  |
| 1st Line Message |  | Data access logs are not available because its database crashes or disappears. |  |
| Cause |  | The database crash and service stop because of deleting folder or volume |  |
| Action |  | reset the service of data access log |  |
| Event ID |  | 0x824904D5 |  |
| Severity |  | Warning |  |
| Category-Module |  | Account - User |  |
| Message |  | Unable to import the user list. The file must be encoded in UTF-8. |  |
| 1st Line Message |  | Unable to import the user list. The file must be encoded in UTF-8. |  |
| Cause |  | The data file is not encoded in UTF-8. |  |
| Action |  | Open data file and save the file in UTF-8 format again. |  |
| Event ID |  | 0x820904D6 |  |
| Severity |  | Information |  |
| Category-Module |  | Account - User |  |
| Message |  | The user list is successfully imported to the system. |  |
| 1st Line Message |  | The user list is successfully imported to the system. |  |
| Cause |  | Import users success. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824904D7 |  |
| Severity |  | Warning |  |
| Category-Module |  | Account - User |  |
| Message |  | The user list cannot contain more than 1,000 user accounts. |  |
| 1st Line Message |  | The user list cannot contain more than 1,000 user accounts. |  |
| Cause |  | The number of user accounts is more than 1000. |  |
| Action |  | Remove some user data. |  |
| Event ID |  | 0x824904D8 |  |
| Severity |  | Warning |  |
| Category-Module |  | Account - User |  |
| Message |  | Unable to import the user list. The system detects an error in line {0}. |  |
| 1st Line Message |  | Unable to import the user list. The system detects an error in line {0}. |  |
| Cause |  | Data format incorrect. |  |
| Action |  | Check the data of each column in specific line. |  |
| Event ID |  | 0x824904D9 |  |
| Severity |  | Warning |  |
| Category-Module |  | Account - User |  |
| Message |  | The system failed to create a user account for the username: {0}. |  |
| 1st Line Message |  | The system failed to create a user account for the username: {0}. |  |
| Cause |  | Failed to create user. |  |
| Action |  | Create the user from GUI again. |  |
| Event ID |  | 0x824904DA |  |
| Severity |  | Warning |  |
| Category-Module |  | Account - User |  |
| Message |  | The system failed to create a home directory for the user account: {0}. |  |
| 1st Line Message |  | The system failed to create a home directory for the user account: {0}. |  |
| Cause |  | Failed to create the user's home directory. |  |
| Action |  | Configure the user from GUI. |  |
| Event ID |  | 0x824904DB |  |
| Severity |  | Warning |  |
| Category-Module |  | Account - User |  |
| Message |  | The home directory of the user account {0} cannot be accessed via file sharing protocols (FTP, SFTP, and CIFS/SMB). |  |
| 1st Line Message |  | The home directory of the user account {0} cannot be accessed via file sharing protocols (FTP, SFTP, and CIFS/SMB). |  |
| Cause |  | Failed to share the user's home directory via CIFS, FTP or SFTP. |  |
| Action |  | Configure the file sharing settings from GUI. |  |
| Event ID |  | 0x824904DC |  |
| Severity |  | Warning |  |
| Category-Module |  | Account - User |  |
| Message |  | The user account {0} has some minor settings not properly configured. |  |
| 1st Line Message |  | The user account {0} has some minor settings not properly configured. |  |
| Cause |  | Some user's attributes are not configured properly. |  |
| Action |  | Check the user's attributes from Edit user |  |
| Event ID |  | 0x824B04DD |  |
| Severity |  | Warning |  |
| Category-Module |  | RR |  |
| Message |  | Unable to run the scheduled task: {0}. The target folder has new permission settings that require an authorized access. |  |
| 1st Line Message |  | Unable to run the scheduled task: {0}. The target folder has new permission settings that require an authorized access. |  |
| Cause |  | Target folder acl has been replaced by source folder |  |
| Action |  | 1. Cease replicating the ACL of source folder to target. 2. Change the access permission of the user who has been configured in RR task. |  |
| Event ID |  | 0x820604DE |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | Volume {0} starts defragmenting. |  |
| 1st Line Message |  | Volume {0} starts defragmenting. |  |
| Cause |  | Manual launch volume defrag |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820604DF |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | Volume {0} stops defragmenting because the task is ended manually. |  |
| 1st Line Message |  | Volume {0} stops defragmenting because the task is ended manually. |  |
| Cause |  | Manual stop volume defrag |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824604E0 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | Volume {0} skips defragmenting large files because it lacks continuous free space. |  |
| 1st Line Message |  | Volume {0} skips defragmenting large files because it lacks continuous free space. |  |
| Cause |  | Volume skips defragmenting large files because it lacks continuous free space. |  |
| Action |  | 1. Delete unnecessary files 2. Expand volume size 3. Try again later |  |
| Event ID |  | 0x820604E1 |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | Volume {0} stops defragmenting because the time limit is reached. |  |
| 1st Line Message |  | Volume {0} stops defragmenting because the time limit is reached. |  |
| Cause |  | Volume stops defragmenting because the time limit is reached. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x820604E2 |  |
| Severity |  | Information |  |
| Category-Module |  | Storage |  |
| Message |  | Defragmenting volume {0} is complete. |  |
| 1st Line Message |  | Defragmenting volume {0} is complete. |  |
| Cause |  | Defragmenting volume is complete. |  |
| Action |  | Do nothing |  |
| Event ID |  | 0x824604E3 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | Unable to run the scheduled task: {0}. The volumes to defragment are not mounted. |  |
| 1st Line Message |  | Unable to run the scheduled task: {0}. The volumes to defragment are not mounted. |  |
| Cause |  | Unable to run the scheduled task. The volumes to defragment are not mounted. |  |
| Action |  | 1. Check thr volume status 2. Mount the volume manually |  |
| Event ID |  | 0x824604E4 |  |
| Severity |  | Warning |  |
| Category-Module |  | Storage |  |
| Message |  | Unable to run the scheduled task: {0}. The system has too many volumes to defragment. |  |
| 1st Line Message |  | Unable to run the scheduled task: {0}. The system has too many volumes to defragment. |  |
| Cause |  | Unable to run the scheduled task. The system has too many volumes to defragment. |  |
| Action |  | 1. Dothing 2, Try again later. |  |
| Event ID |  | 0x82C204E5 |  |
| Severity |  | Critical |  |
| Category-Module |  | System |  |
| Message |  | The process {0} is terminated due to lack of system memory. |  |
| 1st Line Message |  | The process {0} is terminated due to lack of system memory. |  |
| Cause |  | System memory is not enough |  |
| Action |  | 1. Restart the system 2. Contact support for expanding memory |  |
| Event ID |  | 0x824504E6 |  |
| Severity |  | Warning |  |
| Category-Module |  | Service |  |
| Message |  | Object storage protocol disabled for the folder {0}. Other data transfer protocols are enabled for the folder. |  |
| 1st Line Message |  | Object storage protocol disabled for the folder {0}. Other data transfer protocols are enabled for the folder. |  |
| Cause |  | Service OSS conflict with other service(s) |  |
| Action |  | 1. Check the service configuration 2.do nothing |  |
| Event ID |  | 0x824F04E7 |  |
| Severity |  | Warning |  |
| Category-Module |  | Cloud gateway |  |
| Message |  | Authorization code of the connection {0} has expired. |  |
| 1st Line Message |  | Authorization code of the connection {0} has expired. |  |
| Cause |  | This event is mainly when the token of the cloud task fails |  |
| Action |  | The user can enter a valid token through the UI to replace the expired token and then restart the task. There are oauth cloud and non-oauth cloud oauth cloud: The Google Cloud method is the UI redirect to Google's oauth login page, and obtain a new token after login and authentication. Non-oauth cloud: The user must connect to the background of the cloud to confirm that the key can be used normally and enter it into the UI. |  |
| Event ID |  | 0x824E04E8 |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | The project server crash has been detected and the system will start the recovery process. |  |
| 1st Line Message |  | The project server crash has been detected and the system will start the recovery process. |  |
| Cause |  | DB server has crashed. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820E04E9 |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The project server has been recovered. |  |
| 1st Line Message |  | The project server has been recovered. |  |
| Cause |  | DB server has been recovered. |  |
| Action |  | do nothing |  |
| Event ID |  | 0x824E04EA |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to recover the project server. Please contact the technical support. |  |
| 1st Line Message |  | Failed to recover the project server. Please contact the technical support. |  |
| Cause |  | Failed to recover DB server |  |
| Action |  | Contact spport |  |
| Event ID |  | 0x820E04EB |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The project server is enabled. |  |
| 1st Line Message |  | The project server is enabled. |  |
| Cause |  | DB server has been enabled |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820E04EC |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The project server is disabled. |  |
| 1st Line Message |  | The project server is disabled. |  |
| Cause |  | DB server has been disabled |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820E04ED |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The project server's data has been backed up to the file {0}. |  |
| 1st Line Message |  | The project server's data has been backed up to the file {0}. |  |
| Cause |  | DB Backup |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820E04EE |  |
| Severity |  | Information |  |
| Category-Module |  | Application |  |
| Message |  | The project server's data has been restored from the file {0}. |  |
| 1st Line Message |  | The project server's data has been restored from the file {0}. |  |
| Cause |  | DB restored |  |
| Action |  | do nothing |  |
| Event ID |  | 0x820E04EF |  |
| Severity |  | Warning |  |
| Category-Module |  | Application |  |
| Message |  | Failed to restore the project server's data from the file {0}. |  |
| 1st Line Message |  | Failed to restore the project server's data from the file {0}. |  |
| Cause |  | Failed to restore the DB |  |
| Action |  | try again later |  |